END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 17EC3023 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # RF MEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Explain the different spheres of wireless communication systems (home, office, ground mobile, and space platforms) and analyze their design requirements and challenges. | CO1 | A | 20 |
|  |  | (OR) |  |  |  |
| 2. |  | Discuss various wireless transceiver architectures with neat block diagrams and compare their performance in terms of power efficiency and bandwidth. | CO1 | U | 20 |
| 3. |  | Compare wet etching and dry etching techniques in MEMS fabrication with respect to precision, anisotropy, and applications. | CO2 | An | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | Discuss thin-film deposition techniques such as LPCVD, sputtering, and evaporation, and evaluate their suitability for MEMS fabrication. | CO2 | U | 20 |
| 5. |  | Explain the working of RF MEMS switches and relays with suitable diagrams, and analyze key parameters affecting their performance. | CO3 | A | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Analyze different types of MEMS capacitors such as gap-tuning, area-tuning, and dielectric tunable capacitors with their advantages. | CO3 | An | 20 |
| 7. |  | Summarize reconfigurable antennas using MEMS technology, including their advantages and limitations. | CO4 | U | 20 |
|  |  | (OR) |  |  |  |
| 8. |  | Discuss micro-machined transmission lines and RF components such as coplanar lines, directional couplers, and mixers with diagrams. | CO5 | U | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Explain the fundamentals of RF MEMS phase shifters and analyze their role in phased array antenna systems. | CO6 | A | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the advantages and limitations of RF-MEMS technology. |
| CO2 | Apply the Fabrication methods for MEMS for specific applications. |
| CO3 | Illustrate the working principles of the state-of-the-art RF-MEMS devices. |
| CO4 | Evaluate the merits and drawbacks of an RF-MEMS design. |
| CO5 | Design high-performance circuits and sub-systems using RF MEMS components. |
| CO6 | Design practical RF MEMS devices using analytical and numerical techniques. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 18EC2003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DIGITAL SYSTEM DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State DeMorgan’s theorem for the complement of a product term. |  | CO1 | R | 1 |
| 2. | Explain the role of don't-care conditions in Karnaugh map minimization. |  | CO1 | U | 1 |
| 3. | Define ‘carry look-ahead addition’. |  | CO2 | R | 1 |
| 4. | Identify the function of a barrel shifter in a digital system. |  | CO2 | U | 1 |
| 5. | Define ‘setup time’ in a flip-flop. |  | CO3 | U | 1 |
| 6. | Identify the type of counter in which the output of the last flip-flop is fed back to the input of the first flip-flop. |  | CO3 | U | 1 |
| 7. | Define fan-out in a digital logic family. |  | CO4 | R | 1 |
| 8. | State the purpose of a tristate TTL gate. |  | CO4 | R | 1 |
| 9. | Define PLA with the structure. |  | CO5 | R | 1 |
| 10. | Write the Verilog HDL representation of an EX-OR gate using gate-level modelling. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate Sum of Products (SOP) from Product of Sums (POS) forms. |  | CO1 | U | 3 |
| 12. | Explain the operation of a multiplexer and a demultiplexer with suitable logic diagrams. |  | CO2 | U | 3 |
| 13. | Differentiate edge triggering from level triggering in flip-flops. |  | CO3 | U | 3 |
| 14. | Differentiate TTL from CMOS logic families based on power consumption, speed, and noise immunity. |  | CO4 | U | 3 |
| 15. | Differentiate PROM from EEPROM based on programmability and erasability. |  | CO5 | U | 3 |
| 16. | Explain Behavioural and switch level modeling using Verilog with suitable example. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Illustrate the minimization of the Boolean function F(A,B,C,D)=Σm(1,2,6,7,8,13,14,15)+d(0,3,5,12) using a Karnaugh map with don't-care conditions, and realize the simplified expression. | CO1 | A | 12 |
| 18. |  | Explain the operation of a 2-bit magnitude comparator with suitable logic expressions. | CO2 | A | 12 |
| 19. |  | Illustrate the design of a Johnson Counter using the state diagram, state table, excitation table, state minimization, and circuit implementation. | CO3 | A | 12 |
| 20. |  | Analyze the operation of a TTL NAND gate and discuss its specifications, including fan-in, fan-out, propagation delay, and noise margin. | CO4 | An | 12 |
| 21. |  | Illustrate the implementation of the Boolean function X(A,B,C,D)=Σm (0,3,7,9,8,11,12,13,15) using a PLA. | CO5 | A | 12 |
| 22. |  | Illustrate the implementation of the Boolean function X(A,B,C,D)=Σm (0,2,6,7,8,9,12,13,14) using a PAL. | CO5 | A | 12 |
| 23. |  | Design a Full Adder and Full Subtractor using data flow modelling in Verilog with truth table. | CO6 | C | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Design Verilog-based HDL code using gate-level modelling, and verify the corresponding truth table. | CO6 | C | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Illustrate the basic postulates of Boolean algebra and the operation of logic gates. |
| CO2 | Choose an optimal method for simplification of Boolean expressions. |
| CO3 | Design and distinguish various combinational logic circuits. |
| CO4 | Design and compare various sequential logic circuits. |
| CO5 | Illustrate different logic families; classify memory devices and identify methods for implementation of logic circuits. |
| CO6 | Design simple logic circuits using HDL codes. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 18EC2012 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # LINEAR INTEGRATED CIRCUITS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define slew rate of an OP-AMP. |  | CO1 | R | 1 |
| 2. | State the significance of the number '741' in the IC 741 operational amplifier. |  | CO1 | R | 1 |
| 3. | Calculate the hysteresis voltage of a Schmitt trigger if VUT=10V and VLT=2V. |  | CO2 | A | 1 |
| 4. | Identify the type of comparator from the given output waveform. |  | CO2 | U | 1 |
| 5. | State Barkhausen criterion for oscillation. |  | CO3 | R | 1 |
| 6. | Identify the output voltage of the IC 7802 voltage regulator. |  | CO3 | U | 1 |
| 7. | Identify the type of low-pass filter that provides ripples in the passband. |  | CO4 | U | 1 |
| 8. | Interpret the significance of the damping factor in a Chebyshev filter. |  | CO4 | U | 1 |
| 9. | List any one application of IC 555 timer. |  | CO5 | R | 1 |
| 10. | Identify the ADC that uses a comparator for each quantization level. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | List the ideal characteristics of an OP-AMP. |  | CO1 | R | 3 |
| 12. | A differential amplifier is designed using an op-amp with the following resistor values: R1=1kΩ, R2=10kΩ, R3=1kΩ, R4=10kΩ. Input voltages: V1=0.6V, V2 =0.2V. Compute the output voltage. |  | CO2 | A | 3 |
| 13. | Construct a triangular wave generator circuit. |  | CO3 | A | 3 |
| 14. | Differentiate active filters from passive filters. |  | CO4 | An | 3 |
| 15. | Sketch the schematic diagram of a Schmitt trigger using IC 555. |  | CO5 | A | 3 |
| 16. | Determine the output voltage of a 3-bit DAC if the input is 101 and the reference voltage is 8 V. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Analyze the operation of inverting and non-inverting amplifiers using IC 741 and derive their gain expressions. | CO1 | An | 12 |
| 18. |  | Implement OP-AMP integrator and differentiator circuits and examine the resulting output waveforms for a given input. | CO2 | A | 12 |
| 19. |  | Explain the operation of RC phase-shift oscillator and Wien bridge oscillator and determine their frequencies of oscillation. | CO3 | A | 12 |
| 20. |  | Explain the operation of a second-order low-pass filter and determine its transfer function and cut-off frequency. | CO4 | A | 12 |
| 21. |  | Discuss the operation of IC 555 timer in astable mode and derive the expressions for time period and duty cycle. | CO5 | U | 12 |
| 22. |  | Examine the operation of weighted resistor DAC and R–2R ladder DAC and compare their performance characteristics. | CO6 | A | 12 |
| 23. |  | Illustrate the working principle of OP-AMP Schmitt trigger with relevant diagram and obtain the hysteresis voltage VH.. | CO2 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze the fabrication sequence of Bipolar Junction Transistor ICs and classify the major process steps. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the fundamentals of OP-AMP and its characteristics. |
| CO2 | Use OP-AMP to design circuits such as Amplifiers, differentiator and Integrator. |
| CO3 | Infer the significance of OP-AMP in Multivibrators and Oscillators. |
| CO4 | Design filters using OP-AMP. |
| CO5 | Explore the usefulness of IC555 timer and Phase Locked Loop. |
| CO6 | Design ADC, DAC and understand the IC fabrication. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 18EC2019 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DIGITAL IC DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Discuss the role of metal interconnects in MOS devices and list two commonly used interconnect materials. |  | CO1 | U | 1 |
| 2. | State and explain the built-in potential equation of a PN junction diode. |  | CO1 | R | 1 |
| 3. | State ‘the rise time of a signal’ |  | CO2 | R | 1 |
| 4. | Identify and define the type of power dissipation present when a CMOS gate is not undergoing switching activity. |  | CO2 | R | 1 |
| 5. | Identify the fan-in for the following logic diagram given below: |  | CO3 | U | 1 |
| 6. | Identify the FET devices used as signal-passing elements and explain their operation. |  | CO3 | R | 1 |
| 7. | Discuss the role of metal interconnects in MOS devices and list two commonly used interconnect materials. |  | CO4 | U | 1 |
| 8. | Cite the operation of Domino CMOS logic and specify the clock signal value during its evaluation phase. |  | CO4 | U | 1 |
| 9. | Identify the FET devices used as signal-passing elements and explain their operation. |  | CO5 | U | 1 |
| 10. | Name the major digital storage systems used for storing large quantities of information. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Illustrate the structure of an NMOS transistor and explain the function of each terminal. |  | CO1 | An | 3 |
| 12. | Analyze the importance of noise margins in digital circuits using appropriate equations. |  | CO2 | U | 3 |
| 13. | Construct a 3-input NAND gate using CMOS technology and explain its operation. |  | CO3 | An | 3 |
| 14. | Design and evaluate a Domino CMOS implementation for the logic expression Z=(A⋅B)+CZ = (A \cdot B) + CZ=(A⋅B)+C. |  | CO4 | U | 3 |
| 15. | Compare and categorize various semiconductor memory types based on their functionalities and applications. |  | CO5 | An | 3 |
| 16. | Examine the complete ASIC design methodology and discuss the significance of each design stage. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Illustrate the construction and explain the operation of an enhancement-mode NMOS transistor under its three operating regions. | CO1 | A | 12 |
| 18. | a. | Explain the various types of capacitances in MOS transistor. | CO2 | A | 6 |
|  | b. | Explain the SPICE MOS LEVEL1 Device and BSIM3 model with the design parameters in each model. | CO3 | An | 6 |
| 19. |  | Explain the process involved in making transistors and wires in deep submicron CMOS process. | CO3 | An | 6 |
| 20. | a. | Design Z=A.B.C.D+EF using CMOS Logic. | CO5 | C | 6 |
|  | b. | Analyze the architecture and working principl1e of Dynamic CMOS logic, illustrating its operation with a relevant example. | CO4 | A | 6 |
| 21. |  | Compare and analyze the read and write operations of 1T and 6T SRAM cells, illustrating their architectures with neat diagrams and discussing their performance characteristics. | CO4 | A | 12 |
| 22. | a. | Describe the architecture, working, advantages, and limitations of Transmission Gate (TG) Logic. Using TG logic, construct a 4×1 Multiplexer with a neat circuit diagram. | CO2 | U | 6 |
|  | b. | Implement a 2-input XOR gate using complementary CMOS logic and justify the pull-up and pull-down network design. | CO1 | C | 6 |
| 23. | a. | ## Determine the device sizes for 4-input NAND gate in conventional CMOS. Assume that the basic inverter is sized according to figure given below and the goal of the design is to have NAND4 gate with the same delay characteristics as an inverter. | CO3 | A | 6 |
|  | b. | Discuss the different types of power estimation in Digital IC circuits with suitable equations. | C02 | U | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Classify the different types of semi-custom ASICs and explain their design methodologies, features, and applications with the help of neat diagrams. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic concepts of MOS transistor |
| CO2 | Illustrate different second order effects of MOS transistor |
| CO3 | Analyze static and dynamic behavior of CMOS inverter |
| CO4 | Design combinational logic circuits in CMOS. |
| CO5 | Interpret different logic style for design of sequential logic circuits. |
| CO6 | Comprehend the significance of optimizing the logic circuit design. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 18EC2021 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # MICROWAVE AND OPTICAL COMMUNICATION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Indicate the importance of Microwave Communication. |  | CO1 | U | 1 |
| 2. | Name the device that converts coaxial to waveguide transitions |  | CO1 | R | 1 |
| 3. | Enumerate the importance of an isolator in microwave communication. |  | CO1 | R | 1 |
| 4. | Indicate any one application of a thermistor. |  | CO1 | R | 1 |
| 5. | Give examples of microwave sensors. |  | CO2 | U | 1 |
| 6. | Identify the material used in GUNN diode fabrication. |  | CO2 | R | 1 |
| 7. | Write an application of a magnetron. |  | CO3 | A | 1 |
| 8. | List linear M-tube devices. |  | CO3 | R | 1 |
| 9. | Determine the numerical aperture at the core–cladding interface if n₁ = 1.50 and n₂ = 1.47. |  | CO5 | A | 1 |
| 10. | Write the significance of the acceptance angle in optical fibre. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the functions of tuning screws and windows in achieving impedance matching in waveguides. |  | CO1 | A | 3 |
| 12. | Sketch a Directional Coupler and explain its concept. |  | CO2 | A | 3 |
| 13. | Compare Reciprocal Devices with Non-Reciprocal Devices. |  | CO3 | An | 3 |
| 14. | List the properties of the S matrix applicable to a 2-port Isolator. |  | CO4 | R | 3 |
| 15. | Differentiate micro bending loss from macro bending loss. |  | CO5 | U | 3 |
| 16. | Illustrate the stimulated emission process. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Discuss the importance of waveguide corners, bends and twists in microwave systems. | CO1 | U | 8 |
|  | b. | Summarize waveguide excitation methods | CO1 | E | 4 |
| 18. |  | Explain the operation of the Magic Tee and derive its scattering matrix. | CO2 | A | 12 |
| 19. |  | Describe the construction and operation of a Reflex Klystron with the Applegate diagram. | CO3 | U | 12 |
| 20. | a. | Write in detail about the working principle of IMPATT diode with neat diagrams | CO4 | A | 10 |
|  | b. | Sketch the structure of BARITT diode. | C04 | A | 2 |
| 21. |  | Analyze the factors responsible for attenuation when an optical signal passes through the fibre. | CO5 | An | 12 |
| 22. | a. | Explain the optical fibre communication system, detailing its components and operations with a neat block diagram. | CO5 | A | 8 |
|  | b. | Analyze different types of optical fibers based on their mode of propagation | CO5 | An | 4 |
| 23. |  | Apply the concept of light gathering capacity in optical fiber communication and derive the relation between the core and cladding refractive indices. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the working principle of edge-emitting LED. | CO6 | A | 8 |
|  | b. | List the types of photodetectors. | C06 | R | 4 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the operation of passive waveguide components. |
| CO2 | Distinguish the limitations of existing vacuum tubes and solid-state devices at microwave frequencies. |
| CO3 | Predict the performance of specialized microwave tubes such as two cavity klystron, reflex klystron, magnetron and Travelling wave tube. |
| CO4 | Classify microwave circuits using scattering parameters. |
| CO5 | Relate the characteristics of Optical Fiber components |
| CO6 | Summarize optical source, Fiber and Detector operational parameters |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 18EC2028 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # MICROPROCESSOR AND MICROCONTROLLER | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Indicate the size of address bus in 8085 microprocessor. |  | CO1 | U | 1 |
| 2. | Write the signal used to demultiplex the address and data lines of 8085 microprocessor. |  | CO1 | A | 1 |
| 3. | Identify the 8051 register that holds the memory address of the next instruction to be executed. |  | CO2 | U | 1 |
| 4. | Write the I/O port of 8051 that has no alternate function. |  | CO2 | A | 1 |
| 5. | Identify the addressing mode for the given 8051 microcontroller instruction. MOV 45H, A |  | CO3 | U | 1 |
| 6. | Write any one debugging tool used in microcontroller development. |  | CO3 | A | 1 |
| 7. | State the need for ADC in a microcontroller system. |  | CO4 | R | 1 |
| 8. | Indicate the number of address lines required to interface 8051 microcontroller with 8K RAM. |  | CO4 | U | 1 |
| 9. | Name the two-wire serial communication protocol used in embedded systems. |  | CO5 | R | 1 |
| 10. | Name the LCD register that stores the data to be displayed on the LCD. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare microprocessor with microcontroller. |  | CO1 | U | 3 |
| 12. | Sketch the flag register format of 8051 microcontroller and specify the function of each bit. |  | CO2 | A | 3 |
| 13. | Differentiate global variables from local variables in a C program. |  | CO3 | An | 3 |
| 14. | Write the format of TMOD register of 8051 microcontroller. |  | CO4 | A | 3 |
| 15. | List the three types of data transfer modes used in external communication interface. |  | CO5 | R | 3 |
| 16. | Sketch the interfacing diagram of three LEDs connected to port pins P2.2, P2.3, and P2.4 of 8051 microcontroller. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the internal architecture of 8085 microprocessor with a neat block diagram. | CO1 | A | 12 |
| 18. |  | Explain the architecture of 8051 microcontroller with a neat functional block diagram. | CO2 | A | 12 |
| 19. | a. | Discuss the addressing modes of 8051 microcontroller with suitable examples. | CO3 | U | 10 |
|  | b. | List any two Special Function Registers (SFRs) of 8051 microcontroller. | CO2 | R | 2 |
| 20. |  | Sketch the diagram to interface external 16K RAM with 8051 microcontroller and write its address mapping. | CO4 | A | 12 |
| 21. |  | Explain the working principles of Zigbee and Bluetooth protocol with neat diagrams. | CO5 | A | 12 |
| 22. | a. | Explain the operation of 8051 timer in 16-bit mode with a neat diagram. | CO4 | A | 8 |
|  | b. | Sketch the internal RAM structure of 8051 microcontroller. | CO2 | A | 4 |
| 23. |  | Discuss in detail the working of SPI synchronous serial communication protocol with a neat diagram. | CO5 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Sketch the interfacing diagram of a stepper motor with 8051 microcontroller and explain its operation in detail. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Discuss the microprocessor organization and its evolution. |
| CO2 | Describe the architecture of 8051 controllers. |
| CO3 | Express their knowledge in designing a system using 8051. |
| CO4 | Differentiate controller/processor architecture and features. |
| CO5 | Write processor/controller specific programs in Embedded C. |
| CO6 | Simulate the real time system using integrated development environment. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 18EC3013 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ADVANCED DIGITAL IMAGE PROCESSING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Illustrate the fundamental steps involved in Digital Image Processing using a block diagram. | CO1 | A | 10 |
|  | b. | Analyze the influence of spatial and intensity resolution on digital image representation. | CO1 | An | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Analyze the effectiveness and limitations of spatial domain techniques in improving image quality with suitable examples. | CO2 | An | 10 |
|  | b | Evaluate the importance of DFT properties (linearity and symmetry) in image processing applications. | CO2 | E | 10 |
| 3. |  | Discuss about the various morphological operations employed in Image processing. | CO3 | U | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | Explain the various transformation functions involved in image registration. | CO4 | A | 20 |
| 5. |  | Illustrate the concepts of volumetric display, stereo viewing and measurements on 3D image processing. | CO5 | A | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Explain the role of digital image processing in medical imaging and evaluate its importance in diagnosis and treatment. | CO1 | A | 20 |
| 7. | a. | Discuss the image enhancement techniques in frequency domain. | CO2 | U | 10 |
|  | b. | Illustrate the concept of edge detection using morphological operations. | CO3 | A | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Evaluate the challenges involved in scene understanding for natural images and propose suitable solutions. | CO6 | E | 10 |
|  | b. | Explain the principles and techniques of 3D image visualization. | CO5 | A | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Analyze the role, advantages, and limitations of virtual reality in image processing applications. | CO6 | An | 10 |
|  | b. | Interpret the concept and applications of object detection in SAR (Synthetic Aperture Radar) images. | CO6 | U | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Explain the basic concepts of image formation and representation. |
| CO2 | Design techniques for enhancing the quality of the images. |
| CO3 | Frame morphology-based methodologies for image segmentation. |
| CO4 | Assess the performances of various image registration approaches. |
| CO5 | Differentiate the concepts of 2D and 3D image processing approaches. |
| CO6 | Solve practical problems using image processing techniques. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 18EC3054 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # HIGH SPEED VLSI DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Examine the performance of Differential Cascode Voltage Switch (DCVS) logic in high-speed digital circuits. Compare its advantages over static CMOS in terms of delay, noise immunity and power dissipation. | CO1 | A | 20 |
|  |  | (OR) |  |  |  |
| 2. | a. | Design a clocked pass transistor logic circuit integrated with dynamic logic techniques for improved speed. Discuss the timing constraints and signal degradation issues. | CO2 | C | 10 |
|  | b. | Illustrate the functions of dual-rail domino logic circuit and analyze the trade-offs between energy consumption and operational speed. | CO2 | An | 10 |
| 3. |  | Propose a robust design methodology to mitigate the impact of variations while ensuring adequate noise margins and yield improvement. | CO3 | C | 20 |
|  |  | (OR) |  |  |  |
| 4. | a. | Compare differential latches and single-ended latches in terms of speed, power efficiency, and susceptibility to noise. | CO4 | An | 10 |
|  | b. | Design an asynchronous latch circuit suitable for high-speed applications. Discuss how timing hazards and race conditions can be minimized. | CO4 | C | 10 |
| 5. |  | Design a logic gate using Static Random Pass Logic (SRPL) and evaluate its performance in terms of delay and power consumption. | CO5 | C | 20 |
|  |  | (OR) |  |  |  |
| 6. | a. | Design a logic gate using SRPL for a low-latency digital circuit. Describe how this logic family enhances speed and reduces propagation delay. | CO1 | C | 10 |
|  | b. | Develop a high-speed digital subsystem using Pseudo NMOS or Pass Transistor Logic (PTL), focusing on area optimization and reduced power dissipation. | CO1 | C | 10 |
| 7. |  | Design a driver circuit for high-speed chip-to-chip communication. Discuss techniques such as impedance matching, signal buffering and voltage scaling. | CO5 | C | 20 |
|  |  | (OR) |  |  |  |
| 8. |  | Design an efficient clock distribution network considering skew minimization, jitter control and power optimization. | CO6 | C | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Develop an asynchronous VLSI system aimed at minimizing power consumption while maintaining high throughput. Discuss design challenges and solutions. | CO6 | C | 10 |
|  | b. | Analyze the issues involved in clock generation and synchronization across multiple clock domains in modern VLSI systems. | CO6 | An | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Summarize the characteristics of Non-Clocked Logic Styles |
| CO2 | Illustrate the characteristics of Clocked Logic Styles |
| CO3 | Generalize the circuit design margin |
| CO4 | Categorize the latching strategies |
| CO5 | Outline the Interface techniques in high-speed VLSI Design |
| CO6 | Design and develop high speed CMOS VLSI design circuits |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 19EC2001 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRONICS FOR INTELLIGENT MACHINES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define Digitalization. |  | CO1 | R | 1 |
| 2. | Give examples of Intelligent Machines. |  | CO1 | U | 1 |
| 3. | Describe Intelligent Manufacturing. |  | CO2 | U | 1 |
| 4. | List any two applications of Intelligent Machines. |  | CO2 | R | 1 |
| 5. | Describe Internet of Things. |  | CO3 | U | 1 |
| 6. | Define Machine to Machine (M2M) communication. |  | CO3 | R | 1 |
| 7. | List any two types of sensors used in intelligent machines. |  | CO4 | R | 1 |
| 8. | Give examples of an application of a GPS module. |  | CO4 | U | 1 |
| 9. | List the steps involved in transforming data into information. |  | CO5 | R | 1 |
| 10. | Describe Big Data. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Describe the impact of Industry of 4.0. |  | CO1 | U | 3 |
| 12. | List the applications of Big Data. |  | CO2 | R | 3 |
| 13. | Interpret the significance of Internet of Things. |  | CO3 | U | 3 |
| 14. | Describe GPS. |  | CO4 | U | 3 |
| 15. | List the disadvantages of public cloud. |  | CO5 | R | 3 |
| 16. | Discuss the advantages of supply chain management. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the various Industrial Revolutions and their key features. | CO1 | A | 12 |
| 18. |  | Infer the concept of intelligent machines and their important characteristics. | CO2 | An | 12 |
| 19. |  | Analyze the role of Internet of Things (IoT) in connecting intelligent devices. | CO3 | An | 12 |
| 20. |  | Explain the basics of sensors and their importance in intelligent machines. | CO4 | A | 12 |
| 21. | a. | Explain the concept of Big Data and its applications. | CO5 | A | 6 |
|  | b. | Infer the significance of cloud computing and discuss its applications. | CO5 | An | 6 |
| 22. |  | Illustrate with examples the drivers, enablers and challenges of Industry 4.0. | CO1 | A | 12 |
| 23. |  | Infer the working of GSM and GPS interface modules with suitable applications. | CO4 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Examine the implementation of intelligent machines in industrial applications through suitable case studies. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compare the different industry standards |
| CO2 | Articulate the structure of an Intelligent machine |
| CO3 | Illustrate the m2m interface needed in intelligent machining |
| CO4 | Categorize the sensors for various intelligent machines |
| CO5 | Assess the data requirements for cloud storage |
| CO6 | To be able to Grade various types of Intelligent machines |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 19EC2012 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # WIRELESS SENSOR NETWORKS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define a Wireless Sensor Network. |  | CO1 | R | 1 |
| 2. | List any two components of a sensor node. |  | CO1 | R | 1 |
| 3. | State the purpose of IEEE 802.15.4 in wireless sensor networks. |  | CO2 | R | 1 |
| 4. | State one function of the data link layer in WSNs. |  | CO2 | R | 1 |
| 5. | Define Time Difference of Arrival (TDoA). |  | CO3 | R | 1 |
| 6. | Name one location tracking algorithm used in WSNs. |  | CO3 | R | 1 |
| 7. | Define data aggregation. |  | CO4 | R | 1 |
| 8. | State one desired property of a sensor network database. |  | CO4 | R | 1 |
| 9. | Define idle power management. |  | CO5 | R | 1 |
| 10. | Define an EDGE network. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the functional architecture of a wireless sensor network. |  | CO1 | U | 3 |
| 12. | Compare ZigBee with IEEE 802.15.4. |  | CO2 | U | 3 |
| 13. | Explain the principle of Angle of Arrival (AoA) technique. |  | CO3 | U | 3 |
| 14. | Differentiate packet-level aggregation from geographic aggregation. |  | CO4 | U | 3 |
| 15. | Explain the concept of active power management. |  | CO5 | U | 3 |
| 16. | Describe the role of base stations in wireless sensor networks. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the architecture of a sensor node with a neat diagram. | CO1 | U | 6 |
|  | b. | Compare wireless sensor networks with traditional networks. | CO1 | E | 6 |
| 18. | a. | Explain the operation of the SMAC protocol. | CO2 | A | 6 |
|  | b. | Analyze the design constraints of MAC protocols for WSNs. | CO2 | An | 6 |
| 19. |  | Explain the positioning and location tracking algorithms used in wireless sensor networks. | CO3 | U | 12 |
| 20. | a. | Explain the goals and benefits of data aggregation in wireless sensor networks. | CO4 | U | 6 |
|  | b. | Analyze the challenges associated with sensor network databases. | CO4 | An | 6 |
| 21. | a. | Compare the power management mechanisms of IEEE 802.11 with Bluetooth. | CO5 | E | 6 |
|  | b. | Explain the impact of state transition overhead and sleep time threshold derivation on energy savings. | CO5 | U | 6 |
| 22. |  | Explain node-level energy management in wireless sensor networks. | CO5 | A | 12 |
| 23. | a. | Explain the security architecture of wireless sensor networks. | CO6 | A | 6 |
|  | b. | Analyze the privacy issues associated with location information in WSNs. | CO6 | An | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the architecture and features of cell-based wireless sensor networks. | CO6 | U | 6 |
|  | b. | Explain the applications of wireless sensor networks in health monitoring and agriculture. | CO6 | A | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Develop the architecture of wireless sensor networks, including sensor node components and functional architecture. |
| CO2 | Apply communication protocols like MAC, transport, and network layer protocols, focusing on IEEE 802.15.4 and Zigbee standards. |
| CO3 | Plan tracking techniques using positioning algorithms like ToA, TDoA, AoA, and signal strength-based location tracking. |
| CO4 | Develop sensor network databases using aggregation methods and selecting optimal aggregation points. |
| CO5 | Design power management strategies to improve energy efficiency in wireless sensor networks. |
| CO6 | Evaluate the applications of wireless sensor networks in domains such as health monitoring and agriculture, focusing on security and privacy considerations. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 19EC2019 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ASIC DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List the levels of integration in VLSI. |  | CO1 | R | 1 |
| 2. | A fabrication facility provides a silicon wafer where p-type and n-type transistors are already arranged in a fixed, repetitive pattern. The designer only customizes the interconnect layers. Identify the ASIC approach being applied. |  | CO1 | A | 1 |
| 3. | Identify the Programming technology that is volatile. |  | CO2 | U | 1 |
| 4. | Describe the equation to find delay in Digital logic circuits. |  | CO2 | R | 1 |
| 5. | Define Propagation delay in Digital System Design. |  | CO3 | R | 1 |
| 6. | Define Halfgate ASIC. |  | CO3 | R | 1 |
| 7. | Identify the net cutset in the following network. |  | CO4 | U | 1 |
| 8. | State Hooke’s law used in force-directed placement. |  | CO4 | R | 1 |
| 9. | Describe the inputs required for Global routing. |  | CO5 | U | 1 |
| 10. | State the input files provided to the Genus tool in Cadence EDA Tool. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Evaluate Z=A.B)+C using CMOS Logic. |  | CO1 | A | 3 |
| 12. | Analyze the differences between EPROM and EEPROM technologies in terms of erasing mechanism, programming method, speed, and application suitability. |  | CO2 | An | 3 |
| 13. | Enumerate the architecture of the Altera MAX 5000 series. |  | CO3 | A | 3 |
| 14. | Determine the use of cutset cut and edge cut methods to partition a given VLSI circuit for minimizing interconnection complexity and improving layout efficiency. |  | CO4 | A | 3 |
| 15. | Summarize the different interconnect models for a VLSI circuit and determine the delay in each case. |  | CO5 | A | 3 |
| 16. | Describe the Verilog Code for a 2X1 Multiplexer in any one modeling. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Determine the sequence of stages in the design flow of an ASIC with a neat diagram and applications. | CO1 | A | 12 |
| 18. | a. | Determine the architecture of the Xilinx XC4000 Configurable Logic Block (CLB) for implementing a specified combinational and sequential logic function. | CO2 | A | 6 |
|  | b. | Examine the suitable programming technologies for implementing programmable ASICs based on design requirements. | CO2 | A | 6 |
| 19. | a. | Illustrate the configuration of the Actel ACT Interconnect to implement a given logic design in an FPGA. | CO3 | A | 6 |
|  | b. | Analyze the architecture and structure of Xilinx LCA and determine its impact on logic implementation and routing efficiency. | CO3 | An | 6 |
| 20. |  | With relevant equations, determine the connectivity matrix for the network shown below using KL algorithm. Also compute the gain in the network graph if (a) nodes 1 and 6 are swapped with proper explanation using KL algorithm. | CO4 | A | 12 |
| 21. |  | Illustrate the implementation of Global and Detailed Routing for connecting modules in a given VLSI layout | CO5 | A | 12 |
| 22. | a. | Interpret the improvement in Iterative Placement algorithm by using various placement improvement methods. | CO4 | A | 6 |
|  | b. | Explain the implementation of Schematic Entry for creating a circuit representation in VLSI design. | CO3 | A | 6 |
| 23. |  | Analyze the structure and distribution network of Power Routing in VLSI layout design. | CO5 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Design a synthesizable Verilog implementation of a 4×1 multiplexer with its gate-level circuit architecture. | CO6 | C | 8 |
|  | b. | Develop synthesizable Verilog code for a 4-bit Magnitude Comparator using combinational logic constructs and conditional operators. | CO6 | C | 4 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define the types of ASICs, combinational and sequential logic cells, concepts of design rules and logical effort. |
| CO2 | Describe the programmable ASICs and programmable ASIC logic cells. |
| CO3 | Demonstrate programmable ASIC interconnect and programmable ASIC design software. |
| CO4 | Illustrate the goals and objectives of partitioning, floorplanning and placement. |
| CO5 | Develop algorithms for various types of routing and explain the concepts of circuit extraction and DRC. |
| CO6 | Develop the HDL logic synthesis skills. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 19EC2019 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ASIC DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the mode in which an enhancement-mode MOSFET operates when VGS=0V. |  | CO1 | U | 1 |
| 2. | A designer selects predesigned AND, OR, multiplexers, and flip-flops from a library and focuses only on arranging them and connecting signals.Identify the ASIC style being applied. |  | CO1 | A | 1 |
| 3. | State the programming voltage of the EPROM transistor. |  | CO2 | R | 1 |
| 4. | Describe the primary function of flip-flops in digital circuits. |  | CO2 | R | 1 |
| 5. | State the number of wires contained in each track of the Actel ACT family interconnect |  | CO3 | R | 1 |
| 6. | State the interconnect scheme to which the Actel ACT family is identical. |  | CO3 | R | 1 |
| 7. | Summarize the two objectives of Floor planning in Physical Design Flow. |  | CO4 | U | 1 |
| 8. | Construct the graph from the following network for system partitioning. |  | CO4 | A | 1 |
| 9. | Identify the channel capacity in Global routing for seven horizontal wiring tracks available in one row of gate-array base cells. |  | CO5 | U | 1 |
| 10. | Identify the tool used for Physical design flow in Cadence EDA Tool. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Construct Z=A+B+C using CMOS Logic. |  | CO1 | A | 3 |
| 12. | Determine the configuration of an Actel antifuse FPGA for a given logic circuit using antifuse programming. |  | CO2 | A | 3 |
| 13. | ## Determine the device sizes for 3-input NAND gates in conventional CMOS. Assume that the basic inverter is sized according to the figure given below and that the goal of the design is to have NAND3 gates with the same delay characteristics as the inverter. |  | CO3 | A | 3 |
| 14. | Convert the representation of the routing region to channel connectivity graph. |  | CO4 | U | 3 |
| 15. | Determine the appropriate placement methods for implementing a given design based on performance and area requirements. |  | CO5 | A | 3 |
| 16. | Describe the Verilog Code for a Full adder in any one modeling. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Analyze the different types of an ASIC and determine their suitability for specific applications based on performance and cost. | CO1 | An | 12 |
| 18. | a. | Determine the suitable use of EPROM and EEPROM for implementing programmable ASICs based on design requirements. | CO2 | A | 4 |
|  | b. | Determine the architecture of the Xilinx XC 3000 Configurable Logic Block (CLB) and its application in implementing a combinational and sequential logic function. | CO2 | A | 8 |
| 19. | a. | Illustrate the configuration of Altera FLEX to implement a given digital logic design. | CO3 | A | 6 |
|  | b. | Analyze the architecture and structure of Xilinx LCA and determine its impact on logic implementation and routing efficiency. | CO3 | An | 6 |
| 20. | a. | Determine the objectives of Partitioning methods and sketch the graph from the given Fig.1 and Fig.2 and write the edge cut set of the graph obtained from Fig.2. | CO4 | A | 8 |
|  | b. | Illustrate the implementation of the Force Directed Placement algorithm for placing modules in a given VLSI layout. | CO4 | A | 4 |
| 21. |  | Illustrate the VLSI verification step that ensures the layout complies with manufacturing rules before fabrication to detect defects in an IC. | CO5 | A | 12 |
| 22. |  | Analyze the hierarchical syntax, netlist representation, and library mapping of EDIF in VLSI design data exchange. | CO3 | An | 12 |
| 23. | a. | Apply circuit extraction to determine the parasitic elements and delay in a given routed interconnect. | CO5 | A | 6 |
|  | b. | Illustrate the implementation of Schematic Entry for creating a circuit representation in VLSI design. | CO3 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Develop synthesizable Verilog code for a 4×4 array multiplier and illustrate its gate-level circuit architecture. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define the types of ASICs, combinational and sequential logic cells, concepts of design rules and logical effort. |
| CO2 | Describe the programmable ASICs and programmable ASIC logic cells. |
| CO3 | Demonstrate programmable ASIC interconnect and programmable ASIC design software. |
| CO4 | Illustrate the goals and objectives of partitioning, floorplanning and placement. |
| CO5 | Develop algorithms for various types of routing and explain the concepts of circuit extraction and DRC. |
| CO6 | Develop the HDL logic synthesis skills. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 19EC2021 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # LOW POWER TECHNIQUES IN VLSI DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State Moore’s law. |  | CO1 | R | 1 |
| 2. | State the different types of capacitances present in gate-level models. |  | CO1 | R | 1 |
| 3. | Identify the primary sources of leakage current in CMOS circuits. |  | CO2 | R | 1 |
| 4. | Identify the type of power consumed due to signal switching. |  | CO2 | R | 1 |
| 5. | Define clock masking. |  | CO3 | R | 1 |
| 6. | Determine the total number of bit transitions in a 3-bit binary counting sequence. |  | CO3 | A | 1 |
| 7. | Describe read operation of a memory cell. |  | CO4 | U | 1 |
| 8. | Identify the oscillator implemented using an odd number of cascaded NOT gates connected in a ring. |  | CO4 | R | 1 |
| 9. | Classify the different types of RAM. |  | CO5 | U | 1 |
| 10. | Identify the logic family that uses two transmission gates and a dual-rail encoded output. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Examine the consequences of static current flow in a pseudo-NMOS NOR gate. |  | CO1 | A | 3 |
| 12. | Analyze the role of logic encoding techniques in minimizing power dissipation. |  | CO2 | An | 3 |
| 13. | Analyze the given logic circuit and estimate the total number of CMOS transistors required for implementation. |  | CO3 | An | 3 |
| 14. | Evaluate the given logic circuit and determine the corresponding logic expression. |  | CO4 | E | 3 |
| 15. | Compare Domino NAND logic with static CMOS NAND logic. |  | CO5 | An | 3 |
| 16. | Describe the four operational phases of a pulsed power supply. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Analyze the Gate Reorganization technique in a gate-level network and explain its benefits. | CO1 | An | 8 |
|  | b. | A 32 bit off-chip bus operating at 3.3V and 66MHz clock rate is driving a capacitance of 20pF/bit. Each bit is estimated to have a toggling probability of 0.26 at each clock cycle. Calculate the power dissipation in operating the bus. | CO1 | A | 4 |
| 18. |  | Determine the power consumed during the charging and discharging of a capacitive load and derive the relevant expressions. | CO2 | A | 12 |
| 19. | a. | Analyze the implementation of parallel architecture using voltage reduction techniques in high-performance digital circuits. | CO3 | An | 6 |
|  | b. | Construct the flow graph for the given logic expression: Y=A.B+A.C+B.C | CO3 | C | 6 |
| 20. |  | Evaluate the power dissipation in Pass Transistor Logic and TSPC flip-flop architectures for negative and positive edge triggering, respectively. | CO4 | E | 12 |
| 21. | a. | Draw and explain the architecture of a 6T SRAM cell with a suitable diagram. | CO5 | A | 6 |
|  | b. | Explain the basic types of semiconductor memories. | CO5 | A | 6 |
| 22. | a. | Discuss Equivalent Pin Ordering and its significance in CMOS circuits. | CO2 | U | 6 |
|  | b. | Analyze the behavior of CMOS floating nodes. | CO3 | An | 6 |
| 23. | a. | Design Z=(A.B.C+DE) using Dynamic CMOS Logic. | CO4 | C | 6 |
|  | b. | Illustrate the organization of a Static RAM with neat block diagram. | CO5 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze the stepwise charging technique in adiabatic logic and derive the expressions for output voltage and energy dissipation with necessary diagrams. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Develop skills to simulate gate-level logic and perform power analysis. |
| CO2 | Develop optimized circuits through techniques such as transistor and gate sizing, network reconstruction, and reorganization. |
| CO3 | Evaluate the impact of switching activity reduction and parallel architectures on power consumption and performance management. |
| CO4 | Examine the development of latches and flip-flops and their influence on power consumption and performance. |
| CO5 | Develop strategies for reducing power in the write driver and sense amplifier circuits of SRAM architectures. |
| CO6 | Design adiabatic logic gates to achieve low-power operation in digital circuits. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 19EC2043 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # TESTING OF VLSI CIRCUITS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State the compact version of the truth table in a logic gate. |  | CO1 | U | 1 |
| 2. | List the term that tells whether a good system will work after some time. |  | CO1 | R | 1 |
| 3. | Identify the term used in D-algorithm that is used to specify the existence of a given fault. |  | CO2 | R | 1 |
| 4. | Identify the Non-controlling value for AND / NAND gates. |  | CO2 | U | 1 |
| 5. | The circuit is first simulated in the true-value mode for all vectors and primary output values are saved in a file. Identify the simulation process. |  | CO3 | U | 1 |
| 6. | Identify the term used for the ratio of detected faults to the total number of faults in the fault list. |  | CO3 | A | 1 |
| 7. | Name any two DFT Techniques. |  | CO4 | R | 1 |
| 8. | State the two types of test points. |  | CO4 | R | 1 |
| 9. | Name the pin at which the state transitions in the TAP controller occur on the rising edge of the clock pulse. |  | CO5 | R | 1 |
| 10. | Describe the technique that generates random test sequences and compacts the output data. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Apply the concept of bridging faults to determine the effect of an unwanted connection between two signal lines in a digital circuit. |  | CO1 | A | 3 |
| 12. | Construct the circuit diagram of time sharing I/O Ports. |  | CO2 | A | 3 |
| 13. | Analyze the role of a fault simulator in determining fault coverage and test effectiveness. |  | CO3 | An | 3 |
| 14. | Apply the concept of controllability to determine the ease of setting a specified logic value at an internal node. |  | CO4 | A | 3 |
| 15. | Analyze the limitations of multiplexer-based testing with respect to area, delay, and power overhead. |  | CO5 | An | 3 |
| 16. | Apply the Built-In Self-Test (BIST) technique to test a digital circuit and determine its effectiveness in fault detection. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Determine the output of Z1 and Z2 of the circuit shown in Fig.1 with short and without short. Fig.1 | CO1 | A | 6 |
|  | b. | Let f be the OR bridging fault between x1 and x2 in the following circuit. Determine the outputs Z1 and Z2 with the fault f and also the test vector to detect the fault f in Fig.2. Fig.2 | CO1 | A | 6 |
| 18. |  | Determine the properties in Boolean difference and find the test patterns for stuck-at faults with respect to X4 for the circuit shown in Fig.3. Fig.3 | CO2 | A | 12 |
| 19. |  | Describe the concurrent fault simulation algorithm step-by-step and explain it with a suitable example circuit. | CO3 | A | 12 |
| 20. |  | Analyze the role of boundary scan methodology in enabling board-level testing and discuss its impact on fault isolation and repair in multi-chip configurations. | CO4 | An | 12 |
| 21. |  | Explain the application of exhaustive testing and pseudorandom testing in Built-In Self-Test (BIST) for detecting faults in a digital circuit. | CO5 | A | 12 |
| 22. |  | Apply the path sensitization technique to determine the test vector for the fault shown in the given circuit in Fig.4. Fig.4 | CO2 | A | 12 |
| 23. | a. | Analyze the functionality of LSSD by comparing the behavior of different latches used in scan-based testing. | CO5 | An | 6 |
|  | b. | Analyze the working principle and effectiveness of Signature Analysis in detecting faults in digital circuits. | CO5 | An | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Apply BIST concepts to determine the roles of Hardcore BIST and various Levels of Test in a digital system. | CO6 | A | 8 |
|  | b. | Analyze the Test-Pattern Generation techniques used in BIST and determine their impact on fault coverage with suitable diagrams. | CO6 | An | 4 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Gain basic knowledge in VLSI testing |
| CO2 | Particular testing method for a Device under Test (DUT) |
| CO3 | Categorize various fault simulation techniques |
| CO4 | Illustrate the Design for Testability (DFT) concepts |
| CO5 | Articulate Scan Based Design concepts |
| CO6 | Demonstrate the various BIST architectures |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 19EC2059 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # FUNDAMENTALS OF SATELLITE COMMUNICATION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Name the first man-made satellite launched by former USSR. |  | CO1 | R | 1 |
| 2. | State Kepler’s first law. |  | CO1 | R | 1 |
| 3. | Identify the 6xxx series alloy that has good strength, light weight and better corrosion properties. |  | CO2 | R | 1 |
| 4. | State the temperature range of spacecraft interior. |  | CO2 | R | 1 |
| 5. | Define EIRP. |  | CO3 | R | 1 |
| 6. | Give examples of HPA. |  | CO3 | U | 1 |
| 7. | Infer the importance of Guard band. |  | CO4 | U | 1 |
| 8. | Identify the type of frequency-hop spread spectrum while Tc >= Ts. |  | CO4 | U | 1 |
| 9. | State the reasons of rain induced attenuation. |  | CO5 | R | 1 |
| 10. | List any two use cases of VSAT star topology. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | State Newton’s law of gravitation. |  | CO1 | R | 3 |
| 12. | Explain the role of solar sails in high power generation. |  | CO2 | A | 3 |
| 13. | Illustrate single amplifier HPA configuration. |  | CO3 | A | 3 |
| 14. | With a suitable diagram, explain the operation of a transponder, receiving signals from three transmitting earth stations operating with FDMA. |  | CO4 | A | 3 |
| 15. | Calculate the EIRP in dBW, if a satellite downlink at 4 GHz operates with a transmit power of 6W and an antenna gain of 48.2dB. |  | CO5 | A | 3 |
| 16. | Tabulate the recent INSAT satellites and their status. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Apply Newton’s first law to rocket liftoff. | CO1 | A | 6 |
|  | b. | Deduce the equation of orbital velocity. | CO1 | An | 6 |
| 18. | a. | Explain the significance of MEO over LEO. | CO1 | An | 6 |
|  | b. | Summarize the frequency bands used for satellite communications. | CO1 | U | 6 |
| 19. | a. | Evaluate the common battery types used in space. | CO2 | E | 6 |
|  | b. | Deduce the role of passive and active thermal components in temperature management. | CO2 | An | 6 |
| 20. | a. | Illustrate non-regenerative satellite transponder. | CO2 | A | 6 |
|  | b. | Explain the antenna subsystem of the satellite. | CO2 | A | 6 |
| 21. | a. | Construct the architecture of an earth station with suitable baseband and RF sections. | CO3 | A | 6 |
|  | b. | Illustrate power combining using Magic-T. | CO3 | A | 6 |
| 22. |  | Develop Slow frequency-hop spread spectrum for the following data: | CO4 | A | 12 |
| 23. |  | Compare the Ionosphere F1 layer with F2 layer. | CO5 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Compare VSAT star topology with mesh topology. | CO6 | An | 6 |
|  | b. | Illustrate the future developments of INSAT system. | CO6 | An | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the satellite orbits, elements of satellite and operation of satellite communication. |
| CO2 | Interpret the concepts of space segment, propulsion, payload, and TTC. |
| CO3 | Analyze the design requirements and the performance of earth station. |
| CO4 | Develop the multiplexing techniques, modulation techniques, and multiple access techniques for satellite communication. |
| CO5 | Illustrate the concepts of link design, rain fading and link availability and perform interference calculations. |
| CO6 | Design various satellite applications. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 20EC2002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRONIC DEVICES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define semiconductor. |  | CO1 | R | 1 |
| 2. | Identify the majority carrier in a p-type semiconductor. |  | CO1 | R | 1 |
| 3. | State the diode equation. |  | CO2 | R | 1 |
| 4. | Explain the significance of barrier potential. |  | CO2 | U | 1 |
| 5. | Name the regions of a transistor. |  | CO3 | R | 1 |
| 6. | Sketch the Ebers-Moll equivalent model of an NPN transistor. |  | CO3 | A | 1 |
| 7. | Define MOSFET. |  | CO4 | R | 1 |
| 8. | List any two applications of a JFET. |  | CO4 | R | 1 |
| 9. | Sketch the symbol of Schottky barrier diode. |  | CO5 | A | 1 |
| 10. | State any two applications of an LED. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Describe carrier generation and recombination. |  | CO1 | U | 3 |
| 12. | Discuss the current components in a PN diode. |  | CO2 | U | 3 |
| 13. | Explain emitter injection efficiency. |  | CO3 | A | 3 |
| 14. | Explain the construction of a MOS capacitor. |  | CO4 | A | 3 |
| 15. | Describe the working principle of a Zener diode. |  | CO5 | U | 3 |
| 16. | Classify solid-state memories with examples. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Differentiate intrinsic semiconductors from extrinsic semiconductors with respect to carrier concentration and conductivity. | CO1 | U | 4 |
|  | b. | Determine the carrier type and carrier concentration of a semiconductor using the Hall effect principle. | CO1 | A | 8 |
| 18. | a. | Explain the operation of a PN junction under forward and reverse bias. | CO2 | A | 6 |
|  | b. | Compare diffusion capacitance with transition capacitance with suitable diagrams. | CO2 | An | 6 |
| 19. | a. | Explain the working of NPN and PNP transistors with neat diagrams. | CO3 | A | 6 |
|  | b. | Compare CB, CE, and CC configurations based on input and output characteristics. | CO3 | An | 6 |
| 20. | a. | Discuss the static characteristics of JFET and explain various regions of operation. | CO4 | U | 6 |
|  | b. | Describe the operation and characteristics of a UJT with neat sketches. | CO4 | U | 6 |
| 21. |  | Illustrate the construction, operation, and V–I characteristics of a tunnel diode with neat sketches. | CO5 | A | 12 |
| 22. | a. | Explain the construction and working of an Enhancement MOSFET with suitable diagrams and characteristics. | CO4 | A | 6 |
|  | b | Illustrate the construction and operation of a LASER diode. | CO5 | A | 6 |
| 23. |  | Analyze the advantages and limitations of TRIAC and DIAC in power control applications. | CO6 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Discuss the turn-on and turn-off mechanisms of an SCR. | CO6 | U | 6 |
|  | b. | Explain the construction, working, and characteristics of a solar cell. | CO6 | A | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Interpret the effect of electrical parameters on the flow of charge carriers. |
| CO2 | Evaluate the performance of PN diode and Bipolar Junction Transistor in RF applications. |
| CO3 | Choose relevant BJT configuration for various applications. |
| CO4 | Evaluate the performance of FET devices in switching applications. |
| CO5 | Evaluate the performance of power and display devices in industrial applications. |
| CO6 | Choose relevant special semiconductor devices for electronics applications. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 20EC2003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SIGNALS AND SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define the unit step signal and write its mathematical form. |  | CO1 | R | 1 |
| 2. | Calculate the length of the resulting signal obtained after performing linear convolution of two discrete-time signals with lengths 5 and 6. |  | CO1 | A | 1 |
| 3. | List any two properties of convolution sum. |  | CO2 | R | 1 |
| 4. | Distinguish linear from non-linear systems. |  | CO2 | An | 1 |
| 5. | Write the expression for the trigonometric Fourier series. |  | CO3 | R | 1 |
| 6. | State the time shirting property of Continuous Time Fourier Transform. |  | CO3 | R | 1 |
| 7. | Enumerate any two types of sampling. |  | CO4 | R | 1 |
| 8. | State the linearity property of Laplace Transform. |  | CO4 | R | 1 |
| 9. | Compute the Discrete Time Fourier transform for the signal x(n)={1,-2,-1}. |  | CO5 | A | 1 |
| 10. | List any two properties of ROC. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | State the conditions for a signal to be an energy signal or a power signal. |  | CO1 | R | 3 |
| 12. | Analyze whether the system y(t) = x(t-2) + x (2-t) is causal or not. |  | CO2 | An | 3 |
| 13. | List the Dirichlet conditions for the existence of Continuous Time Fourier Transform. |  | CO3 | R | 3 |
| 14. | Calculate the Nyquist rate and Nyquist interval for the signal x(t) = 2 sin (100πt) + 100 cos (200πt). |  | CO4 | A | 3 |
| 15. | Compute the DTFT of the signal x(n)=(1/2)n u(n). |  | CO5 | A | 3 |
| 16. | Determine the Z transform of u[-n] and plot its ROC. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Sketch the following signals from the given input signal x(t). (i) x(t-1) (ii) x(2t+1) (iii) x(-t-1) (iv) x(2t) Given: | CO1 | A | 12 |
| 18. | a. | Check whether the system y(n) = x(n)x(n-1) is Static or dynamic, Causal or non-causal and Time variant or Invariant. | CO2 | An | 6 |
|  | b. | Compute the convolution sum of the system whose impulse response h(n) = {1, 2, - 1, 3} and the input x(n) = {1, 2, 1}. | CO2 | A | 6 |
| 19. | a. | Determine the exponential Fourier series coefficient Cn of the given periodic signal. | CO3 | A | 6 |
|  | b. | State and prove any two properties of Continuous Time Fourier Transform. | CO3 | R | 6 |
| 20. |  | Explain the sampling theorem and the impulse train sampling process with neat diagrams. | CO4 | A | 12 |
| 21. |  | Determine the frequency response and impulse response of a causal system that is represented by the following difference equation using DTFT. yn-16yn-1-16yn-2=x(n) | CO5 | A | 12 |
| 22. |  | State and prove any four properties of Laplace transform. | CO4 | R | 12 |
| 23. |  | Illustrate the process of signal reconstruction with necessary equations and diagrams. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Determine the z transform of x(n) = anu(n) – bnu(-n-1) and find the region of convergence. | CO6 | A | 8 |
|  | b. | Summarize the time scaling property of z-transform. | CO6 | U | 4 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze different types of signals for mathematical modelling. |
| CO2 | Realize the system properties to build basic model. |
| CO3 | Represent continuous time system using Fourier series and Fourier transform. |
| CO4 | Investigate the sampling process and Laplace Transform. |
| CO5 | Signify discrete time system using Fourier series and Fourier transform. |
| CO6 | Familiarize the frequency analysis of discrete time system using Z transform. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 20EC2004 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # COMPUTER ARCHITECTURE | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define register transfer and state its notation with an example. |  | CO1 | R | 1 |
| 2. | State the basic components of an Arithmetic Logic Shift Unit |  | CO1 | R | 1 |
| 3. | List the four types of shift micro-operations. |  | CO1 | U | 1 |
| 4. | What is an instruction code? State its two fields. |  | CO2 | R | 1 |
| 5. | State the function of the Program Counter in a basic computer. |  | CO2 | R | 1 |
| 6. | Define a subroutine and state how it is called in a program. |  | CO3 | U | 1 |
| 7. | Name the memory hierarchy levels in order. |  | CO4 | R | 1 |
| 8. | Define pipelining and state the advantage it offers over sequential execution. |  | CO4 | U | 1 |
| 9. | State any two addressing modes supported by the 8086 microprocessor. |  | CO2 | R | 1 |
| 10. | Cite the function of the 8255 Programmable Peripheral Interface |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the bus and memory transfer operations in a computer with suitable register transfer statements. |  | CO1 | U | 3 |
| 12. | Describe the common bus system of the basic computer and explain the function of each register connected to it. |  | CO2 | U | 3 |
| 13. | Explain general register organization and describe how it differs from a single accumulator organization. |  | CO3 | U | 3 |
| 14. | Describe the organization of main memory. Explain the types of RAM and ROM used in a computer system. |  | CO4 | U | 3 |
| 15. | Explain the signal description of the 8086 microprocessor and classify the signals based on their function. |  | CO2 | An | 3 |
| 16. | Describe the 8251 Serial Communication Interface (USART) and explain its transmitter and receiver sections. |  | CO5 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the different types of arithmetic micro-operations performed in a computer. Describe the design and operation of the Arithmetic Logic Shift Unit with a neat diagram. | CO1 | U | 6 |
|  | b. | Describe the logic micro-operations and shift micro-operations in a basic computer. Explain how these operations are combined and performed in the Arithmetic Logic Shift Unit. | CO1 | An | 6 |
| 18. | a. | Explain the complete instruction set of the basic computer. Categorize the instructions and describe the format of each type with suitable examples. | CO2 | U | 6 |
|  | b. | Discuss the instruction cycle of the basic computer with a flowchart. Trace the register transfer operations involved in fetching and executing a memory reference instruction. | CO2 | An | 6 |
| 19. | a. | Explain control memory and address sequencing in detail. Describe the design of a microprogrammed control unit with a neat block diagram. | CO3 | U | 6 |
|  | b. | Explain the concept of subroutines in the basic computer with a suitable example. Describe the input-output programming technique used in the basic computer. | CO3 | An | 6 |
| 20. | a. | Explain the program control instructions of the basic computer. Describe the types of branch instructions and discuss how they alter the normal flow of execution. | CO3 | A | 6 |
|  | b. | Describe the concept of microprogramming with an example. Explain how a microprogram is written and executed in a microprogrammed control unit. | CO3 | A | 6 |
| 21. | a. | Explain the memory hierarchy with a neat diagram. Discuss the principle of locality of reference and describe how cache memory improves overall system performance. | CO4 | U | 6 |
|  | b. | Describe arithmetic pipeline and instruction pipeline with suitable examples. Explain any two types of pipeline hazards and the methods used to resolve them. | CO4 | An | 6 |
| 22. | a. | Explain the internal architecture of the 8086 microprocessor with a neat diagram. Describe the functions of the Bus Interface Unit and the Execution Unit. | CO2 | U | 6 |
|  | b. | Discuss the maximum mode configuration of the 8086 microprocessor. Explain the purpose of each signal specific to maximum mode and compare it with minimum mode. | CO2 | An | 6 |
| 23. | a. | Explain the working of the 8255 Programmable Peripheral Interface with its internal block diagram. Describe its three operating modes with suitable configuration examples. | CO5 | A | 6 |
|  | b. | Describe the 8254 Programmable Interval Timer with its internal block diagram. Explain its six operating modes and discuss a typical application. | CO5 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain vector processing in parallel computing. Describe the architecture of a vector processor with a neat diagram and discuss how it achieves high-performance computation. | CO4 | An | 6 |
|  | b. | Compare RISC and CISC processor architectures in terms of instruction set complexity, pipeline design, number of registers, memory access, and overall performance. Justify with examples. | CO6 | An | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Illustrate the basics of computer organization including register transfer and micro-operations. |
| CO2 | Outline the architecture, instruction set, and organization of a basic computer and 8086 microprocessor. |
| CO3 | Implement micro operations, microprogramming, subroutines, and program control concepts. |
| CO4 | Formulate memory hierarchy, main memory, cache memory, and pipelining concepts. |
| CO5 | Demonstrate the concepts of I/O interfacing using programmable peripheral devices. |
| CO6 | Outline the importance of parallel processing, vector processing, and RISC architecture. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 20EC2005 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # IoT FOR COMMUNICATION ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Write the technical definition for IoT. |  | CO1 | A | 1 |
| 2. | Identify the role of brokers in communication protocols. |  | CO1 | R | 1 |
| 3. | List the functional elements of measurement system. |  | CO2 | R | 1 |
| 4. | Distinguish a sensor from an actuator. |  | CO2 | An | 1 |
| 5. | Write the significance of Universal Middleware Bridge in IoT. |  | CO3 | A | 1 |
| 6. | Define Ad Hoc wireless networks. |  | CO3 | R | 1 |
| 7. | Interpret the concept of eavesdropping in IoT security. |  | CO4 | U | 1 |
| 8. | Explain clone node attacks in wireless sensor networks. |  | CO4 | U | 1 |
| 9. | Define fog computing. |  | CO5 | R | 1 |
| 10. | List any two applications of an IoT system. |  | CO5 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain any three core IoT technologies. |  | CO1 | A | 3 |
| 12. | List the Operating Frequency Bands of IEEE 802.15.4. |  | CO2 | R | 3 |
| 13. | List the characteristics of wireless sensor networks. |  | CO3 | R | 3 |
| 14. | Describe the three access control models. |  | CO4 | U | 3 |
| 15. | Explain the characteristics fog computing. |  | CO5 | A | 3 |
| 16. | Compare edge computing with fog computing. |  | CO5 | E | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Evaluate the various communication patterns of IoT with necessary diagrams. | CO1 | E | 12 |
| 18. | a. | Illustrate the ZigBee architecture and frame formats with neat sketches and describe in detail. | CO2 | A | 6 |
|  | b. | Illustrate the Functional elements of a measurement system with a block diagram and explain the details. | CO2 | A | 6 |
| 19. | a. | Illustrate and describe the architecture of a WSN node. | CO3 | An | 6 |
|  | b. | Interpret the WSN challenges and constraints in detail. | CO3 | A | 6 |
| 20. |  | Evaluate the various types of attacks that can occur in all the phases of an IoT system. | CO4 | E | 12 |
| 21. |  | Illustrate the OpenStack architecture and explain the details. | CO5 | An | 12 |
| 22. |  | Justify the requirement for Interoperability in IoT systems. Elaborate on the Interoperability types, issues and solution. | CO3 | E | 12 |
| 23. | a. | Examine the characteristics of IoT. | CO1 | A | 6 |
|  | b. | Illustrate the IoT stack and give a brief description of the protocols used in the stack. | CO1 | An | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Construct a block diagram for a smart parking system and explain in detail. | CO5 | A | 6 |
|  | b. | Construct a block diagram for a smart irrigation system and explain in detail. | CO5 | A | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the fundamental concepts of IoT, architecture and communication pattern. |
| CO2 | Analyze the various sensors/actuators and the various IoT protocols. |
| CO3 | Analyze WSN architecture, node behaviour and interoperability issues. |
| CO4 | Analyze the various network security issues and its prevention. |
| CO5 | Apply their understanding and to apply the IoT principles in real time applications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 20EC2008 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # 5G COMMUNICATIONS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Explain the key requirements of 5G communication. |  | CO1 | U | 1 |
| 2. | Define the term LTE. |  | CO1 | R | 1 |
| 3. | State one component of an RF front-end system. |  | CO2 | R | 1 |
| 4. | Explain the importance of millimetre wave (mm Wave) communication. |  | CO2 | U | 1 |
| 5. | Define NOMA. |  | CO3 | R | 1 |
| 6. | Summarize the applications of V2X communication |  | CO3 | U | 1 |
| 7. | Determine the subcarrier spacing for Numerology 1. |  | CO4 | A | 1 |
| 8. | Write about the resource block in OFDM |  | CO4 | A | 1 |
| 9. | Identify the reason for the UEs at the cell border experiencing much lower throughput than those nearer to the transmitting Base Station. |  | CO5 | U | 1 |
| 10. | Infer about the communication that is not directly operated by humans. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Illustrate a handoff scenario at the cell boundary. |  | CO1 | R | 3 |
| 12. | Explain SvC in 5G. |  | CO2 | A | 3 |
| 13. | Compare OFDM with GFDM. |  | CO3 | An | 3 |
| 14. | List the parameters of the channel paths that have to be modelled accurately. |  | CO4 | R | 3 |
| 15. | Discuss JT CoMP. |  | CO5 | U | 3 |
| 16. | List any three requirements of massive MTC. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the protocol architecture of interfacing LTE and New Radio | CO1 | A | 6 |
|  | b. | Illustrate the 5G building blocks. | CO1 | A | 6 |
| 18. |  | Categorize the beamforming architectures used in 5G. | CO2 | An | 12 |
| 19. |  | Construct the SIC-based iteration process for finding the desired signal in NOMA. | CO3 | A | 12 |
| 20. |  | Explain the transmitter and receiver structure of an OFDM system with a neat block diagram. | CO4 | A | 12 |
| 21. | a. | Analyze the importance of Coordinated Multi-Point (CoMP) transmission | CO5 | An | 6 |
|  | b. | Explain Dynamic clustering with an example | CO5 | A | 6 |
| 22. |  | Examine a map-based METIS channel model that provides accurate and realistic spatial channel properties suitable for advanced beamforming. | CO1 | A | 12 |
| 23. |  | Explain the core networks for 5G Non-Standalone and Standalone deployments, with appropriate diagrams. | CO2 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Evaluate the fundamentals of Machine-Type Communication (MTC) in 5G. | CO6 | An | 6 |
|  | b. | Describe the working of Device-to-Device (D2D) communication and its advantages in 5G networks | CO6 | R | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the key part of the evolution to 5G, requirements of 5G, building blocks of 5G and 5G spectrum. |
| CO2 | Explain the Millimeter Wave Communications and massive MIMO. |
| CO3 | Discuss the 5G Radio Access Technologies and Modulation Techniques. |
| CO4 | Outline the significance New Radio Air Interface and wireless propagation channel models. |
| CO5 | Analyze the multi-point transmission and network coding in 5G. |
| CO6 | Acquire basic knowledge on 5G applications like Machine-type communications. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 20EC2008 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # 5G COMMUNICATIONS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the frequency reuse factor for N=7 |  | CO1 | U | 1 |
| 2. | Define the term “Latency” in 5G. |  | CO1 | R | 1 |
| 3. | Identify the importance of interfaces in Cellular Communication |  | CO2 | U | 1 |
| 4. | Identify the materials used in the manufacturing of mm-wave ICs. |  | CO2 | U | 1 |
| 5. | State the disadvantages of OFDM. |  | CO3 | R | 1 |
| 6. | Relate the number of baseband chains with the number of RF chains in a massive MIMO OFDM transmitter employing digital precoding. |  | CO3 | U | 1 |
| 7. | Determine the importance of frequency raster in 5G |  | CO4 | A | 1 |
| 8. | Sketch the 5G NR slot format 2. |  | CO4 | A | 1 |
| 9. | Name the technique used for improving cell edge user throughput. |  | CO5 | R | 1 |
| 10. | State the battery life requirement of massive MTC. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Illustrate various types of blockages in mm-wave propagation. |  | CO1 | An | 3 |
| 12. | Explain the functions of VRAN. |  | CO2 | U | 3 |
| 13. | Compare OFDM with NOMA |  | CO3 | An | 3 |
| 14. | Calculate subcarrier spacing and frame duration for Numerology 2 |  | CO4 | A | 3 |
| 15. | Explain the use of relays in multipoint transmission. |  | CO5 | An | 3 |
| 16. | Interpret the techniques used to achieve 1ms delay. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the role of standardization organizations in the development and regulation of 5G systems. | CO1 | U | 6 |
|  | b. | Illustrate the architecture of 5G with a neat diagram. | CO1 | A | 6 |
| 18. |  | Explain the role of FDD MIMO and TDD MIMO in achieving reliable communication. | CO2 | A | 12 |
| 19. |  | Explain the discrete representation of an OFDM system with a suitable block diagram. | CO3 | A | 12 |
| 20. |  | Illustrate a map-based METIS channel model that provides accurate and realistic spatial channel properties suitable for advanced beamforming. | CO4 | A | 12 |
| 21. | a. | Explain clustering and interference floor shaping in signal transmission. | CO5 | U | 6 |
|  | b. | Illustrate a Massive MIMO transmitter, employing hybrid digital precoding and analog beamforming. | CO5 | A | 6 |
| 22. |  | Explain the two types of 5G NR architecture with necessary diagrams. | CO1 | U | 12 |
| 23. |  | Compare Hybrid beamforming architecture with Digital beamforming architecture with suitable figures. | CO2 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | List the use cases of low-rate MTC. | CO6 | R | 6 |
|  | b. | Explain the new challenges in 5G modelling | CO6 | R | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the key part of the evolution to 5G, requirements of 5G, building blocks of 5G and 5G spectrum. |
| CO2 | Explain the Millimeter Wave Communications and massive MIMO. |
| CO3 | Discuss the 5G Radio Access Technologies and Modulation Techniques. |
| CO4 | Outline the significance New Radio Air Interface and wireless propagation channel models. |
| CO5 | Analyze the multi-point transmission and network coding in 5G. |
| CO6 | Acquire basic knowledge on 5G applications like Machine-type communications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 20EC2009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ARTIFICIAL NEURAL NETWORKS AND DEEP LEARNING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | How are humans different from computers? |  | CO1 | R | 1 |
| 2. | Sketch a simple biological neuron and label it. |  | CO1 | A | 1 |
| 3. | List any 3 different activation functions used in ANN. |  | CO2 | R | 1 |
| 4. | Find the total weights required for an artificial neural network with 8 input neurons and 4 output neurons. |  | CO2 | An | 1 |
| 5. | Compare discrete and continuous perceptron. |  | CO3 | An | 1 |
| 6. | Identify limitation of single-layer perceptron network. |  | CO3 | U | 1 |
| 7. | Outline the process of backpropagation training in a neural network. |  | CO4 | R | 1 |
| 8. | Compare forward and backward pass. |  | CO4 | An | 1 |
| 9. | Which method would you use to minimize overfitting in deep learning models? |  | CO5 | A | 1 |
| 10. | State any two real life applications of deep learning. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate biological and artificial neural networks based on structure, learning, and processing. |  | CO1 | An | 3 |
| 12. | Summarize the different categories in the taxonomy of ANN. |  | CO2 | U | 3 |
| 13. | Analyze the limitations of perceptron models in solving complex problems. |  | CO3 | An | 3 |
| 14. | Evaluate the effectiveness of the Generalized Delta Rule in updating neural network weights. |  | CO4 | E | 3 |
| 15. | Explain the concept of eigenvalues and eigenvectors. |  | CO5 | U | 3 |
| 16. | Illustrate the encoding and decoding process in an Autoencoder. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Apply the McCulloch–Pitts neuron model to implement the AND logic function by designing the neuron architecture and algorithm steps. | CO1 | A | 12 |
| 18. |  | Explain in detail about supervised, unsupervised, and reinforcement learning. | CO2 | U | 12 |
| 19. |  | Demonstrate the implementation of perceptron learning with an example. | CO3 | A | 12 |
| 20. |  | Design a step-by-step backpropagation training algorithm for a feedforward neural network consisting of one input layer, one hidden layer, and one output layer. | CO4 | C | 12 |
| 21. |  | Describe the architecture of deep feedforward neural networks. | CO5 | U | 12 |
| 22. |  | Enumerate the architecture and working of Convolutional Neural Networks. | CO6 | U | 12 |
| 23. | a. | Explain the architecture and working of the Hopfield network. | CO3 | U | 10 |
|  | b. | List out the applications of ANN | CO1 | U | 2 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Illustrate how deep learning networks can be used in real-world applications. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compare and comprehend the functioning of human brain and ANN |
| CO2 | Gain an understanding about training methodologies of neural networks |
| CO3 | Summarize the pros and cons of different single layer ANN |
| CO4 | Apply artificial neural networks for solving engineering problems |
| CO5 | Outline the basic concepts and applications of deep learning |
| CO6 | Make use of different Deep networks for real time applications |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 20EC2010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # VLSI FOR IOT SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List the four basic building blocks of an IoT system. |  | CO1 | R | 1 |
| 2. | Differentiate a Sensor from an Actuator. |  | CO2 | U | 1 |
| 3. | State the three main layers of a standard IoT Architecture. |  | CO3 | R | 1 |
| 4. | Explain any one use of SoC in IoT. |  | CO4 | U | 1 |
| 5. | Identify the type of processor suitable for a low-power IoT device. |  | CO3 | R | 1 |
| 6. | Apply the concept of SoC by selecting components required for a wearable IoT device. |  | CO3 | A | 1 |
| 7. | Identify the appropriate power management technique for a battery-operated IoT system. |  | CO5 | R | 1 |
| 8. | Determine the total memory required for one day an IoT device stores 1 reading per second, where each reading is 16 bytes. |  | CO4 | A | 1 |
| 9. | Identify a method to reduce EMI/EMC issues in IoT hardware. |  | CO5 | R | 1 |
| 10. | Enumerate an appropriate security mechanism for protecting IoT devices. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Apply the IoT layered architecture to design a smart parking system. Identify any three layers and their functions. |  | CO1 | A | 3 |
| 12. | Select appropriate sensors and connectivity technologies for a smart agriculture system. |  | CO2 | R | 3 |
| 13. | In a smart IoT monitoring system, a sensor transmits environmental data at 200 bytes per second to a cloud server. Determine the data transmission rate in bits per second. |  | CO3 | A | 3 |
| 14. | Differentiate EEPROM from EPROM with respect to architecture and working principle. |  | CO4 | U | 3 |
| 15. | Apply the concept of power management LDO in designing a low-power IoT device for remote applications. |  | CO5 | A | 3 |
| 16. | Evaluate different connectivity technologies such as Wi-Fi, BLE, Zigbee and recommend the best option for a smart home application with justification. |  | CO6 | E | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Analyse the limitations of legacy systems and explain the IoT architecture overcomes these limitations in smart city applications. | An | CO1 | 6 |
|  | b. | Apply an industrial automation scenario, select appropriate sensors, computing nodes, and connectivity technologies. Justify your choices. | A | CO2 | 6 |
| 18. | a. | Explain the components of IoT systems (sensors, connectivity, computing nodes) and explain their interaction in a real-time application. | U | CO2 | 7 |
|  | b. | Analyze the suitability of different sensor types for a wearable health monitoring system and justify your selection. | An | CO4 | 5 |
| 19. |  | Illustrate the 5-layer IoT architecture with a suitable real-time application and explain how data flows across layers. | U | CO3 | 12 |
| 20. | a. | Apply IoT architecture to design a smart agriculture system, detailing sensors, processing units, communication modules, and data flow. | A | CO4 | 6 |
|  | b | In an IoT product development, the Non-Recurring Engineering (NRE) cost is Rs. 5,00,000, and the per unit manufacturing cost is Rs. 5,000. - Calculate the total cost for manufacturing 100 units. - Determine the cost per unit when 100 units are produced. - Comment the per unit cost changes with increase in production volume. | A | CO3 | 6 |
| 21. | a. | Evaluate various power management units (PMUs) in IoT devices and their impact on system efficiency and battery life. | E | CO4 | 7 |
|  | b. | Apply power management techniques to design a low-power IoT sensor node and explain the role of LDO. | A | CO4 | 5 |
| 22. |  | Analyze the memory architecture (NVM, embedded NVM, OTP) used in IoT devices and compare their performance and suitability. | An | CO3 | 12 |
| 23. | a. | Develop a secure VLSI IoT system architecture, incorporating hardware security, encryption, and reliability mechanisms. | A | CO5 | 7 |
|  | b. | Apply power management techniques to design a low-power IoT sensor node and explain the role of LDO. | A | CO5 | 5 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Apply the guidelines and prevailing standards to design a reliable VLSI IoT system, considering operating conditions and component selection. | A | CO6 | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Gain an understanding of IoT framework. |
| CO2 | Compare and comprehend features of IoT-based systems. |
| CO3 | Summarize the use of IC technology for IoT. |
| CO4 | Apply IC technology for designing IoT devices. |
| CO5 | Outline electronic system design concepts for IoT. |
| CO6 | Use guidelines and prevailing standards to design IoT systems. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 20EC2018 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # FUNDAMENTALS OF PRINTED CIRCUIT AND ARDUINO BOARD DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List any two applications of lead acid battery. |  | CO1 | R | 1 |
| 2. | Identify an example of an electromechanical switch. |  | CO1 | R | 1 |
| 3. | Classify active electronic components. |  | CO2 | U | 1 |
| 4. | State the types of bias conditions in a PN junction diode. |  | CO2 | R | 1 |
| 5. | Sketch the symbol of NOR gate. |  | CO3 | A | 1 |
| 6. | Classify sensors. |  | CO3 | U | 1 |
| 7. | Identify the metal foil used to create conductive layers in a printed circuit board. |  | CO4 | R | 1 |
| 8. | Define Netlist. |  | CO4 | R | 1 |
| 9. | Illustrate the significance of the address bus in a microcontroller. |  | CO5 | U | 1 |
| 10. | State the important salient features of the Internet of Things (IoT). |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Analyze the energy consumption per month (30days) of 60W ceiling fan which operates continuously for 8 hours per day. |  | CO1 | An | 3 |
| 12. | Describe the formation of P-type semiconductor material. |  | CO2 | U | 3 |
| 13. | Identify the three primary logic gates and explain their operations using truth tables. |  | CO3 | R | 3 |
| 14. | Analyze the major issues encountered in modern PCB design. |  | CO4 | An | 3 |
| 15. | Analyze the significance of the Arduino IDE in microcontroller programming. |  | CO5 | An | 3 |
| 16. | List the hardware components needed for an obstacle avoidance robot. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the conversion of heat energy into electrical energy in a thermal power plant using a neat block diagram. | CO1 | A | 12 |
| 18. |  | Illustrate the construction of a PN junction diode using a neat diagram and discuss forward biasing with necessary diagrams. | CO2 | A | 12 |
| 19. | a. | Sketch the block diagram of a basic microcontroller. | CO3 | A | 6 |
|  | b. | Describe the fundamental blocks of combinational and sequential circuits and list their differences. | CO3 | U | 6 |
| 20. | a. | Classify PCBs into their three main types and explain each type. | CO4 | A | 6 |
|  | b. | Analyze the need for multilayer PCBs and explain the advantages over single-layer and double-layer PCBs. | CO4 | An | 6 |
| 21. |  | Explain the architecture of an Arduino board with the help of a neat block diagram. | CO5 | A | 12 |
| 22. | a. | Explain the operation of a transistor as a switch. | CO2 | A | 6 |
|  | b. | Sketch the logic diagram of full adder with its truth table. | CO2 | A | 6 |
| 23. | a. | Discuss the different types of IC packages and their applications. | CO4 | U | 6 |
|  | b. | Describe the PCB fabrication process with suitable illustrations. | CO5 | U | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Develop an automatic room ventilation system that senses the room temperature and controls a fan/cooling system accordingly. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the importance of electro mechanical systems in everyday life. |
| CO2 | Understand the properties of semiconductor devices. |
| CO3 | Understand and relate various digital concepts and circuits. |
| CO4 | Acquire the knowledge about the packages of electronic components, types of PCBs and history of PCBs |
| CO5 | Understand the design and programming of Arduino boards. |
| CO6 | Acquire the knowledge and skills to implement various smart systems application. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 20EC3002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # LOW POWER VLSI DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. |  | Analyze the sources of power dissipation in CMOS circuits and evaluate the contribution of dynamic, short-circuit, and leakage power components. | CO1 | An | 16 |
| 2. |  | Illustrate simulation-based power estimation techniques and evaluate their accuracy in VLSI systems. | CO2 | An | 16 |
| 3. |  | Correlate circuit-level power reduction techniques including transistor sizing and gate reorganization. | CO3 | An | 16 |
| 4. |  | Analyze system-level power optimization techniques and evaluate their impact on performance. | CO4 | An | 16 |
| 5. |  | Examine SRAM architecture and evaluate techniques for reducing power in memory design. | CO5 | A | 16 |
| 6. |  | Evaluate the design and operation of adiabatic logic gates and assess their performance. | CO6 | An | 16 |
| 7. |  | Examine MOSFET capacitances and evaluate their influence on switching speed and power consumption. | CO1 | A | 16 |
| PART – B (1 X 20 = 20 MARKS) [COMPULSORY QUESTION] |  |  |  |  |  |
| 8. |  | Analyze the principles of adiabatic logic and evaluate its advantages over conventional CMOS circuits. | CO6 | An | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the various sources of power dissipation. |
| CO2 | Acquire the knowledge on simulation power analysis and Probabilistic power analysis Techniques. |
| CO3 | Learn the various low power reduction techniques at circuit and logic level. |
| CO4 | Analyze the various low power techniques at Architecture and system level. |
| CO5 | Design low power clock Networks, bus and low power SRAM circuits. |
| CO6 | Design various adiabatic logic circuits. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 20EC3004 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SOLAR CELLS AND THEIR APPLICATIONS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Evaluate the principle of photovoltaic energy conversion in a solar cell and assess the impact of semiconductor properties on its efficiency and overall performance. | CO1 | An | 20 |
|  |  | (OR) |  |  |  |
| 2. | a. | Examine the concept of Maximum Power Point (MPP) in solar cells and analyze the methods used for Maximum Power Point Tracking. | CO2 | A | 10 |
|  | b. | Determine the expressions for solar cell efficiency and fill factor, and interpret their influence on photovoltaic module performance. | CO2 | A | 10 |
| 3. |  | Analyze microcrystalline silicon as a material for solar cells by examining its fabrication methods and material characterization techniques, and evaluate its advantages in solar cell applications. | CO3 | An | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | Illustrate the fabrication process of Cadmium Telluride thin-film solar cells and examine the advanced characterization techniques used to evaluate their performance. | CO4 | A | 20 |
| 5. | a. | Explain the working principle and photovoltaic performance of dye-sensitized solar cells. | CO5 | An | 10 |
|  | b. | Compare solid-state dye-sensitized solar cells with liquid electrolyte-based DSSCs in terms of structure, performance, and stability. | CO5 | An | 10 |
|  |  | (OR) |  |  |  |
| 6. |  | Examine microcrystalline silicon as a solar cell material by discussing its fabrication methods and material characterization techniques, and assess its advantages in photovoltaic applications. | CO2 | A | 20 |
| 7. | a. | Explain the material properties of Cadmium Telluride and discuss their suitability for thin-film solar cell applications. | CO4 | A | 10 |
|  | b. | Illustrate the structure and explain the working principle of Cadmium Telluride-based solar cells. | CO4 | A | 10 |
|  |  | (OR) |  |  |  |
| 8. |  | Design a suitable material system for solar cells used in space applications, considering key performance parameters under space environmental conditions. | CO6 | C | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Explain the effect of operating temperature on the open-circuit voltage of a solar cell. | CO6 | A | 10 |
|  | b. | Explain the structure and working principle of multijunction solar cells used in space applications. | CO6 | A | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate the basics of solar cells. |
| CO2 | Estimate and analyse the performance characteristics of solar cells. |
| CO3 | Evaluate the characteristics of different silicon solar cells. |
| CO4 | Assess the performance of Cadmium Telluride Thin Film Solar Cells. |
| CO5 | Analyze the performance of Dye Sensitized Solar Cell and Polymer Organic Thin-Film Solar cells. |
| CO6 | Demonstrate knowledge of solar cells for space applications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 20EC3007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # BIO MEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Explain the driving forces behind biomedical applications of MEMS and illustrate their influence on device miniaturization. | CO1 | A | 10 |
|  | b. | Develop a fabrication procedure using soft lithography techniques for producing a microfluidic channel. | CO1 | A | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Construct a microfluidic system incorporating micro valves and illustrate its operation for controlling fluid flow. | CO2 | A | 10 |
|  | b. | Construct a detailed model of electrokinetic phenomena in microchannels and explain its applications. | CO2 | A | 10 |
| 3. |  | Analyze the working principles of optical fiber sensors, piezoelectric sensors, and Surface Acoustic Wave (SAW) devices by comparing and differentiating their operating mechanisms and evaluating their suitability for medical applications. | CO3 | An | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | Explain the working principles of micro actuators and their role in microfluidic systems, and examine the various activation methods used in micro actuators to evaluate their suitability for biomedical applications. | CO4 | A | 20 |
| 5. |  | Analyze the concept of Lab-on-Chip (LOC) technology and the working principle of Capillary Electrophoresis (CE) arrays in micro total analysis systems by explaining their operation and evaluating their role in miniaturized biochemical analysis. | CO5 | An | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Design a micro sensor system by explaining the basic principles of sensors and microsensors, developing suitable fabrication techniques, and evaluating their impact on sensor performance in biomedical applications. | CO3 | C | 20 |
| 7. |  | Illustrate the equivalent circuit representation of micro actuators and analyze the working principles and biomedical applications of micro actuator-based drug delivery systems. | CO4 | A | 20 |
|  |  | (OR) |  |  |  |
| 8. | a. | Illustrate the various detection and measurement methods used in emerging Bio-MEMS technology with suitable examples. | CO6 | A | 10 |
|  | b. | Interpret the importance of packaging in Bio-MEMS devices. | CO6 | A | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Analyze the power requirements, data transmission methods, and RF safety considerations in Bio-MEMS systems by evaluating their impact on system performance, and explain the concept of biocompatibility to infer the need for standards in Bio-MEMS technology. | CO6 | An | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the MEMS applications in Bio Medical Engineering. |
| CO2 | Gain knowledge on the Micro fluidic Principles and its applications. |
| CO3 | Gain knowledge in the applications of Sensors in Health Engineering. |
| CO4 | Understand the principles of Micro Actuators and Drug Delivery system. |
| CO5 | Learn the principles and applications of Micro Total Analysis. |
| CO6 | Explore the emerging Bio-MEMS technology. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21EC1003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # PROBLEM SOLVING AND ALGORITHMIC THINKING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the error and correct it. #include <stdio.h> int main() { int a = 10 printf("%d", a); } |  | CO6 | U | 1 |
| 2. | Construct a statement in C to display: “Welcome to Algorithmic Thinking” |  | CO3 | A | 1 |
| 3. | Classify the construct as Sequencing / Selection / Iteration: if (marks >= 50) printf("Pass"); |  | CO1 | U | 1 |
| 4. | Detect the logical error and correct it. for(int i=1; i<5; i--) printf("%d", i); |  | CO6 | U | 1 |
| 5. | Define abstraction with a simple example. |  | CO2 | U | 1 |
| 6. | Differentiate recursion from iteration |  | CO3 | U | 1 |
| 7. | Relate stack concept with an example |  | CO4 | U | 1 |
| 8. | Compare Searching with Sorting |  | CO4 | U | 1 |
| 9. | State the purpose of variables in programming. |  | CO2 | U | 1 |
| 10. | Identify the type of construct used: while(n > 0) { n--; } |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Illustrate the Input–Process–Output model with a simple example. |  | CO1 | U | 3 |
| 12. | Construct a C program to print multiplication table of a number. |  | CO2 | A | 3 |
| 13. | Illustrate sequence, selection, and iteration with examples. |  | CO3 | U | 3 |
| 14. | Compare compile-time with run-time errors with examples. |  | CO4 | An | 3 |
| 15. | Illustrate Arrays with a simple use case. |  | CO5 | U | 3 |
| 16. | Distinguish time complexity from space complexity. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Develop a C program to find the largest of three numbers. | CO3 | A | 12 |
| 18. |  | Construct a flowchart to represent ATM withdrawal process. | CO1 | A | 12 |
| 19. |  | Illustrate algorithmic thinking with an Industrial Automation example. | CO2 | U | 12 |
| 20. |  | Illustrate the working of stack and queue with examples. | CO4 | U | 12 |
| 21. | a. | Explain Bubble Sort with a simple example. | CO5 | U | 6 |
|  | b. | Demonstrate Linear Search with steps. | CO6 | U | 6 |
| 22. |  | Develop a C program to store student details using structures and display them. | CO3 | A | 12 |
| 23. |  | Apply a searching technique to find an element in a list and explain the steps. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Construct a C program to add two 2×2 matrices using arrays. | CO3 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply algorithmic thinking to understand, define and solve problems |
| CO2 | Design and implement algorithm(s) for a given problem |
| CO3 | Apply the basic programming constructs for problem solving |
| CO4 | Understand an algorithm by tracing its computational states |
| CO5 | Identifying bugs in algorithms |
| CO6 | Analyze the bugs and correct them |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC1003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # PROBLEM SOLVING AND ALGORITHMIC THINKING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the error and correct it. #include <stdio.h> int main() { float a = 10 printf("%d", a) } |  | CO6 | U | 1 |
| 2. | Construct a statement in C to display: “Programming is Fun” |  | CO3 | A | 1 |
| 3. | Classify the construct as Sequencing / Selection / Iteration: switch(choice) { case 1: printf("Add"); break; } |  | CO1 | U | 1 |
| 4. | Detect the logical error and correct it. (Suppose the intention is to print odd numbers from 1 to 9) for(int i = 0; i <= 10; i += 2) printf("%d ", i); |  | CO6 | U | 1 |
| 5. | Define decomposition with a simple example. |  | CO2 | U | 1 |
| 6. | Differentiate Compiler from Interpreter. |  | CO3 | U | 1 |
| 7. | Relate Queue concept with an example |  | CO4 | U | 1 |
| 8. | Infer: Sorting |  | CO4 | U | 1 |
| 9. | State the purpose of Loops in programming. |  | CO2 | U | 1 |
| 10. | Identify the type of construct used: do { n--; } while(n > 0); |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Illustrate the concept of Pseudocode with a simple example. |  | CO1 | U | 3 |
| 12. | Construct a C program to find the smallest of two numbers. |  | CO2 | A | 3 |
| 13. | Illustrate the use of decision-making statements (if, if-else) with examples. |  | CO3 | U | 3 |
| 14. | Compare syntax errors and semantic errors with examples. |  | CO4 | An | 3 |
| 15. | Illustrate Functions in C with a simple use case. |  | CO5 | U | 3 |
| 16. | Distinguish between Linear Search and Binary Search. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Develop a C program to check whether a number is positive, negative, or zero. | CO3 | A | 12 |
| 18. |  | Construct a flowchart to determine whether a student has passed or failed based on marks obtained. | CO1 | A | 12 |
| 19. |  | Illustrate algorithmic thinking with a Smart Traffic Control System example. | CO2 | U | 12 |
| 20. | a. | Illustrate the working of a stack using the browser Back button functionality. | CO4 | U | 6 |
|  | b. | Illustrate the working of a queue in a ticket reservation counter scenario. | CO4 | U | 6 |
| 21. | a. | A teacher wants to arrange students' marks from lowest to highest. Explain how Bubble Sort can be used to perform the sorting. | CO5 | U | 6 |
|  | b. | A railway reservation system searches for a passenger's ticket number in a list. Demonstrate the steps involved in Linear Search. | CO6 | U | 6 |
| 22. |  | Develop a C program to store product details using structures and display all records. | CO3 | A | 12 |
| 23. |  | Apply Linear Search to locate a student roll number in a class list and explain the search process. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Construct a C program to subtract two 2×2 matrices using arrays. | CO3 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply algorithmic thinking to understand, define and solve problems |
| CO2 | Design and implement algorithm(s) for a given problem |
| CO3 | Apply the basic programming constructs for problem solving |
| CO4 | Understand an algorithm by tracing its computational states |
| CO5 | Identifying bugs in algorithms |
| CO6 | Analyze the bugs and correct them |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # INTROCUCTION TO COMPUTER ENGINEERING | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # 21EC1006 | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State the main function of a Central Processing Unit (CPU) |  | CO1 | R | 1 |
| 2. | Define a system bus in a computer. |  | CO1 | R | 1 |
| 3. | State the primary purpose of the Accumulator register. |  | CO2 | U | 1 |
| 4. | Give one example of a logical operation performed by the ALU. |  | CO2 | R | 1 |
| 5. | List the two main types of control units. |  | CO3 | R | 1 |
| 6. | Define an 'opcode' in an instruction format. |  | CO3 | R | 1 |
| 7. | Expand the acronym MIMD. |  | CO4 | R | 1 |
| 8. | Define parallel processing in simple terms. |  | CO4 | R | 1 |
| 9. | Cite the primary purpose of an I/O module. |  | CO5 | U | 1 |
| 10. | Give one example of primary memory. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Distinguish a ‘Data Bus’ from an ‘Address Bus’. |  | CO1 | An | 3 |
| 12. | Describe the basic purpose of a Flag Register. |  | CO2 | U | 3 |
| 13. | Define a micro-instruction in the context of a control unit. |  | CO3 | R | 3 |
| 14. | Briefly describe data-level parallelism. |  | CO4 | U | 3 |
| 15. | State one major advantage of using Direct Memory Access (DMA). |  | CO5 | U | 3 |
| 16. | Describe the role of Cache memory in a computer system. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the basic functional units of a computer system with a neat block diagram. | CO1 | U | 12 |
| 18. |  | Explain any four addressing modes and provide one simple example for each. | CO2 | U | 12 |
| 19. |  | Discuss the standard phases of a basic instruction cycle (Fetch, Decode, Execute). | CO3 | U | 12 |
| 20. |  | Explain Flynn’s classification of computer architectures (SISD, SIMD, MISD, MIMD). | CO4 | An | 12 |
| 21. |  | Describe the standard functions and structure of an I/O module. | CO5 | U | 12 |
| 22. |  | What is a flag register? Explain the function of the Zero Flag, Sign Flag, and Carry Flag. | CO2 | An | 12 |
| 23. |  | Describe the basic components of a DMA controller and outline the steps involved in a DMA data transfer. | CO5 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Describe the Memory Hierarchy design and explain why it is necessary in a computer system. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basics structure of computers, operations and instructions |
| CO2 | Design arithmetic and logic unit |
| CO3 | Understand pipelined execution and design control unit |
| CO4 | Understand parallel processing architectures |
| CO5 | Understand the various memory systems and I/O communication |
| CO6 | Design Memory Systems |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC1007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SOFTWARE ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Describe the need for software engineering. |  | CO1 | U | 1 |
| 2. | List two embedded software. |  | CO1 | R | 1 |
| 3. | Define Software Development Life Cycle (SDLC). |  | CO2 | R | 1 |
| 4. | Describe non -functional requirements in software development. |  | CO2 | U | 1 |
| 5. | Explain software design. |  | CO3 | U | 1 |
| 6. | List the two key activities of software design. |  | CO3 | R | 1 |
| 7. | Describe the need for consistency in user interface design. |  | CO4 | U | 1 |
| 8. | Name two navigation mechanisms used in webapps. |  | CO4 | R | 1 |
| 9. | Define alpha testing. |  | CO5 | R | 1 |
| 10. | Describe software measurement. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the purpose of umbrella activities in software engineering. |  | CO1 | U | 3 |
| 12. | Differentiate ‘functional requirements’ from ‘non-functional requirements’. |  | CO2 | An | 3 |
| 13. | Explain the role of Theo Mandel's Golden Rules in designing a user-friendly web application. |  | CO3 | A | 3 |
| 14. | Analyze the role of breadcrumbs in enhancing navigation within a WebApp. |  | CO4 | An | 3 |
| 15. | Differentiate ‘testing’ from ‘debugging’ based on their purpose and outcomes. |  | CO5 | An | 3 |
| 16. | Examine the contribution of functional independence to effective software design |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Compare generic process model with prescriptive process model for software development process. | CO1 | An | 12 |
| 18. |  | Illustrate the application of the Requirements Engineering (RE) process stages in identifying functional and non-functional requirements of a software system. | CO2 | A | 12 |
| 19. |  | Analyze the role of architectural mapping in the software design process and examine how it transforms software requirements into an effective software architecture. | CO3 | An | 12 |
| 20. |  | Discuss the four major activities of the User Interface Design Process and their significance in developing user-friendly software. | CO4 | U | 12 |
| 21. |  | Illustrate the application of validation testing and strategic testing approaches during software testing to improve software reliability and performance. | CO5 | A | 12 |
| 22. | a. | Compare and contrast system software and application software with suitable examples. | CO1 | An | 6 |
|  | b. | Discuss the significance of design concepts in software engineering | CO3 | U | 6 |
| 23. | a. | Demonstrate the use of appropriate navigation mechanisms in designing a web application for a college management system. | CO4 | A | 6 |
|  | b. | Discuss the characteristics of system software and application software with examples. | CO5 | U | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze the Risk Management Process and examine the significance of each stage in minimizing software project failures. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Explain the fundamental concepts of software engineering and agile development methodologies. |
| CO2 | Analyze and validate software requirements for diverse application domains. |
| CO3 | Apply appropriate software design methodologies to develop effective software solutions. |
| CO4 | Perform software testing and debugging techniques to ensure software quality and reliability. |
| CO5 | Utilize software maintenance and project management practices throughout the software development lifecycle. |
| CO6 | Evaluate software products for compliance with established quality standards and best practices. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRONIC DEVICES AND CIRCUITS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the position of the Fermi energy level in an intrinsic semiconductor. |  | CO1 | R | 1 |
| 2. | Infer the effect of increasing temperature on the conductivity of a semiconductor. |  | CO1 | U | 1 |
| 3. | Classify the BJT configuration that provides the highest current gain among CE, CB, and CC circuits. |  | CO2 | U | 1 |
| 4. | Interpret the effect of increasing gate-to-source voltage on the drain current of an n-channel JFET. |  | CO2 | A | 1 |
| 5. | Differentiate a photodiode from a phototransistor based on their mode of operation. |  | CO3 | U | 1 |
| 6. | Determine the effect of filtering on the ripple content of a rectifier output. |  | CO3 | A | 1 |
| 7. | Identify the rectifier that utilizes both half cycles of the input AC signal. |  | CO4 | R | 1 |
| 8. | Define the stability factor of a transistor biasing circuit. |  | CO4 | R | 1 |
| 9. | The voltage-divider biasing provides good stability in a BJT.Justify. |  | CO5 | An | 1 |
| 10. | List the types of feedback amplifier. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare drift current with diffusion current in semiconductors based on their origin and carrier movement. |  | CO1 | U | 3 |
| 12. | Sketch the shape of the output voltage waveform for this “clipper” circuit, assuming an ideal diode with no forward voltage drop: |  | CO2 | A | 3 |
| 13. | Interpret the accumulation, depletion, and inversion regions of a MOS capacitor. |  | CO3 | A | 3 |
| 14. | Justify the use of voltage regulators in electronic power supplies. |  | CO4 | An | 3 |
| 15. | Explain the need for biasing in a transistor amplifier. |  | CO5 | U | 3 |
| 16. | A differential amplifier has a differential gain of 1000 and a common-mode gain of 2. Calculate the CMRR. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Analyze the various mechanisms of electron-hole generation and recombination in semiconductors and distinguish their characteristics. | CO1 | An | 12 |
| 18. |  | Interpret the DC behavior of a PN junction diode under forward-biased and reverse-biased conditions using its V–I characteristics. | CO2 | A | 12 |
| 19. |  | Illustrate the structure and working principle of a JFET and analyze its drain and transfer characteristics. | CO2 | A | 12 |
| 20. | a. | Explain the structure, working principle, and V–I characteristics of a Schottky barrier diode. | CO3 | U | 6 |
|  | b. | Interpret the V–I characteristics of an SCR and explain the various operating regions. | CO3 | A | 6 |
| 21. |  | Explain the working principle of a full-wave rectifier and determine its ripple factor and rectification efficiency. | CO4 | U | 12 |
| 22. |  | Interpret the functioning of an RC-coupled amplifier and sketch its frequency response. | CO5 | A | 12 |
| 23. |  | Derive the expression for the efficiency of a Class A power amplifier and determine its maximum theoretical efficiency. | CO5 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the operation of a single tuned amplifier with a neat frequency response curve. | CO6 | U | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the basic properties of semiconductor physics |
| CO2 | Identify and differentiate the functioning of diode, BJT and FET. |
| CO3 | Define the fundamental operation principles and applications of special semiconductor devices. |
| CO4 | Demonstrate the functioning of DC Power supply. |
| CO5 | Analyze the biasing property and frequency response of amplifier circuits. |
| CO6 | Distinguish between amplifiers and oscillators. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2006 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # MATHEMATICS FOR SIGNAL ANALYSIS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Determine whether the following signal is causal or not: x(n)=-u(n−2). |  | CO1 | A | 1 |
| 2. | Sketch the signalx(t)=2r(t). |  | CO1 | A | 1 |
| 3. | Define Linear system. |  | CO2 | R | 1 |
| 4. | Test whether the following system is static or dynamic: y(n) = 2x(n-8). |  | CO2 | An | 1 |
| 5. | Write equation of the trigonometric Fourier series. |  | CO3 | A | 1 |
| 6. | State the convolution property of the Continuous-Time Fourier Transform (CTFT). |  | CO3 | R | 1 |
| 7. | Define sampling. |  | CO4 | R | 1 |
| 8. | State the initial value theorem of the Laplace Transform. |  | CO4 | R | 1 |
| 9. | Calculate the Fourier Transform of δ[n]. |  | CO5 | A | 1 |
| 10. | Define Region of Convergence (ROC) of the Z Transform. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compute the even and odd component of the signal x(n) = n2 |  | CO1 | A | 3 |
| 12. | List the properties of the convolution sum. |  | CO2 | R | 3 |
| 13. | Write the conditions for the existence of the Fourier Series. |  | CO3 | A | 3 |
| 14. | Compute the Nyquist rate and Nyquist interval for the following message signal: m(t)= (cos1000πt.cos (100πt)). |  | CO4 | A | 3 |
| 15. | Evaluate the convolution property of the Discrete-Time Fourier Transform (DTFT). |  | CO5 | E | 3 |
| 16. | Determine the Z transform of the discrete time signal x(n)={3,7,1,2,3}. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Sketch th following signals derived from the Discrete-Time (DT) unit step signal u(n). - u(n-1) (ii) 2u(n+2) (iii) u(-2n+1) (iv) u(n)+u(-n) | CO1 | A | 12 |
| 18. | a. | Test the properties of the system yt=2x2t a) Static or Dynamic (b) Linear or Non-linear (c) c) Time invariant or variant d) Causal or Non- causal | CO2 | An | 8 |
|  | b. | Determine the response of the CT system, given the input xt=e2tu(t)and the impulse response ht=ut). | CO2 | A | 4 |
| 19. | a. | Calculate the trigonometric Fourier series for the given signal with a period T=4 seconds, defined as: xt=1 ; -1 ≤t≤1-1 ; 1<t≤30 ; otherwise | CO3 | An | 8 |
|  | b. | Determine the continuous time Fourier transform of xt=e-atu(t). | CO3 | A | 4 |
| 20. |  | Explain impulse train sampling and the sampling theorem, including neat diagrams. | CO4 | A | 12 |
| 21. |  | Explain the time-shifting, time-scaling, and time-reversal properties of the DTFT. | CO5 | A | 12 |
| 22. |  | Evaluate the transfer function and impulse response of a causal system represented by the differential equation d2y(t)dt2+11dy(t)d(t)+24yt=5dx(t)d(t)+3xt. | CO3 | E | 12 |
| 23. |  | Determine the output response of a system using the graphical method, given the input x(n) = {1,-4,2, -2} and the impulse response hn={1,2,3}. | CO2 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Apply the long division method to determine the inverse Z transform of Xz=1+4z-11-4z-1+z-2 (i) x(n) is causal (ii) x(n) is non-causal | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze different types of matrices, signals for mathematical modelling |
| CO2 | Realize the system properties to build basic model |
| CO3 | Represent continuous time system using Fourier series and Fourier transform |
| CO4 | Investigate the sampling process and Laplace Transform |
| CO5 | Signify discrete time system using Fourier series and Fourier transform |
| CO6 | Familiarize the frequency analysis of discrete time system using Z transform |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21EC2007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DATA STRUCTURES AND ALGORITHMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify an example of a linear and non-linear data structure. |  | CO1 | R | 1 |
| 2. | In what way are linked list nodes stored in memory? |  | CO3 | U | 1 |
| 3. | Consider the following operation performed on a stack of size 5. Push(1); Pop(); Push(2); Push(3); Pop(); Push(4); Pop(); Pop(); Push(5); After the completion of all operation, how many elements are present in stack? |  | CO2 | A | 1 |
| 4. | Under what situations do stack overflow and stack underflow happen? |  | CO3 | U | 1 |
| 5. | What problem-solving strategy does the Merge Sort algorithm follow? |  | CO2 | R | 1 |
| 6. | Consider an array has 10 elements and the searching element is at array index 6. A starting element is present at index zero. How many comparisons are required to search an element using linear search? |  | CO2 | A | 1 |
| 7. | Does Dijkstra’s Algorithm work with negative edge weights? Justify your answer. |  | CO3 | U | 1 |
| 8. | In which type of graph is the Minimum Spanning Tree defined? |  | CO4 | R | 1 |
| 9. | Name the common notations used for expressing algorithms. |  | CO6 | R | 1 |
| 10. | Which algorithm design technique selects the locally optimal choice at each step? |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | The function of the given Linked List with the first node as the head is void funx1(struct node* head) { if (head == NULL) return; funx1 (head -> next) ; printf (" %d ", head -> data ) ; } What will be the result for the following input of Linked List: (12345) |  | CO5 | A | 3 |
| 12. | Mention any two applications where the stack and queue could be used. |  | CO3 | U | 3 |
| 13. | Sort the given sequence 55, 22, 44, 99, 88, 11, 33 using insertion sort. |  | CO2 | A | 3 |
| 14. | Derive the adjacency matrix and adjacency list representation of the following graph. |  | CO4 | U | 3 |
| 15. | Recall the characteristics of P, NP, NP-Hard, and NP-Complete problems and highlight the differences between them with examples. |  | CO5 | U | 3 |
| 16. | Define Algorithm. Specify its main characteristics. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Design an algorithm to implement the List ADT for the following operations and illustrate it diagrammatically. - Insert at Last ii) Delete the given element iii) Display the Element in the list | CO3 | U | 12 |
| 18. |  | Sketch the algorithm for converting infix expression into postfix expression for the following expression (a+b)*((d-c) / e* (f+g). Show the contents of the stack during the conversion. Also, evaluate the converted post fix expression assuming the following values for the operands a= 3, b=2, c=3, d=4, e=8, f=5, g=7 | CO3 | A | 12 |
| 19. |  | Given the array [170, 45, 75, 90, 802, 24, 2, 66], implement Bubble Sort to arrange the elements in ascending order and report the sorted array. Determine the number of swaps needed to sort the array. Provide the intermediate steps along with its pseudocode. | CO2 | A | 12 |
| 20. |  | The in-order and post-order traversal of a tree is given below: - In-order: DBMINEAFCJGK - Post-order: ABDEIMNCFGJK Construct the corresponding Binary Tree. Determine the post-order traversal of the tree. Provide the recursive code for the tree traversal | CO5 | A | 12 |
| 21. |  | Consider the following graph with starting position at node B. Make use of BFS and DFS traversal, list the nodes in the order of visit. Also, provide the recursive code for the same. | CO5 | A | 12 |
| 22. |  | Write down the pseudo code to perform the topological sort. Perform topological sort on the following graph and show the result of applying this sort: | CO4 | A | 12 |
| 23. |  | A system has to schedule jobs to maximize profit. Each job requires one unit of time and must be completed before its deadline. Job; Deadline; Profit. J1; 2; 100. J2; 1; 19. J3; 2; 27. J4; 1; 25. J5; 3; 15. Apply the Greedy algorithm to schedule the jobs by determining the optimal job sequence and maximum profit. Write the pseudocode for the Job Sequencing algorithm. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Construct the minimum spanning tree from the following graph using Prim’s algorithm. | CO4 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze the time and space efficiency of data structures and algorithms and apply this analysis to select the best tools for solving particular problems |
| CO2 | Implement a variety of algorithms for searching and sorting, including linear search, binary search, insertion sort, selection sort, merge sort, quicksort, and heap sort |
| CO3 | Describe, explain, and use abstract data types including stacks, queues, lists, sets, maps and graphs |
| CO4 | Implement those data types using both contiguous and linked representations |
| CO5 | Read and write recursive algorithms. Understand when recursion is, and is not, appropriate |
| CO6 | Implement an advanced algorithm using Elementary and Greedy Method with Single Source Shortlist Paths |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21EC2009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # FUNDAMENTALS OF JAVA PROGRAMMING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State the significance of the public keyword in Java. |  | CO1 | R | 1 |
| 2. | Describe the purpose of a constructor in Java. |  | CO2 | R | 1 |
| 3. | Identify the special type of method in a class that will be automatically called whenever new objects are created. |  | CO2 | U | 1 |
| 4. | Name any one built-in Java packages. |  | CO3 | R | 1 |
| 5. | Define the role of access specifiers in inheritance. |  | CO3 | R | 1 |
| 6. | Identify the difference between ArrayList and Vector. |  | CO5 | U | 1 |
| 7. | Identify the purpose of the try-catch block. |  | CO3 | U | 1 |
| 8. | List any two classes in the Collection Framework. |  | CO5 | R | 1 |
| 9. | Define the term “random access file” in Java. |  | CO5 | R | 1 |
| 10. | State the difference between text and binary file operations. |  | CO5 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | State the difference between JDK and JRE. |  | CO1 | R | 3 |
| 12. | Write a Java program to calculate the area of a circle for a given radius. Print the calculated area as the output. |  | CO1 | A | 3 |
| 13. | Summarize the use of collections in Java. |  | CO2 | U | 3 |
| 14. | Describe about User defined exception. |  | CO3 | R | 3 |
| 15. | Compare method overloading and method overriding. |  | CO3 | An | 3 |
| 16. | List the methods available in the Thread class. |  | CO4 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the fundamental concepts of Object-Oriented Programming (OOP) in Java, namely Encapsulation, Inheritance, Polymorphism, and Abstraction. Illustrate each concept with suitable examples, and describe how a Java program can demonstrate these principles. | CO1 | U | 12 |
| 18. |  | Examine the use of if, if-else, and switch statements in Java for assigning grades based on student marks. Analyze the differences in their behavior and demonstrate the logic through a Java program. | CO2 | An | 12 |
| 19. | a. | Differentiate between checked and unchecked exceptions in Java, highlighting their key characteristics and differences. | CO3 | U | 6 |
|  | b. | Write a Java program to demonstrate exception handling using try, catch, and finally blocks by performing division of two user-input numbers, handling division by zero using ArithmeticException. | CO3 | A | 6 |
| 20. |  | Explain the working of the Java Stack class, and demonstrate its operations such as push, pop, isEmpty, search, size, and clear using a Java program with expected output. | CO4 | A | 12 |
| 21. |  | Explain the different types of inheritance in object-oriented programming. Describe each type with an example and discuss its advantages and limitations. | CO2 | U | 12 |
| 22. | a. | Illustrate the concept of method overloading in Java by explaining how it allows multiple methods with the same name to coexist in a class with different parameter lists and how the compiler differentiates between them during method calls. | CO2 | U | 6 |
|  | b. | Define a Java interface named Calculator with methods for basic operations (add, subtract, multiply, divide). Write a class BasicCalculator that implements this interface with actual methods. | CO3 | A | 6 |
| 23. | a. | Illustrate the concept of Java Database Connectivity (JDBC) by explaining the steps involved in establishing a database connection, executing queries, and handling the results in a Java application. | CO6 | U | 6 |
|  | b. | Write a Java program to copy the contents of one file to another using FileInputStream and FileOutputStream. | CO5 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Apply Java Swing and database connectivity concepts to develop a Hotel Booking System that captures user inputs such as customer name, room type, and number of days, and stores the data in a database. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basics of Object-Oriented design. |
| CO2 | Identify classes, abstract classes, objects and members needed for specific application. |
| CO3 | Create JAVA application programs using sound OOP practices. |
| CO4 | Develop programs using multitasking applications. |
| CO5 | Analyze real time applications. |
| CO6 | Apply the skills for designing GUI based applications. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # FUNDAMENTALS OF JAVA PROGRAMMING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Write a Java method to convert the string "hello" into uppercase. |  | CO1 | A | 1 |
| 2. | State the purpose of synchronization in Java multithreading. |  | CO4 | R | 1 |
| 3. | Identify the purpose of method overloading in Java. |  | CO3 | U | 1 |
| 4. | Interpret the role of the “this” keyword in constructor calls. |  | CO2 | A | 1 |
| 5. | Define inter-thread communication in Java. |  | CO4 | R | 1 |
| 6. | Determine the result of applying charAt(3) on the string "Processor". |  | CO1 | A | 1 |
| 7. | Recall the role of the throws keyword in Java. |  | CO3 | R | 1 |
| 8. | Cite the use of the Iterator interface in Java collections. |  | CO5 | U | 1 |
| 9. | Identify the purpose of the JTextField component in Java Swing. |  | CO6 | U | 1 |
| 10. | Discuss the significance of JButton in GUI applications. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | State the significance of the break statement in Java. |  | CO1 | R | 3 |
| 12. | Discuss the role of the final keyword in Java. |  | CO3 | U | 3 |
| 13. | Implement a Java program to convert a float value into an int using explicit type casting. |  | CO2 | A | 3 |
| 14. | Discuss the impact of method overriding in achieving runtime polymorphism. |  | CO2 | U | 3 |
| 15. | Write a Java program to demonstrate the use of an abstract class. |  | CO2 | A | 3 |
| 16. | Develop a Java Swing program to handle mouse events. |  | CO5 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Describe the core Object-Oriented Programming (OOP) concepts in Java Encapsulation, Inheritance, Polymorphism, and Abstraction with examples, and demonstrate them using a Java program. | CO1 | U | 12 |
| 18. |  | Analyze how for, while, and do-while loops are used in Java to compute the sum of numbers up to a given input, and demonstrate their working through a Java program. | CO2 | An | 12 |
| 19. | a. | Explain the different states of a thread in Java and describe the transitions between these states using appropriate methods. | CO3 | U | 6 |
|  | b. | Write a Java program to demonstrate inter-thread communication using wait() and notify() methods, where one thread produces data and another consumes it. | CO3 | A | 6 |
| 20. |  | Explain the functions of the Java Hashtable class, and demonstrate operations such as put(), get(), containsKey(), containsValue(), remove(), and keySet() using a Java program with expected output. | CO4 | A | 12 |
| 21. |  | Explain the different types of inheritance in object-oriented programming, with examples, and discuss their advantages and limitations. | CO2 | U | 12 |
| 22. | a. | Explain the concept of packages in Java and how they support class organization and modularity. | CO2 | U | 6 |
|  | b. | Write a Java program to demonstrate the use of inner classes, where the inner class accesses and displays a private member of the outer class. | CO3 | A | 6 |
| 23. | a. | Illustrate the concept of Java Database Connectivity (JDBC) by explaining the steps involved in establishing a connection, executing queries, and processing results in a Java application. | CO6 | U | 6 |
|  | b. | Write a Java program to read a file line by line using BufferedReader and count the total number of words, displaying the output. | CO5 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Develop a Java Swing-based Library Management System that supports adding, searching, and deleting books, and integrates with a MySQL database. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | understand the basics of Object-Oriented design. |
| CO2 | identify classes, abstract classes, objects and members needed for specific application. |
| CO3 | create JAVA application programs using sound OOP practices. |
| CO4 | develop programs using multitasking applications. |
| CO5 | analyze real time applications. |
| CO6 | apply the skills for designing GUI based applications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21EC2010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # LINEAR INTEGRATED CIRCUITS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Name the number of terminals in IC741 OP-AMP. |  | CO1 | U | 1 |
| 2. | Estimate the value of CMRR of an amplifier with differential gain of 10000 and common mode gain of 2. |  | CO1 | An | 1 |
| 3. | Name the circuit with unity gain used as a buffer for impedance matching. |  | CO2 | R | 1 |
| 4. | Indicate the phase difference between input and output in a non-inverting amplifier. |  | CO2 | U | 1 |
| 5. | Predict the output voltage of IC7815 monolithic voltage regulator. |  | CO3 | A | 1 |
| 6. | Write the multivibrator that has one quasi-stable state and one stable state. |  | CO3 | A | 1 |
| 7. | Name the filter that passes low frequencies and attenuates high frequencies. |  | CO4 | R | 1 |
| 8. | Indicate the roll-off rate of a second-order high-pass filter. |  | CO4 | U | 1 |
| 9. | Write any one application of 555 timer. |  | CO5 | A | 1 |
| 10. | Predict the number of comparators required for a 4-bit Flash ADC. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Write any three ideal characteristics of an OP-AMP. |  | CO1 | A | 3 |
| 12. | Determine the closed-loop voltage gain of an inverting amplifier with Rf = 100KΩ and Ri = 10KΩ. |  | CO2 | A | 3 |
| 13. | Write the barkhausen criterion for sustained oscillations. |  | CO3 | A | 3 |
| 14. | Classify the types of active filters used in communication systems. |  | CO4 | An | 3 |
| 15. | List the basic building blocks of a Phase Locked Loop (PLL). |  | CO5 | R | 3 |
| 16. | Calculate the output voltage of an 8-bit DAC with an output range of 0 to 10 V for the input binary number 10111100. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the DC characteristics of an OP-AMP. | CO1 | A | 8 |
|  | b. | Sketch the circuit symbol of IC741 OP-AMP. | CO1 | A | 4 |
| 18. | a. | Examine the operation of a non-inverting amplifier with a neat circuit diagram and derive the expression for its closed-loop voltage gain. | CO2 | A | 10 |
|  | b. | Calculate the current i1 for the input voltage V1 = 2V in the given inverting amplifier circuit. | CO2 | An | 2 |
| 19. |  | Explain the working principle of an RC phase shift oscillator with a neat circuit diagram. Verify the Barkhausen criterion and derive the expression for its frequency of oscillation. | CO3 | A | 12 |
| 20. | a. | With a neat circuit diagram and frequency response, explain the working of a first-order low-pass filter. | CO4 | A | 10 |
|  | b. | Compare band-pass filter with band-reject filter. | CO4 | U | 2 |
| 21. | a. | Dramatize the working principle of 555 timer IC with a functional diagram. | CO5 | A | 10 |
|  | b. | Sketch the pin diagram of 555 timer IC. | CO5 | A | 2 |
| 22. |  | Explain the operation of an astable multivibrator using an OP-AMP with a neat circuit diagram and waveforms. Derive the expression for the total time period (T). | CO3 | A | 12 |
| 23. | a. | Calculate the output voltage in the given non-inverting adder circuit. Assume Rf = 10 KΩ, R1= R2= R3=R= Rf/2, V1= 3V, V2= 3V and V3= 2V. | CO2 | An | 4 |
|  | b. | Justify that an OP-AMP can be used as an integrator. | CO2 | E | 6 |
|  | c | A non-inverting amplifier with a gain of 100 is nulled at 250C. Predict the output voltage if the temperature rises to 500C for an offset voltage drift of 0.15mV/0C. | CO1 | A | 2 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the working principle of a Weighted resistor DAC with neat circuit diagram and transfer characteristics. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the fundamentals of OP-AMP and its characteristics. |
| CO2 | Use OP-AMP to design circuits such as Amplifiers, Differentiator, and Integrator. |
| CO3 | Infer the significance of OP-AMP in Multivibrators and Oscillators |
| CO4 | Design filters using OP-AMPs. |
| CO5 | Explore the usefulness of IC555 timer and Phase Locked Loop. |
| CO6 | Design ADC, DAC and understand the IC fabrication. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2011 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ANALOG ELECTRONICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State the condition for a MOSFET to operate in the saturation region. |  | CO1 | R | 1 |
| 2. | Name the process used to produce single crystal silicon ingots. |  | CO1 | R | 1 |
| 3. | Identify the region of operation in which a MOSFET acts as an amplifier. |  | CO2 | U | 1 |
| 4. | Write the drain current (ID) equation of a MOSFET in the linear region. |  | CO2 | A | 1 |
| 5. | Name any one type of MOSFET. |  | CO3 | R | 1 |
| 6. | Identify the basic chemical reaction involved in thermal oxidation. |  | CO3 | U | 1 |
| 7. | Write the input impedance parameter of a common-gate amplifier. |  | CO4 | A | 1 |
| 8. | Name the layer that isolates gate terminal from the semiconductor substrate. |  | CO4 | R | 1 |
| 9. | Define current mirror. |  | CO5 | R | 1 |
| 10. | Define drain conductance(gd) in a MOSFET. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | List the steps involved in the preparation of silicon wafers. |  | CO1 | R | 3 |
| 12. | Differentiate MOSFET from JFET. |  | CO2 | An | 3 |
| 13. | Compare CMOS with Bipolar technologies. |  | CO3 | U | 3 |
| 14. | Write the expression for the transconductance of an n-channel MOSFET in linear region. |  | CO4 | A | 3 |
| 15. | Explain the cascade MOS mirror circuit. |  | CO5 | A | 3 |
| 16. | Write any three characteristics of CMOS technology. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the various steps involved in the Integrated Circuit (IC) fabrication process with neat diagrams. | CO1 | A | 12 |
| 18. |  | Discuss the CMOS p-well fabrication process in detail with a neat diagram. | CO1 | U | 12 |
| 19. | a. | Determine the characteristic parameters and overall voltage gain of a Common Gate amplifier with necessary small-signal equivalent circuit. | CO3 | A | 6 |
|  | b. | Solve the bias current of M1 in given circuit. Assume K=100µA/V2 and Vth =0.4V | CO3 | A | 6 |
| 20. | a. | Explain the basic MOS current mirror circuit with necessary equations and neat diagrams. | CO5 | A | 8 |
|  | b. | Construct the circuit symbol and truth table of a two input NAND gate using CMOS technology. | CO1 | A | 4 |
| 21. | a. | Formulate the equation for channel length modulation and explain its effect on MOSFET operation. | CO2 | C | 8 |
|  | b. | Apply the concept of channel length modulation to find the drain current of a MOSFET when VDS increases from 0.5V to 1V, given that the drain current of 1mA at VDS =0.4V in saturation and the channel length modulation parameter λ=0.2V-1. | CO2 | A | 4 |
| 22. |  | Estimate the voltage gain, and impedance parameters of a common-source amplifier with appropriate small-signal equivalent circuit. | CO4 | A | 12 |
| 23. |  | Discuss the operations of the following MOS current mirrors with neat diagrams and necessary equations. i)Basic current mirror ii) Wilson current mirror | CO5 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the working principles of the given CMOS amplifier topologies with neat circuit diagrams: i) Common-source amplifier with resistive load ii) Common-source amplifier with diode-connected load | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basics of MOSFETs. |
| CO2 | Analyze the Transistor amplifier. |
| CO3 | Understand the configurations of MOSFET. |
| CO4 | Adopt different techniques in Cascode amplifiers. |
| CO5 | Demonstrate the current mirrors and Differential amplifiers. |
| CO6 | Apply the entire concepts to design CMOS amplifiers. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2014 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # MICROPROCESSORS AND MICROCONTROLLERS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the size of the data bus of 8085 Microprocessor. |  | CO1 | U | 1 |
| 2. | State the importance of Instruction Queue in 8086 Microprocessor. |  | CO1 | R | 1 |
| 3. | Recall the size of the internal RAM in 8051 Microcontroller. |  | CO2 | R | 1 |
| 4. | Name any one SFR register involved in the operation of 8051 Timers. |  | CO2 | R | 1 |
| 5. | List any one addressing mode of 8051 Microcontroller. |  | CO3 | R | 1 |
| 6. | Interpret the 8051-instruction used to perform decrement operation. |  | CO3 | U | 1 |
| 7. | Predict the function of 8251 IC. |  | CO4 | A | 1 |
| 8. | State the importance of ADC in embedded systems. |  | CO4 | R | 1 |
| 9. | Recall any one wireless communication protocol used in present day applications. |  | CO5 | R | 1 |
| 10. | Identify one embedded system application using LCD and Keypad. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare 8085 microprocessor with 8051 Microcontroller. |  | CO1 | An | 3 |
| 12. | Identify the important functional blocks of 8051 Microcontroller. |  | CO2 | U | 3 |
| 13. | Write an Assembly language program to add 2 numbers and store in memory using 8051 Microcontroller. |  | CO3 | A | 3 |
| 14. | Sketch the control word register of 8253 IC. |  | CO4 | A | 3 |
| 15. | Compare Synchronous with Asynchronous communication. |  | CO5 | An | 3 |
| 16. | Discuss the applications of stepper motor in real time. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain with a neat block diagram the architecture of 8086 microprocessor. | CO1 | A | 12 |
| 18. |  | Discuss the addressing modes of 8051 microcontroller with examples. | CO2 | U | 12 |
| 19. |  | Explain the timer/counter architecture of 8051 microcontroller and discuss the various timer modes. | CO2 | A | 12 |
| 20. |  | Explain the architecture and features of the 8255 Programmable Peripheral Interface with a neat block diagram. | CO4 | A | 12 |
| 21. | a. | Write an assembly language program for serial data transmission of the character ‘A’ using serial mode 1. | CO3 | An | 6 |
|  | b. | Discuss the interrupt enable (IE) and interrupt priority (IP) registers of 8051. | CO5 | U | 6 |
| 22. |  | Explain the architecture and internal block diagram of 8253 programmable interval timer. | CO5 | A | 12 |
| 23. |  | Explain with a neat block diagram the architecture of 8051 Microcontroller. | CO2 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Illustrate the interfacing of a stepper motor with the 8051 microcontroller and explain the step sequence required for controlling motor rotation. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the microprocessor architecture and its interfacing techniques. |
| CO2 | Describe the architecture of 8051 controller. |
| CO3 | Execute basic and advanced Assembly language and C programs. |
| CO4 | Interface I/O devices with Microcontroller. |
| CO5 | Describe various communication Interface. |
| CO6 | Recognize the functionality of Microcontroller and its applications to solve real world engineering problems. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21EC2015 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # WEB TECHNOLOGY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define HTTP request message. |  | CO1 | R | 1 |
| 2. | State the purpose of relative URLs. |  | CO1 | U | 1 |
| 3. | Identify the CSS property used to change text color. |  | CO2 | R | 1 |
| 4. | Define JavaScript variable. |  | CO2 | R | 1 |
| 5. | State the meaning of DOM. |  | CO3 | U | 1 |
| 6. | Define session in web applications. |  | CO3 | R | 1 |
| 7. | Expand JSP. |  | CO4 | R | 1 |
| 8. | State the purpose of XML namespaces. |  | CO5 | U | 1 |
| 9. | Evaluate the role of AJAX in dynamic web applications. |  | CO6 | E | 1 |
| 10. | Create a simple XML element with proper tags. |  | CO5 | C | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Construct an HTML program to create an ordered list of three subjects. |  | CO1 | A | 3 |
| 12. | Construct a HTML table to display Name and Age. |  | CO1 | A | 3 |
| 13. | Develop a JavaScript function to find the sum of two numbers. |  | CO3 | A | 3 |
| 14. | Illustrate DOM event handling with a suitable example. |  | CO3 | U | 3 |
| 15. | Differentiate Servlets from JSP. |  | CO4 | An | 3 |
| 16. | Describe the AJAX callback function with a suitable example. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | A college plans to develop an online student registration portal. Design a HTML5 form with appropriate input fields, validation attributes, and structure for this system. | CO1 | C | 6 |
|  | b. | Describe each element used in the form supports user interaction and data collection. | CO1 | C | 6 |
| 18. | a. | Describe CSS selectors can be used to target elements effectively in improving the visual layout of a company’s website. | CO2 | U | 6 |
|  | b. | Apply CSS properties to construct a responsive layout for the homepage using suitable layout techniques. | CO2 | A | 6 |
| 19. | a. | An e-commerce website needs to maintain user login sessions across multiple pages. Analyze different session tracking techniques such as cookies, URL rewriting, hidden fields, and Http-Session for this requirement. | CO3 | An | 6 |
|  | b. | Compare these techniques and determine the most suitable approach for secure session management. | CO3 | An | 6 |
| 20. | a. | Evaluate the servlet lifecycle and its phases. | CO4 | E | 6 |
|  | b. | Justify the role of servlets in generating dynamic web content. | CO4 | E | 6 |
| 21. |  | Apply XPath expressions and XML DOM parsing techniques to retrieve and manipulate data from XML documents. | CO5 | A | 12 |
| 22. |  | A web application uses AJAX to update content dynamically without refreshing the page. Analyze the role of XML Http-Request object in this process. | CO6 | An | 12 |
| 23. |  | Create a JavaScript-based application to validate user input in a login form and demonstrate its functionality. | CO2 | C | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Evaluate RESTful web services and justify their advantages compared to SOAP-based web services. | CO6 | E | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design simple web pages using markup languages like HTML and XHTML. |
| CO2 | Create dynamic web pages using DHTML and java script that is easy to navigate and use. |
| CO3 | Program server side web pages that have to process request from client side web pages. |
| CO4 | Represent web data using XML and develop web pages using JSP. |
| CO5 | Understand various web services. |
| CO6 | Comprehend how these web services interact. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2015 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # WEB TECHNOLOGY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define web client in Internet communication. |  | CO1 | R | 1 |
| 2. | Describe the working of HTTP response message. |  | CO1 | U | 1 |
| 3. | Identify the CSS property used to control font size. |  | CO2 | R | 1 |
| 4. | Define JavaScript data types. |  | CO2 | R | 1 |
| 5. | Describe the structure of the DOM tree. |  | CO3 | U | 1 |
| 6. | Define cookies in web applications. |  | CO3 | R | 1 |
| 7. | Expand JAX-RPC. |  | CO4 | R | 1 |
| 8. | Describe the role of XML Schema in XML documents. |  | CO5 | U | 1 |
| 9. | Evaluate the need for web services in distributed systems. |  | CO6 | E | 1 |
| 10. | Create a simple HTML input element with attributes. |  | CO1 | C | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Construct an HTML program to create a nested list structure. |  | CO1 | A | 3 |
| 12. | Describe CSS positioning techniques used in webpage layout. |  | CO2 | U | 3 |
| 13. | Develop a JavaScript program to reverse a given string. |  | CO2 | A | 3 |
| 14. | Illustrate servlet request and response handling process. |  | CO3 | U | 3 |
| 15. | Analyze the features of JSP compared with traditional HTML pages. |  | CO4 | An | 3 |
| 16. | Describe the role of XML Schema in validating XML documents. |  | CO5 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Develop a structured webpage using HTML elements such as headings, lists, tables, and hyperlinks for an information portal. | CO1 | A | 12 |
| 18. | a. | A web application requires proper alignment and spacing of elements. Apply CSS box model and layout properties to construct an organized webpage. | CO2 | A | 6 |
|  | b. | Analyze how intrinsic event handling and DOM manipulation are used together to respond to user actions in a webpage. | CO3 | An | 6 |
| 19. |  | Apply XSLT transformation to convert XML data into structured HTML for browser display in an organization managing digital content. | CO5 | A | 12 |
| 20. |  | Analyze how cookies, sessions, and URL rewriting differ in maintaining user state across multiple requests. | CO3 | An | 12 |
| 21. | a. | Analyze how AJAX architecture enables asynchronous communication in a web application that updates user data without refreshing the page. | CO6 | An | 6 |
|  | b. | Evaluate the role of JSP implicit objects in handling client requests and managing responses in web applications. | CO4 | E | 6 |
| 22. | a. | Apply SAX parsing technique to process XML data efficiently in event-driven applications. | CO5 | A | 6 |
|  | b. | Evaluate JSP lifecycle and justify its role in dynamic content generation in web applications. | CO4 | E | 6 |
| 23. | a. | Analyze how SOAP protocol facilitates structured communication in an enterprise system integrating services across different platforms. | CO6 | An | 6 |
|  | b. | Evaluate the effectiveness of SOAP-based web services in ensuring secure and reliable communication in enterprise applications. | CO6 | E | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Evaluate the role of WSDL and XML Schema in defining and structuring interoperable web services in an enterprise environment. | CO6 | E | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design simple web pages using markup languages like HTML and XHTML. |
| CO2 | Create dynamic web pages using DHTML and java script that is easy to navigate and use. |
| CO3 | Program server side web pages that have to process request from client side web pages. |
| CO4 | Represent web data using XML and develop web pages using JSP. |
| CO5 | Understand various web services. |
| CO6 | Comprehend how these web services interact. |

# END SEMESTER EXAMINATION – APRIL / MAY 2026

| Course Code | 21EC2016 | Duration | 3hrs |
| --- | --- | --- | --- |
| Course Title | INTERNET OF THINGS | Max. Marks | 100 |

| Q. No. | Questions |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |  |
| 1. | A smart home uses sensors to detect motion. Which IoT component is responsible for sensing this movement? |  |  | CO2 | R | 3 |
| 2. | In a smart parking system, vehicles communicate with the server automatically. What type of communication is this? |  |  | CO1 | R | 1 |
| 3. | A temperature sensor sends data directly to cloud through a gateway. Which network topology is followed? |  |  | CO1 | R | 1 |
| 4. | A soil moisture sensor sends low power data periodically. Which IoT device category does it belong to. |  |  | CO2 | R | 3 |
| 5. | A supermarket uses RFID tags on products. Expand and explain RFID. |  |  | CO2 | R | 2 |
| 6. | A student programs a smart light using Arduino. Which IDE is used? |  |  | CO3 | R | 2 |
| 7. | A smart meter publishes data to a broker. Name the messaging protocol used. |  |  | CO3 | R | 4 |
| 8. | A city uses cloud to monitor pollution. Name one suitable IoT cloud platform. |  |  | CO4 | R | 5 |
| 9. | Traffic data is processed near the signal instead of cloud. What is this called? |  |  | CO5 | R | 5 |
| 10. | Vehicles exchange accident information automatically. Expand IoV. |  |  | CO6 | R | 6 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |  |
| 11. | A smart city collects data from street lights and parking sensors. Explain the IoT layers involved. |  |  | CO1 | U | 1 |
| 12. | In a health monitoring system, sensors collect patient data. Explain the role of sensors in this scenario. |  |  | CO2 | U | 3 |
| 13. | A weather station sends data to cloud every minute. Explain the communication pattern used. |  |  | CO3 | U | 2 |
| 14. | A Raspberry Pi is used in a smart classroom project. Describe its features suitable for this application. |  |  | CO3 | U | 3 |
| 5. | A startup hosts its IoT data on cloud. Explain the service models of cloud computing for this use case. |  | CO4 |  | U | 4 |
| 16. | A smart home was hacked. Write short notes on security challenges in this system. |  | CO5 |  | U | 5 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |  |
| 17. |  | A smart airport monitors baggage flow and runway temperature. - Design an IoT architecture for this system. (6) - Explain suitable communication protocols. (6) | CO1 CO3 |  | AN AN | 6 6 |
| 18. |  | A manufacturing plant wants to monitor machines in real time. - Analyze IoT performance challenges in this system. (6) - Explain IoT protocol architecture for industrial use. (6) | CO3 CO3 |  | AN AN | 6 6 |
| 19. |  | A greenhouse uses Arduino to control temperature and humidity. - Explain Arduino architecture and boards used. (6) - Write an Arduino program to control a temperature sensor. (6) | CO3 |  | A | 12 |
| 20. |  | A smart city uploads traffic data to cloud. - Describe Azure IoT platform components. (6) - Explain IoT app development using Google Cloud. (6) | CO4 |  | A | 12 |
| 21. |  | A power grid system detects faults automatically. - Explain data analytics tools for fault detection. (6) - Discuss edge and network analytics. (6) | CO5 |  | AN | 12 |
| 22. |  | A smart home faced cyber-attack. - Explain IoT security challenges in this scenario. (6) - Describe risk analysis structures. (6) | CO5 |  | E | 12 |
| 23. |  | A factory wants to become a future smart factory. - Explain Future Factory concepts. (6) - Discuss Internet of Vehicles applications in logistics. (6) | CO6 |  | AN | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |  |
| 24. |  | A smart farming system integrates sensors and cloud. - Explain IoT architecture and communication patterns. (6) - Describe cloud IoT platforms and their role. (6) | CO4 |  | AN | 12 |

CO – COURSE OUTCOME    BL – BLOOM’S LEVEL M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basics of IoT, networks and communication. |
| CO2 | Realize the importance of sensors and understand the sensor mechanism. |
| CO3 | Possess the required knowledge and expertise in IoT architecture, layering concepts and analysis of various protocols in IoT. |
| CO4 | Understand the various networks and development platforms in IoT |
| CO5 | Develop knowledge to overcome the challenges in IoT application platforms. |
| CO6 | Familiarize the various IoT applications. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | 21EC2019 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | SYSTEM SOFTWARE AND COMPILER DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define System Software. |  | CO1 | U | 1 |
| 2. | What is a Language Processor? |  | CO1 | R | 1 |
| 3. | Define Symbol Table. |  | CO2 | R | 1 |
| 4. | State ‘Macro Expansion’ |  | CO2 | R | 1 |
| 5. | What is an Interpreter? |  | CO3 | U | 1 |
| 6. | Name any two phases of a Compiler. |  | CO3 | R | 1 |
| 7. | Infer ‘Relocation’. |  | CO4 | U | 1 |
| 8. | Define Linking. |  | CO4 | R | 1 |
| 9. | What is a Token? |  | CO5 | U | 1 |
| 10. | Define Context Free Grammar. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate System Software from Application Software. |  | CO1 | An | 3 |
| 12. | Explain Machine Independent Assembler Features. |  | CO2 | U | 3 |
| 13. | Compare Compiler with Interpreter. |  | CO3 | An | 3 |
| 14. | Explain the Functions of a Relocating Loader. |  | CO4 | U | 3 |
| 15. | Illustrate Token Recognition using Finite Automata. |  | CO5 | An | 3 |
| 16. | Explain Top-Down Parsing with a Simple Example. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain various language processors and development tools used in software development. | CO1 | U | 6 |
|  | b. | Discuss editors, debug monitors, programming environments and user interfaces. | CO1 | U | 6 |
| 18. | a. | Explain the design and operation of a two-pass assembler with neat diagrams. | CO2 | An | 6 |
|  | b. | Discuss machine dependent and machine independent assembler features with suitable examples. | CO2 | An | 6 |
| 19. | a. | Explain the phases of a compiler with a neat block diagram. | CO3 | U | 6 |
|  | b. | Differentiate pure from impure interpreters and discuss the aspects of compilation. | CO3 | U | 6 |
| 20. | a. | Explain the basic functions of loaders and machine dependent loader features. | CO4 | An | 6 |
|  | b. | Discuss loader design options and implementation examples. | CO4 | An | 6 |
| 21. | a. | Explain token recognition and finite automata used in lexical analysis. | CO5 | An | 6 |
|  | b. | Describe lexical analyzer implementation and error recovery techniques. | CO5 | An | 6 |
| 22. | a. | Explain Context Free Grammars and discuss the role of parsers in syntax analysis. | CO6 | U | 6 |
|  | b. | Compare top-down with bottom-up parsing techniques with suitable examples. | CO6 | U | 6 |
| 23. | a. | Explain operator precedence parsing with a suitable example. | CO6 | An | 6 |
|  | b. | Discuss shift-reduce parsing and parsing actions involved in bottom-up parsing. | CO6 | An | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Design a lexical analyzer for a simple programming language and explain token generation. | CO6 | A | 6 |
|  | b. | For the grammar E→E+T¦T, T→T*F¦F, F→(E)¦id: - Eliminate left recursion - Demonstrate top-down parsing for id+id*id. | CO6 | A | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Compare various system software related to the given system |
| CO2 | Understand the concepts required to develop the system software |
| CO3 | Make proper use of system software tools |
| CO4 | Understand an algorithm by tracing its computational states |
| CO5 | Identify bugs in algorithms |
| CO6 | Analyze the bugs and correct them |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2020 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DATA COMMUNICATION NETWORKS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Predict the number of links needed in mesh topology to access 500 nodes. |  | CO1 | U | 1 |
| 2. | Name the layer which supports channel accessing. |  | CO1 | R | 1 |
| 3. | Determine the redundant bits for the following data using VRC method 10101001. |  | CO2 | A | 1 |
| 4. | Write the data rate and segment length of 10 BASE 2 thin Ethernet. |  | CO2 | A | 1 |
| 5. | Choose the maximum number of slaves in Piconet of Bluetooth technology. |  | CO3 | A | 1 |
| 6. | Identify the channel accessing method used in IEEE 802 .11 wireless LAN. |  | CO3 | R | 1 |
| 7. | Name the network layer protocol used in unicast routing. |  | CO4 | U | 1 |
| 8. | Write the header size of IPv4 protocol. |  | CO4 | U | 1 |
| 9. | Name the connectionless protocol used in transport layer. |  | CO5 | R | 1 |
| 10. | Give an example of protocol used in Electronic Mail. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | List the components of data communication. |  | CO1 | R | 3 |
| 12. | Determine the check sum for the following data bits 00100 01000 00010 1000 |  | CO2 | A | 3 |
| 13. | Predict the class, NET ID and network address of the given IP address 126.23.45.63 |  | CO3 | An | 3 |
| 14. | Write the format of any one error reporting messages in ICMP protocol. |  | CO4 | U | 3 |
| 15. | Explain the types of traffic profile used in computer networks. |  | CO5 | A | 3 |
| 16. | State the principle of Encryption and Decryption. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Sketch the structure of OSI model and explain the responsibility of each layer. | CO1 | U | 12 |
| 18. | a. | Explain stop and wait protocol. | CO2 | A | 6 |
|  | b. | Examine any one error detection method with an example. | CO2 | An | 6 |
| 19. | a. | Illustrate the types of packets switching in detail. | CO3 | A | 8 |
|  | b. | Discuss the features of Zigbee technology. | CO3 | U | 4 |
| 20. |  | Illustrate Distance Vector algorithm with an example and estimate the initial routing table for all the routers, Router updating process and final routing table of any one router. | CO4 | E | 12 |
| 21. |  | Categories the types of congestion control mechanisms in detail. | CO5 | An | 12 |
| 22. | a. | Discuss the principle of Go back N ARQ error control method. | CO2 | U | 8 |
|  | b. | Explain the features of User Datagram protocol (UDP). | CO5 | U | 4 |
| 23. |  | Categories the types of topologies in detail. | CO1 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Discuss the types of cryptography. | CO6 | U | 6 |
|  | b. | Describe the principle of Domain name System (DNS) | CO6 | R | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Acquire knowledge about OSI model. |
| CO2 | Identify the components required to build different types of networks. |
| CO3 | Choose the required functionality at each layer for given application. |
| CO4 | Trace the flow of information from one node to another node in the network. |
| CO5 | Apply the acquired knowledge to develop the routing algorithms. |
| CO6 | Demonstrate the applications of computer Networks. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2025 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # NEURAL NETWORKS AND DEEP LEARNING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Distinguish soma from axon in human brain. |  | CO1 | U | 1 |
| 2. | Enumerate the activation functions used in artificial neural networks. |  | CO1 | R | 1 |
| 3. | Distinguish bi-level from multi-level classification models. |  | CO2 | U | 1 |
| 4. | Enumerate the different kernels used in Radial Basis Function networks. |  | CO2 | R | 1 |
| 5. | Distinguish Hopfield network from Boltzmann machines. |  | CO3 | U | 1 |
| 6. | List the weight setting rules in Hopfield neural network. |  | CO3 | R | 1 |
| 7. | Distinguish LSTM from RNN networks. |  | CO4 | U | 1 |
| 8. | List the different gates used in LSTM network. |  | CO4 | R | 1 |
| 9. | List the different layers of convolutional neural network. |  | CO5 | R | 1 |
| 10. | Distinguish agents from rewards in reinforcement learning. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Illustrate the function of autoencoder with neat architecture. |  | CO1 | U | 3 |
| 12. | Summarize the technical concepts of RBF network. |  | CO2 | U | 3 |
| 13. | Illustrate the operation of RBM for classification application with neat architecture. |  | CO3 | U | 3 |
| 14. | “LSTM network is suitable for time-series data-based applications”. Justify this statement. |  | CO4 | E | 3 |
| 15. | Explain the max pooling layer of convolutional neural network. |  | CO5 | U | 3 |
| 16. | Distinguish supervised learning from reinforcement learning for classification applications. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Illustrate the process of electrical communication and chemical communication inside human neuron with neat diagrams. | CO1 | U | 12 |
| 18. | a. | Summarize the functions of artificial neuron with neat diagrams. Include mathematical equations wherever necessary. | CO1 | U | 6 |
|  | b. | Distinguish computer from human brain in terms of various performance measures. | CO1 | An | 6 |
| 19. | a. | Explain the training algorithm of perceptron with necessary mathematical equations. | CO1 | U | 6 |
|  | b. | Summarize the linear separable problem of perceptron and suggest suitable solution to overcome the problem. | CO1 | U | 6 |
| 20. |  | Illustrate the training process of RBF neural network with mathematical equations and neat architecture. | CO2 | U | 12 |
| 21. |  | Explain the function of LSTM network with neat architecture and mathematical equations. | CO4 | U | 12 |
| 22. |  | Illustrate the functions of convolutional neural network with different layers of the architecture. Include neat diagrams and mathematical equations wherever necessary. | CO5 | U | 12 |
| 23. | a. | “Hopfield neural networks are more stable than other neural networks”. Justify this statement. | CO3 | E | 6 |
|  | b. | Develop a neural network-based model for classification application with 4 output classes and 8 input features. | CO6 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Illustrate the architecture and training algorithm of back propagation neural network with neat diagrams. Support your answer with necessary mathematical equations. | CO1 | U | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basics of neural networks. |
| CO2 | Comprehend RBF network and its training |
| CO3 | Understand the concepts of Restricted Boltzmann Machines. |
| CO4 | Apply recurrent neural networks. |
| CO5 | Understand the details of Convolution Neural network |
| CO6 | Analyse the significance of Reinforcement learning. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21EC2027 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # LINUX PROGRAMMING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1 | Identify the system component responsible for interpreting user commands in Linux. |  | CO1 | U | 1 |
| 2 | State the purpose of command substitution in shell operations. |  | CO4 | U | 1 |
| 3 | Identify the command used to display the current working directory. |  | CO2 | U | 1 |
| 4 | State the role of file permissions in Linux security. |  | CO2 | U | 1 |
| 5 | Identify the situation where the vi editor is used for file modification. |  | CO3 | U | 1 |
| 6 | State the purpose of the grep command in text processing. |  | CO2 | U | 1 |
| 7 | Identify the role of the read command in shell scripts. |  | CO3 | U | 1 |
| 8 | State the purpose of command-line arguments in shell scripts. |  | CO4 | U | 1 |
| 9 | Identify the system call used to create a new process. |  | CO4 | U | 1 |
| 10 | State the purpose of signals in process management. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11 | A user navigates directories and manages files in Linux. Execute the basic commands to create, move, and list files. |  | CO2 | A | 3 |
| 12 | A system administrator edits configuration files. Explain the contribution of navigation and editing commands in the vi editor. |  | CO3 | An | 3 |
| 13 | A dataset requires filtering and sorting. Execute the filters such as sort and uniq to process data. |  | CO2 | A | 3 |
| 14 | A script processes user input and performs conditional execution. Discuss the role of if, test, and logical operators in controlling flow. |  | CO4 | An | 3 |
| 15 | A program reads input from users and processes positional parameters. Implement the use of command-line arguments and $0, $1 variables. |  | CO4 | A | 3 |
| 16 | A Linux system handles multiple processes. Examine the role of signals and process control commands in ensuring safe execution and controlled termination of processes. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a | A user interacts with the Linux terminal to execute commands. Implement the layered architecture of Linux by illustrating the role of shell, kernel, file system, and hardware during command execution. | CO1 | A | 8 |
|  | b | A new user experiences delays and unexpected outputs after executing terminal commands. Examine the differences in Linux components contribute to user interaction and system operations in such a situation. | CO1 | An | 4 |
| 18. | a | A system administrator manages files and directories in a Linux environment. Develop a sequence of commands to create directories, manage files, and display file attributes. | CO2 | A | 6 |
|  | b | A multi-user system faces unauthorized access issues to shared files. Examine the file permissions and ownership settings contribute to protecting system resources. | CO2 | An | 6 |
| 19. | a | A developer edits configuration files using the vi editor. Execute the commands for inserting, deleting, saving, and navigating within a file. | CO3 | A | 4 |
|  | b | A script requires repeated modifications and corrections. Employ the undo, repeat, and search commands for efficient editing. | CO3 | A | 4 |
|  | c | A developer works on large configuration files with repeated patterns. Examine the keywords - search and substitution features assist in efficient content modification. | CO3 | An | 4 |
| 20. | a | A data analyst processes large text files. Employ the filters such as cut, paste, sort, and uniq for data manipulation. | CO2 | A | 8 |
|  | b | A dataset contains inconsistent patterns and formats. Examine the regular expressions support accurate and flexible text processing in such cases. | CO2 | An | 4 |
| 21. | a | A system administrator develops a shell script for automation. Develop a script using loops and conditional statements to process user input. | CO4 | A | 6 |
|  | b | A developer manages complex scripts with multiple tasks and debugging needs. Examine the features such as arrays, functions, and debugging tools support structured script development. | CO4 | An | 6 |
| 22. | a | A program performs file operations using system calls. Employ the open, read, write, and close system calls. | CO5 | A | 6 |
|  | b | A system processes large volumes of data with performance constraints. Implement the buffered and unbuffered I/O techniques influence efficiency in file handling. | CO5 | An | 6 |
| 23. | a | A system creates and manages processes. Implement the process creation and execution using fork and exec system calls. | CO6 | A | 8 |
|  | b | A system runs multiple processes that need to exchange data and maintain coordination. Examine the process synchronization and inter-process communication support stable execution. | CO6 | An | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a | A system processes multiple tasks and manages file operations simultaneously. Develop a solution integrating shell scripting, file handling, and process control. | CO5 | A | 6 |
|  | b | A server environment encounters issues due to unresponsive processes affecting system stability. Examine the signal handling and process management mechanisms help maintain stable operation. | CO6 | An | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Documentation will demonstrate good organization and readability. |
| CO2 | File processing projects will require data organization, problem solving and research. |
| CO3 | Scripts and programs will demonstrate simple effective user interfaces. |
| CO4 | Scripts and programs will demonstrate effective use of structured programming. |
| CO5 | Scripts and programs will be accompanied by printed output demonstrating completion of a test plan. |
| CO6 | Testing will demonstrate both black and glass box testing strategies. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2027 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # LINUX PROGRAMMING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1 | Identify the feature of Linux that supports multi-user operations. |  | CO1 | U | 1 |
| 2 | State the purpose of giving multiple commands in a single line. |  | CO4 | U | 1 |
| 3 | Identify the command used to list files in a directory. |  | CO2 | U | 1 |
| 4 | State the role of file ownership in Linux systems. |  | CO2 | U | 1 |
| 5 | Identify the situation where ex mode is used in the vi editor. |  | CO3 | U | 1 |
| 6 | State the purpose of the sed command in text processing. |  | CO2 | U | 1 |
| 7 | Identify the role of the case statement in shell scripting. |  | CO4 | U | 1 |
| 8 | State the purpose of exit status in shell commands. |  | CO4 | U | 1 |
| 9 | Identify the system call used to terminate a process. |  | CO6 | U | 1 |
| 10 | State the purpose of pipes ( ¦ ) in Linux commands. |  | CO2 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11 | A user needs to display system information such as date and logged-in users. Implement the suitable commands to retrieve the information. |  | CO2 | A | 3 |
| 12 | A developer modifies files using advanced editing features. Examine the contribution of substitution and navigation commands in efficient text editing. |  | CO3 | An | 3 |
| 13 | A dataset contains mixed and unordered records. Employ the filters such as head, tail, and wc to analyze file content. |  | CO2 | A | 3 |
| 14 | A script evaluates multiple conditions based on user input. Examine the case and conditional expressions support decision-making in scripts. |  | CO4 | An | 3 |
| 15 | A script processes a list of values provided during execution. Implement the loops and positional parameters to handle the input. |  | CO4 | A | 3 |
| 16 | A system administrator needs to observe resource usage of running processes. Implement the suitable commands to monitor CPU and memory utilization |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | A training institute introduces Linux to beginners. Employ the basic commands for system interaction and command execution. | CO1 | A | 8 |
|  | b. | A beginner struggles to interpret command results and system feedback during practice sessions. Examine the command structure and system utilities contribute to meaningful output and user understanding. | CO1 | An | 4 |
| 18. | a. | A user organizes project files and directories in Linux. Develop a sequence of commands to create, remove, and navigate directories. | CO2 | A | 6 |
|  | b. | A system stores sensitive logs that require controlled access and auditing. Examine the file attributes and access logs support tracking and accountability. | CO2 | An | 6 |
| 19. | a. | A developer creates and edits scripts using the vi editor. Employ the commands for entering text, saving changes, and exiting the editor. | CO3 | A | 4 |
|  | b. | A script contains repetitive content that needs quick modification. Employ the repeat and navigation commands to improve editing efficiency. | CO3 | A | 4 |
|  | c. | A developer works on a large configuration file where frequent editing errors occur during updates. Examine the role of undo and repeat commands in ensuring efficient correction and maintaining editing consistency across the file. | CO3 | An | 4 |
| 20. | a. | A system administrator analyzes large server log files to identify error messages and modify their format for reporting. Develop a sequence of commands using grep and sed to extract specific patterns and transform the log data into the required format. | CO2 | A | 8 |
|  | b. | A dataset collected from multiple sources contains duplicate and inconsistent entries. Examine how filtering tools support refinement and organization of such data. | CO2 | An | 4 |
| 21. | a. | A system administrator writes automation scripts for routine tasks. Develop a script using loops, input handling, and conditional statements. | CO4 | A | 6 |
|  | b. | A deployment script used in a production environment performs multiple operations and occasionally fails without clear output. Examine the debugging tools and exit status mechanisms assist in identifying errors and ensuring reliable script execution. | CO4 | An | 6 |
| 22. | a. | A program performs file manipulation tasks using system-level operations. Implement the file-related system calls to manage file data. | CO5 | A | 6 |
|  | b. | A system processes large data files repeatedly, leading to performance variations. Examine the different I/O approaches influence execution efficiency and system performance. | CO5 | An | 6 |
| 23. | a. | A Linux system executes multiple programs concurrently. Implement the process creation and execution using appropriate system calls. | CO6 | A | 8 |
|  | b. | A production server runs multiple background services such as logging, monitoring, and data processing. At times, these services interfere with each other, affecting overall performance. Examine the role of process groups and job control mechanisms in managing independent execution and maintaining system stability. | CO6 | An | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | A system automates tasks involving file processing and user interaction. Develop a solution integrating shell scripting and file handling techniques. | CO5 | A | 6 |
|  | b. | A production server experiences performance degradation due to increasing workloads and uneven resource usage. Examine the role of scheduling techniques and resource monitoring mechanisms in maintaining stable and efficient system performance. | CO6 | An | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Documentation will demonstrate good organization and readability. |
| CO2 | File processing projects will require data organization, problem solving and research. |
| CO3 | Scripts and programs will demonstrate simple effective user interfaces. |
| CO4 | Scripts and programs will demonstrate effective use of structured programming. |
| CO5 | Scripts and programs will be accompanied by printed output demonstrating completion of a test plan. |
| CO6 | Testing will demonstrate both black and glass box testing strategies. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2031 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SEMANTIC MODELING AND ITS APPLICATIONS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the social web that is used for fetching and reading information and lacks interactivity. |  | CO1 | U | 1 |
| 2. | Write the mathematical method employed in social network analysis. |  | CO1 | A | 1 |
| 3. | Identify the ontology language that is domain independent and model real world objects and information resources. |  | CO2 | A | 1 |
| 4. | Indicate the primitive of Resource Description Framework (RDF) that are strings with optional language and datatype identifiers. |  | CO2 | U | 1 |
| 5. | Identify the ontology that defines concepts and their relationships without complex logic or constraints. |  | CO3 | R | 1 |
| 6. | Write the standard used to represent thesauri in RDF. |  | CO3 | A | 1 |
| 7. | Name the data mining technique that finds relationship between items. |  | CO4 | R | 1 |
| 8. | Identify the machine learning algorithm that learns from past data to make predictions. |  | CO4 | U | 1 |
| 9. | Write the key element sources used to develop a semantic model. |  | CO5 | A | 1 |
| 10. | Name the type of feedback that is received before it’s too late to act on it. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Sketch the Graph G for the given nodes and edges of a social network. G= {(A,B), (A, C), (A,E), (B,A), (B,E), (C,B), (D,A), (E,C)} |  | CO1 | A | 3 |
| 12. | Write the ‘triple’ data model of Resource Description Framework (RDF). |  | CO2 | A | 3 |
| 13. | List any three characteristics of ontologies used in semantic web. |  | CO3 | R | 3 |
| 14. | Differentiate Supervised Learning from Unsupervised Learning. |  | CO4 | An | 3 |
| 15. | List the three phases used to obtain the right specifications for the semantic model. |  | CO5 | R | 3 |
| 16. | Indicate the use of lexicalizations terms and state the reasons for its necessity in semantic model. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Evaluate the centrality measures for the given Instagram social network. | CO1 | A | 12 |
| 18. |  | Explain in detail the Resource Description Framework (RDF) ontology language with triple data model, vocabularies and suitable examples. | CO2 | A | 12 |
| 19. |  | Illustrate the ontological modeling of social relationships using the Description & Situations (D&S) ontology design pattern and explain the contextual relationships between entities with a suitable example. | CO3 | A | 12 |
| 20. |  | Explain the apriori algorithm for frequent itemset mining, outlining its step-by-step procedure with suitable example. | CO4 | An | 12 |
| 21. |  | Explain the semantic quality metrics with misleading interpretations and discuss their impact with suitable example. | CO5 | A | 12 |
| 22. |  | Apply the Eclat algorithm to determine the association rules for the given Instagram user activity data with a minimum support threshold of 2. User (TID); Interactions (Hashtags Used). Id 1; #Fitness, #HealthyFood, #Workout. Id 2; #Travel, #Photography, #Nature. Id 3; #Fitness, #Workout, #Yoga. Id 4; #Travel, #Nature. Id 5; #Photography, #Nature. Id 6; #Workout, #Yoga, #HealthyFood. | CO4 | A | 12 |
| 23. | a. | Determine the in-degree of B, outdegree of A, order of the graph, size of the graph, for the given sociogram with 5 vertices and 8 edges. | CO1 | A | 8 |
|  | b. | Determine the value of confidence in association rule mining, if the support containing item A and B is 10% and the support of A is 20%. | CO4 | A | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze the impact and challenges of expressiveness dilemma and content dilemma in semantic model with suitable example. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate the need for semantic Web and analyze properties of social networks using key measures in network analysis. |
| CO2 | Develop a model to aggregate the social network data using ontology languages. |
| CO3 | Apply ontology-based Knowledge Representation for modelling social individuals and social relationships. |
| CO4 | Implement machine learning models for social network analysis. |
| CO5 | Examine the pitfalls in social network analysis. |
| CO6 | Illustrate the dilemmas and their influence in designing semantic models. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21EC2036 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # METAHEURISTIC ALGORITHMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define the “local optima” in an optimization problem. |  | CO1 | R | 1 |
| 2. | Interpret the term “exploration” in search algorithms. |  | CO1 | U | 1 |
| 3. | Indicate the role of aspiration criterion in Tabu Search. |  | CO2 | U | 1 |
| 4. | Name the guiding principle used in Iterated Local Search. |  | CO2 | R | 1 |
| 5. | Indicate the primary purpose of mutation operator in Genetic Algorithm. |  | CO3 | U | 1 |
| 6. | Identify the selection method where the probability of being chosen is directly proportional to a solution's absolute fitness value. |  | CO3 | U | 1 |
| 7. | Name the primary organizational structure in the Shuffled Frog-Leaping Algorithm (SFLA)? |  | CO4 | R | 1 |
| 8. | Indicate the movement pattern followed by brightest firefly in the population. |  | CO4 | U | 1 |
| 9. | Define a “non-dominated’ solution in multi-objective optimization. |  | CO5 | R | 1 |
| 10. | List the advantage of using metaheuristic search in Machine Learning. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Contrast trajectory-based and population-based metaheuristics in terms of their exploration and exploitation capabilities. |  | CO1 | An | 3 |
| 12. | Infer the significance of Tabu list in preventing cycling in Tabu Search. |  | CO2 | An | 3 |
| 13. | Interpret the effect of selection pressure in Tournament Selection operator in Genetic Algorithm. |  | CO3 | An | 3 |
| 14. | Describe the dynamic interplay between Loudness (A) and Pulse Rate (r) in the Bat Algorithm as it approaches a solution |  | CO4 | U | 3 |
| 15. | Compare Apriori technique with Aposteriori technique in multi-objective optimization. |  | CO5 | An | 3 |
| 16. | Give an example of encoding method used in Travel Salesman Problem. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Classify metaheuristic algorithms and provide examples for each. | CO1 | An | 12 |
| 18. |  | Describe the Iterated Local Search (ILS) framework. Explain the critical role of the "perturbation" phase and how its strength affects the search trajectory. | CO2 | An | 12 |
| 19. |  | Analyze the growth phenomenon that happen in Differential evolution using a structured algorithm. | CO3 | An | 12 |
| 20. |  | Describe the processes of sorting the population, partitioning into memeplexes, and the mathematical position update of the worst frog within a memeplex. | CO4 | An | 12 |
| 21. |  | Define multi-modal optimization and explain why identifying multiple high-quality local optima is often highly valuable in practical engineering design. | CO5 | An | 12 |
| 22. |  | Discuss the three rules used to generate a new harmony vector. Analyze the specific mathematical impact of the Harmony Memory Consideration Rate (HMCR) and Pitch Adjustment Rate (PAR). | CO3 | An | 12 |
| 23. |  | Explain the fundamental principles of Pattern Search and detail the polling process. How does the algorithm dynamically adjust the mesh size (step length) based on the success or failure of finding a better solution in the current neighborhood? | CO2 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain how Differential Evolution can be used to optimize the placement of cluster centroids globally in a K-means clustering algorithm. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Devise an optimization problem using foundational principles of metaheuristic techinques. |
| CO2 | Apply local and global search metaheuristic algorithms for solving optimization problems. |
| CO3 | Implement and validate a computational model based on evolutionary algorithms. |
| CO4 | Adopt different nature based metaheuristic algorithms for solving optimization problems. |
| CO5 | Analyse metaheuristic algorithms for solving multi-objective, multi-modal and dynamic optimization problems. |
| CO6 | Apply metaheuristic algorithms in solving real-world problems. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21EC2037 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # HUMAN COMPUTER INTERFACE | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define ergonomics. |  | CO1 | R | 1 |
| 2. | Identify any two text-entry devices. |  | CO1 | U | 1 |
| 3. | State the purpose of encoding methods. |  | CO2 | R | 1 |
| 4. | Define Hick-Hyman Law. |  | CO2 | R | 1 |
| 5. | Name any two linguistic models used in HCI. |  | CO3 | R | 1 |
| 6. | Illustrate contextual inquiry with a simple example. |  | CO3 | U | 1 |
| 7. | List two non-verbal signals in face-to-face communication. |  | CO4 | R | 1 |
| 8. | Identify the different levels of dialog in HCI. |  | CO4 | U | 1 |
| 9. | List the main elements of the groupware framework. |  | CO5 | R | 1 |
| 10. | Identify the role of HTML in web development. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the elements of the WIMP Interface. |  | CO1 | A | 3 |
| 12. | Summarize the components of the Model Human Processor. |  | CO2 | U | 3 |
| 13. | Illustrate Norman’s Seven Principles with suitable examples. |  | CO3 | A | 3 |
| 14. | Differentiate between textual dialog notation and diagrammatic notation. |  | CO4 | An | 3 |
| 15. | Compare Meeting Support Systems and Decision Support Systems. |  | CO5 | An | 3 |
| 16. | Describe direct selection as a web interaction technique. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Apply the concepts of human memory, reasoning, and problem solving to a given HCI scenario and illustrate their impact with suitable examples. | CO1 | An | 08 |
|  | b. | Explain the working of positioning and pointing devices | CO1 | A | 04 |
| 18. | a. | Estimate the user task completion time using the Keystroke-Level Model (KLM) for a given scenario, with a suitable example. | CO2 | U | 6 |
|  | b. | Calculate the time required for a user to move the cursor from Button X to Button Y using Fitts’ Law, given that the distance between the centers of the buttons is D=250 px, the width of the target button (Button Y) is W=20 px, and the constants are a=0.2 sec and b=0.1 sec. | CO2 | A | 6 |
| 19. | a. | Explain Shneiderman’s Eight Golden Rules and examine their application in modern systems. | CO3 | A | 06 |
|  | b. | Illustrate the application of cognitive walkthrough in evaluating a mobile application with suitable examples. | CO3 | A | 06 |
| 20. |  | Analyze the differences between face-to-face communication and text-based communication in group working. Compare their effectiveness in collaborative decision-making. | CO4 | An | 12 |
| 21. | a. | Explain the time–space matrix used to classify groupware, and describe each quadrant with suitable examples. | CO5 | A | 08 |
|  | b. | Differentiate between mixed reality, augmented reality, and virtual reality, and provide one example of each. | CO5 | An | 04 |
| 22. |  | Illustrate the Glimpse Model and the Physical Model with suitable examples, and explain their role in predicting user performance. | CO2 | A | 12 |
| 23. |  | Examine diagrammatic and textual dialog notations and their types, and illustrate each with one example. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze an online shopping interface and examine how interaction techniques such as drag-and-drop, contextual tools, inlays, and process flow patterns enhance user productivity and experience. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the context of building human computer interaction system. |
| CO2 | Adopt different techniques to design an interactive system. |
| CO3 | Analyse different guiding principles in developing synergistic cognitive system. |
| CO4 | Understand communication interfaces used in human computer interactive system. |
| CO5 | Understand design methods in web interface. |
| CO6 | Apply methods to formulate support systems for implementing interactive system. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21EC2037 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # HUMAN COMPUTER INTERFACE | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List the types of human input-output channels. |  | CO1 | R | 1 |
| 2. | Identify any one text entry device. |  | CO1 | R | 1 |
| 3. | State the role of heuristics in operator placement. |  | CO2 | R | 1 |
| 4. | Define Fitts’ Law. |  | CO2 | R | 1 |
| 5. | Identify cognitive models used in HCI. |  | CO3 | U | 1 |
| 6. | Give an example of Secondary Stakeholders. |  | CO3 | U | 1 |
| 7. | Identify any one feature of text-based communication. |  | CO4 | U | 1 |
| 8. | Define group working in HCI. |  | CO4 | R | 1 |
| 9. | Identify the type of groupware used when team members collaborate simultaneously from different locations. |  | CO5 | U | 1 |
| 10. | State the purpose of HTTPS. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Classify the types of human memory in HCI. |  | CO1 | An | 3 |
| 12. | Explain task analysis with a suitable example. |  | CO2 | A | 3 |
| 13. | Explain the steps involved in conducting a cognitive walkthrough. |  | CO3 | A | 3 |
| 14. | Differentiate face-to-face communication from text-based communication. |  | CO4 | An | 3 |
| 15. | Compare synchronous with asynchronous groupware systems. |  | CO5 | An | 3 |
| 16. | Illustrate outlays and virtual pages as web design patterns with suitable examples. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Examine positioning, pointing, and drawing devices, and analyze their role in user interaction. | CO1 | A | 06 |
|  | b. | Explain the different interaction styles in HCI with suitable examples. | CO1 | A | 06 |
| 18. | a. | Examine the Model Human Processor and analyze its implications for interface design. | CO2 | A | 6 |
|  | b. | Calculate the decision time for a user using the Hick–Hyman Law when interacting with a software interface that displays 12 menu options, given a = 0.3 seconds and b = 0.15 seconds. | CO2 | A | 6 |
| 19. | a. | Explain Nielsen’s Ten Heuristics and other usability principles, and assess their effectiveness in interface design. | CO3 | An | 08 |
|  | b. | Distinguish between goal hierarchy and task hierarchy. | CO3 | An | 04 |
| 20. |  | Examine diagrammatic and textual dialog notations and their types, and illustrate each with one example. | CO4 | A | 12 |
| 21. | a. | Explain the architecture of client–server systems and illustrate their role in groupware applications. | CO5 | A | 08 |
|  | b. | Compare shared applications and shared artifacts in collaborative environments. | CO5 | An | 04 |
| 22. |  | Illustrate the application of task analysis in a given scenario and estimate the user task completion time using the Keystroke-Level Model (KLM), and explain how these techniques contribute to effective interface design. | CO2 | A | 12 |
| 23. |  | Examine dialog semantics, dialog analysis, and dialog design, and explain their roles in developing effective interactive systems with suitable examples. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze the web interface design of a file management system, and examine how drag-and-drop, contextual tools, inlays, and process flow patterns are applied to enhance user productivity. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the context of building human computer interaction system. |
| CO2 | Adopt different techniques to design an interactive system. |
| CO3 | Analyse different guiding principles in developing synergistic cognitive system. |
| CO4 | Understand communication interfaces used in human computer interactive system. |
| CO5 | Understand design methods in web interface. |
| CO6 | Apply methods to formulate support systems for implementing interactive system. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 22EC2002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SOCIAL MEDIA ANALYTICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define social media analytics. |  | CO1 | R | 1 |
| 2. | Define the velocity of data. |  | CO1 | R | 1 |
| 3. | List any two types of web spamming techniques. |  | CO2 | R | 1 |
| 4. | Indicate the importance of Hypertext Transfer Protocol Secure (HTTPS). |  | CO2 | U | 1 |
| 5. | Define centrality. |  | CO3 | R | 1 |
| 6. | Give an example of transitivity. |  | CO3 | U | 1 |
| 7. | Define social web mining. |  | CO4 | R | 1 |
| 8. | List the types of sentiments. |  | CO4 | R | 1 |
| 9. | Give an example of an indirect opinion. |  | CO5 | U | 1 |
| 10. | Indicate any two challenges faced by recommendation systems. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare ‘Data in Motion’ with ‘Data at Rest’. |  | CO1 | U | 3 |
| 12. | Summarize the process of information retrieval. |  | CO2 | U | 3 |
| 13. | Sketch a directed graph and identify the size of the graph and the number of edges. |  | CO3 | A | 3 |
| 14. | Describe the role of visualizing interest graphs in GitHub repository mining. |  | CO4 | U | 3 |
| 15. | Explain a sarcastic sentence in the context of social media analytics with an example. |  | CO5 | U | 3 |
| 16. | Discuss the role of collective behaviour analytics in understanding user interactions on social media. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the role of the internal social media domain in organizations with suitable use cases. | CO1 | A | 12 |
| 18. |  | Explain the different techniques employed in web spamming with suitable examples. | CO2 | A | 12 |
| 19. |  | Assess the evolution of random graph models and write about their significance in social network analysis. | CO3 | E | 12 |
| 20. |  | Analyze the challenges and opportunities associated with Facebook mining. | CO4 | An | 12 |
| 21. |  | Examine aspect-based sentiment analysis and write its applications and limitations using a practical example. | CO5 | A | 12 |
| 22. |  | Explain the application of sentiment analysis across different domains to extract valuable insights from textual data. | CO5 | A | 12 |
| 23. |  | Analyze the key metrics and methodologies used for evaluating recommendation systems and their role in assessing recommendation quality. | CO6 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain classical recommendation algorithms used in social media and highlight their advantages and limitations. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the concept of social media analytics and its significance |
| CO2 | Apply web analytics to the realistic data sets |
| CO3 | Analyse social network data to identify important social actors, subgroups and network properties in social media sites. |
| CO4 | Illustrate solutions to the emerging problems with social media |
| CO5 | Design new solutions to opinion extraction, sentiment classification problems |
| CO6 | Summarize the issues in social recommendation systems |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC2004 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DATA VISUALIZATION TECHNIQUES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the stage of the computer graphics pipeline responsible for removing objects not visible to the camera. |  | CO1 | U | 1 |
| 2. | Identify the visualization best suited for representing geographical data. |  | CO1 | U | 1 |
| 3. | Define vector data. |  | CO2 | R | 1 |
| 4. | State the purpose of using normalization in data visualization. |  | CO2 | R | 1 |
| 5. | Name the two parameters that define "Color" as a visual variable. Hue and saturation |  | CO3 | R | 1 |
| 6. | Define "marks" in the context of graphical primitives. |  | CO3 | R | 1 |
| 7. | State the meaning of "transfer functions" in volume rendering. |  | CO4 | R | 1 |
| 8. | Define vorticity. |  | CO4 | R | 1 |
| 9. | Define dot map. |  | CO5 | R | 1 |
| 10. | Give an example of a point-based visualization technique. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Distinguish between quantitative data and qualitative data. |  | CO1 | U | 3 |
| 12. | Determine the normalized value for a test score of 75 using min–max normalization, given that the observed scores range from 40 to 100. |  | CO2 | A | 3 |
| 13. | Compare “expressiveness” with “effectiveness” in data visualization. |  | CO3 | U | 3 |
| 14. | Describe pathlines, streaklines, and timelines in flow visualization. |  | CO4 | U | 3 |
| 15. | Discuss the importance of interpolation in surface map creation. |  | CO5 | U | 3 |
| 16. | Summarize the significance of Infogram in data visualization. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain different types of data visualization techniques and their applications. | CO1 | A | 12 |
| 18. |  | Explain the impact of missing data and extreme outliers on visualization accuracy and the effectiveness of imputation and outlier-handling strategies in sensitive applications such as medical visualization. | CO2 | A | 12 |
| 19. |  | Examine the suitability of the eight visual variables for conveying complex data patterns in data visualization with suitable examples. | CO3 | A | 12 |
| 20. |  | Analyze the use of "Cityscapes" and "Rubber Sheets" in visualizing air pollution density, including the construction of both techniques. | CO4 | An | 12 |
| 21. |  | Analyze the advantages and limitations of cartograms compared to choropleth maps. | CO5 | An | 12 |
| 22. |  | Examine various geospatial visualization techniques and their suitability for representing air pollution data collected from sensors across a city. | CO5 | A | 12 |
| 23. |  | A marketing team needs to create interactive dashboards for large datasets with real-time updates, while a content creator wants to design simple, visually appealing infographics for social media sharing. Explain the suitability of Tableau and Infogram for the given scenarios by describing their features and capabilities. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Write a Python program to visualize the following dataset using appropriate plots. Select suitable visualization techniques, justify your choices, and interpret the results. Day; Temperature (°C). Monday; 30. Tuesday; 32. Wednesday; 31. Thursday; 29. Friday; 33. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Outline the process of visualization and computer graphics. |
| CO2 | Identify the different data types in the context of visualization. |
| CO3 | Organize the various visualization methods into taxonomies/categories. |
| CO4 | Relate the spatial attributes of the data to graphical attributes. |
| CO5 | Apply visualization techniques in the field of geographic information systems. |
| CO6 | Create and interpret plots using software tools. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC2006 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DEEP LEARNING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define feature extraction in deep learning. |  | CO1 | R | 1 |
| 2. | Name two common optimization algorithms used in training deep networks. |  | CO1 | R | 1 |
| 3. | Define convolution operation in CNNs. |  | CO2 | R | 1 |
| 4. | List two activation functions used in deep networks. |  | CO2 | R | 1 |
| 5. | State the role of regularization in deep learning. |  | CO3 | R | 1 |
| 6. | Indicate any one advantage of using an LSTM over a standard RNN. |  | CO3 | R | 1 |
| 7. | Define the term ‘latent space’ in autoencoders. |  | CO4 | R | 1 |
| 8. | Indicate the purpose of hyperparameter tuning. |  | CO4 | R | 1 |
| 9. | List any two deep learning frameworks. |  | CO5 | R | 1 |
| 10. | Give one real-world application of deep learning in media. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | A startup struggles with high-dimensional data. Justify the preference of deep learning over traditional Machine Learning in the given scenario. |  | CO1 | E | 3 |
| 12. | Interpret the role of activation functions in avoiding linearity in deep networks. |  | CO2 | A | 3 |
| 13. | Differentiate ‘AlexNet architecture’ from ‘ResNet architecture’. |  | CO3 | An | 3 |
| 14. | Compare GANs with Auto encoders in terms of learning approach. |  | CO4 | An | 3 |
| 15. | Examine the impact of learning rate on convergence. |  | CO5 | A | 3 |
| 16. | Summarize the applications of deep learning in medical image analysis. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Develop a machine learning model for classifying handwritten digits highlighting its merits and demerits. | CO1 | A | 12 |
| 18. | a. | Evaluate the importance of hyperparameters (learning rate, batch size, layers) in deep network performance. | CO2 | E | 6 |
|  | b. | Explain the AlexNet architecture and write the real-time applications of AlexNet. | CO3 | U | 6 |
| 19. |  | Analyze the architecture of CNN and explain how convolution, pooling, and fully connected layers contribute to feature extraction. | CO3 | An | 12 |
| 20. | a. | Compare DBNs with GANs in terms of architecture and applications | CO4 | An | 6 |
|  | b. | Explain the architecture and applications of long short-term memory (LSTM) networks. | CO4 | U | 6 |
| 21. | a. | Discuss the challenges in training deep networks and strategies to overcome them. | CO5 | U | 6 |
|  | b. | Explain the different categories of hyperparameters used in training a deep learning network. | CO5 | U | 6 |
| 22. |  | Examine the architectural components of deep neural networks. How do layers and loss functions contribute to model performance? | CO2 | A | 12 |
| 23. |  | A deep network shows slow convergence and unstable loss. Develop a tuning strategy involving learning rate, initialization, and activation functions. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Evaluate the role of deep learning in healthcare and sentiment analysis with suitable examples. | CO6 | E | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Examine the machine learning and deep learning algorithms |
| CO2 | Assess the architecture and components of deep networks, including layers, neurons, activation functions and loss functions. |
| CO3 | Evaluate the convolutional neural network architectures including AlexNet and ResNet. |
| CO4 | Apply deep learning architectures to solve real-time problems. |
| CO5 | Examine the concepts in tuning deep networks to enhance model accuracy. |
| CO6 | Assess the deep learning applications in the field of healthcare and media. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC2007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # NATURAL LANGUAGE PROCESSING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define Natural Language in the context of NLP. |  | CO1 | R | 1 |
| 2. | Identify the NLP level for the given sentence: “Can you pass the salt?”. |  | CO1 | R | 1 |
| 3. | List the objectives of Natural Language Processing. |  | CO2 | R | 1 |
| 4. | Interpret the significance of Speech Synthesis. |  | CO2 | U | 1 |
| 5. | Define Context-Free Grammar. |  | CO3 | R | 1 |
| 6. | Identify the method of parsing that uses probability. |  | CO3 | R | 1 |
| 7. | Define lexical semantics. |  | CO4 | R | 1 |
| 8. | Describe the principle of compositionality. |  | CO4 | U | 1 |
| 9. | Define Zero Anaphora. |  | CO5 | R | 1 |
| 10. | List the types of text summarization techniques. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Distinguish ‘Natural Language Understanding’ from ‘Natural Language Generation’. |  | CO1 | An | 3 |
| 12. | Describe the role of an acoustic signal in speech processing. |  | CO2 | U | 3 |
| 13. | Differentiate top-down parsing from bottom-up parsing with an example. |  | CO3 | An | 3 |
| 14. | Compare symbolic approach with distribution approach. |  | CO4 | An | 3 |
| 15. | List the types of anaphora in discourse. |  | CO5 | R | 3 |
| 16. | Describe the key components of Machine Translation. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Analyze the role of regular expressions in text processing and illustrate their use in pattern matching applications. | CO1 | An | 6 |
|  | b. | Explain the fundamentals of NLP and its major applications. | CO1 | A | 6 |
| 18. |  | Apply the concept of speech synthesis to demonstrate the conversion of text into speech using phoneme generation and synthesis methods with a neat block diagram. | CO2 | A | 12 |
| 19. | a. | Explain Context-Free Grammar (CFG) and its components with an example. | CO3 | A | 8 |
|  | b. | Interpret the representation of meaning in Natural Language Processing systems. | CO3 | A | 4 |
| 20. | a. | Explain computational lexical semantics with suitable examples. | CO4 | A | 8 |
|  | b. | Differentiate semantic analysis from pragmatic analysis. | CO4 | An | 4 |
| 21. |  | Analyze the role of lexical resources such as WordNet, PropBank, and FrameNet in NLP. | CO5 | An | 12 |
| 22. |  | Devise the components and applications of Natural Language Processing with suitable examples. | CO1 | An | 12 |
| 23. |  | Examine the different types of lexical semantic relations and its significance in understanding language. | CO4 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Analyze the pipeline of an Information Extraction system with a suitable example. | CO6 | An | 8 |
|  | b. | Infer the challenges of Machine Translation. | CO6 | A | 4 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Evaluate regular expressions, automata, and transducers for text processing. |
| CO2 | Examine phonetics, synthesis, and recognition for advanced speech processing. |
| CO3 | Assess grammars, parsing, and complexity for syntactic analysis. |
| CO4 | Examine meaning, lexical semantics, and discourse for computational language understanding. |
| CO5 | Critique discourse analysis and coreference using lexical resources like WordNet and FrameNet. |
| CO6 | Assess applications in extraction, answering, summarization, and translation for advanced NLP |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC2008 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | INTRODUCTION TO HUMAN COMPUTER INTERACTION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define ergonomics. |  | CO1 | R | 1 |
| 2. | List two examples of I/O channels in HCI. |  | CO1 | R | 1 |
| 3. | State the purpose of using scenarios in design. |  | CO2 | R | 1 |
| 4. | List the key elements of screen design. |  | CO2 | R | 1 |
| 5. | Define a stakeholder in HCI design. |  | CO3 | R | 1 |
| 6. | Identify a real-time communication model used in HCI and state its purpose. |  | CO3 | U | 1 |
| 7. | Name any one application of affect detection. |  | CO4 | R | 1 |
| 8. | Identify physiological signals used in emotion detection. |  | CO4 | U | 1 |
| 9. | Define Brain-Computer Interaction. |  | CO5 | R | 1 |
| 10. | Identify an example of a mobile platform. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain interaction paradigms with examples. |  | CO1 | A | 3 |
| 12. | Differentiate scenarios from navigation design in HCI. |  | CO2 | An | 3 |
| 13. | Illustrate different collaboration models with suitable examples. |  | CO3 | A | 3 |
| 14. | Compare affect induction and affect detection methods. |  | CO4 | An | 3 |
| 15. | Discuss the importance of neurocognitive engineering in improving human–computer interaction. |  | CO5 | U | 3 |
| 16. | Examine the components of a mobile ecosystem and explain their roles. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Examine human memory, reasoning, and problem-solving processes and illustrate their impact on HCI systems. | CO1 | A | 12 |
| 18. | a. | Determine the various types of design rules in HCI and design a user interface by applying them. | CO2 | A | 06 |
|  | b. | Explain the role of scenarios, navigation, and screen design in creating effective user interfaces. | CO2 | A | 06 |
| 19. |  | Examine cognitive models in HCI and determine their role in improving user interface design and user behavior understanding. | CO3 | A | 12 |
| 20. | a. | Evaluate the foundations of affective sciences and assess the relevance of major theories applied in HCI. | CO4 | An | 06 |
|  | b. | Classify the major affect detection methods and explain their applications in detail. | CO4 | An | 06 |
| 21. | a. | Examine the architecture of a BCI system and illustrate its components with a diagram. | CO5 | A | 06 |
|  | b. | Explain sensory substitution devices and interpret their role in assisting differently-abled users. | CO5 | A | 06 |
| 22. |  | Analyze the basic process of interactive design and examine the relationship between its stages with suitable examples. | CO2 | An | 12 |
| 23. |  | Explain socio-organizational issues and interpret their impact on system design and implementation. | CO3 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Determine appropriate mobile design tools and illustrate their usage in developing user-friendly mobile interfaces. | CO6 | A | 06 |
|  | b. | Analyze the user interface of a banking application and examine the interaction techniques used, with suitable examples. | CO6 | An | 06 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Comment on the fundamental components of an interactive system. |
| CO2 | Assess the design process of a usable interactive system within a software engineering framework. |
| CO3 | Examine models of cognitive, collaboration and group interaction. |
| CO4 | Implement groupware systems and examine the role of the world wide web in popularizing hypertext. |
| CO5 | Appraise the essentials of Mobile HCI. |
| CO6 | Execute web interface design. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC2015 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # COMMUNICATION THEORY AND SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State the limitations of a channel. |  | CO1 | R | 1 |
| 2. | Identify the transmission mode wherein the device can act as sender and receiver. |  | CO1 | U | 1 |
| 3. | Determine the modulation index, if Em=10V and Ec = 50V. |  | CO2 | A | 1 |
| 4. | Identify the lower side band frequency, if fm=500Hz and fc = 100 KHz. |  | CO2 | U | 1 |
| 5. | Define Frequency modulation. |  | CO3 | R | 1 |
| 6. | For a FM modulator with a peak frequency deviation Δf = 10KHz, a modulating signal frequency fm = 100KHz, Determine the modulation index β. |  | CO3 | A | 1 |
| 7. | Define selectivity of a radio receiver. |  | CO4 | R | 1 |
| 8. | Select the oscillator frequency fo, if fs is 545 KHz and IF is 455 KHz. |  | CO4 | A | 1 |
| 9. | Identify the circuit used in FM receiver to restrict amplitude. |  | CO5 | U | 1 |
| 10. | Define Noise figure. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the functions of a receiver. |  | CO1 | U | 3 |
| 12. | Compare AM (DSB-FC) with DSB-SC and SSB-SC AM. |  | CO2 | An | 3 |
| 13. | State the advantages of FM. |  | CO3 | R | 3 |
| 14. | Explain the methods used for Image frequency rejection. |  | CO4 | A | 3 |
| 15. | Explain the role of AFC in stable and accurate FM reception. |  | CO5 | A | 3 |
| 16. | Describe Noise temperature. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | List the need for wireless communication and explain the communication system with a neat block diagram. | CO1 | R | 6 |
|  | b. | Classify the various types of modulation used in communication systems. | CO1 | An | 6 |
| 18. |  | Deduce the total power of an Amplitude modulated wave and analyze the power of the various components in an AM wave. | CO2 | An | 12 |
| 19. | a. | A 50 MHz carrier is frequency modulated by a sinusoidal signal such that the maximum frequency deviation being 50 KHz. Determine the bandwidth of the FM signal if the frequency of the modulating sinusoid is (a) 600 KHz (b) 500 Hz (c) 10 KHz | CO3 | A | 6 |
|  | b. | Compare the performance of AM with FM. | CO3 | An | 6 |
| 20. |  | Describe the working of a Super heterodyne receiver with the help of a block diagram. Illustrate the image frequency signal that affect the receiver performance. | CO4 | A | 12 |
| 21. | a. | Explain FM transmitter with neat a block diagram. | CO5 | U | 6 |
|  | b. | Illustrate the methods of frequency stabilization for a particular application. | CO5 | A | 6 |
| 22. |  | Deduce the AM wave equation and analyze each component with the help of frequency spectrum. | CO2 | An | 12 |
| 23. |  | Explain the key receiver characteristics that determine its overall performance and quality. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Deduce the output signal to noise ratio of a SSB-SC system. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply suitable modulation and transmission mode in communication applications. |
| CO2 | Estimate the power and bandwidth requirements of AM, DSBSC, SSBSC and VSB systems. |
| CO3 | Choose direct or indirect FM generation for communication. |
| CO4 | Construct AM transmitter and receiver for communication. |
| CO5 | Evaluate the performance of FM transmitter and receiver. |
| CO6 | Calculate the output signal to noise ratio of DSBSC and SSBSC systems. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 22EC2016 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DIGITAL COMMUNICATION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define a Discrete Memoryless Source (DMS). |  | CO1 | R | 1 |
| 2. | Identify the primary objective of Huffman coding. |  | CO1 | U | 1 |
| 3. | State the primary application of DPCM in digital communication. |  | CO2 | R | 1 |
| 4. | Identify the type of encoding where no character code is the prefix of another character code. |  | CO2 | U | 1 |
| 5. | Express the Nyquist’s criterion for distortionless transmission. |  | CO3 | U | 1 |
| 6. | Illustrate Binary Symmetric Channel. |  | CO3 | U | 1 |
| 7. | Name the two frequency states in BFSK represented on a signal diagram. |  | CO4 | R | 1 |
| 8. | State the use of QPSK in military radios. |  | CO4 | R | 1 |
| 9. | Define code rate. |  | CO5 | R | 1 |
| 10. | Recall the expression for processing gain in Direct Sequence Spread Spectrum. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Calculate the entropy of a binary source emitting ‘1’ with P = 0.6 and ‘0’ with P = 0.4. |  | CO1 | A | 3 |
| 12. | Examine the line encoding techniques for the following data: 1 0 0 1 1 1 0 0 1. - Unipolar RZ - Polar NRZ - Manchester encoding |  | CO2 | A | 3 |
| 13. | Explain the effect of pulse shaping on ISI using raised-cosine filtering. |  | CO3 | An | 3 |
| 14. | Explain the generation process of DPSK signals. |  | CO4 | U | 3 |
| 15. | State any three properties of a linear block code. |  | CO5 | R | 3 |
| 16. | State the properties of a PN sequence. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | A discrete memoryless source emits symbols A–E with probabilities 0.3, 0.25, 0.2, 0.15 and 0.1. Using Huffman code, find the average code-word length, efficiency. | CO1 | A | 12 |
| 18. |  | Explain the principle and operation of a Delta Modulation transmitter and receiver, along with their block diagrams. | CO2 | A | 12 |
| 19. | a. | Illustrate the concept of Inter-Symbol Interference. | CO3 | A | 6 |
|  | b. | Explain the Nyquist criterion for a baseband binary data transmission system. | CO3 | A | 6 |
| 20. |  | Discuss the principle and operation of a BPSK transmitter and receiver with the help of a block diagram and signal space diagram. | CO4 | U | 12 |
| 21. |  | Consider a (7,4) linear block code whose generator matrix is - Find all the code vectors and hamming weights. - Determine the error detecting and correcting capability. - Find the parity check matrix H. - Find the error if received code is (100011). | CO5 | A | 12 |
| 22. |  | Apply the Shannon–Fano coding algorithm to the message ensemble consisting of symbols [A, B, C, D, E, F, G] with corresponding probabilities [0.4, 0.2, 0.12, 0.08, 0.08, 0.08, 0.04] and determine the - Shannon–Fano code for these symbols - Average code-word length - Coding efficiency. | CO1 | A | 12 |
| 23. |  | A convolution encoder has a shift register with two stages (i.e constraint length k=3), three modulo 2 adders and an output multiplexer. The generator sequences are as follows: g(1) = (1,1,1), g(2) = (1,0,1). - Draw the block diagram of the encoder. - Determine the output digit sequence for the data digits 10011. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | With neat block diagram, explain the operation of a Frequency Hopping Spread Spectrum (FHSS) system. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Infer the basics of information theory and source coding techniques to determine the required data rate for a reliable communication over the channel. |
| CO2 | Depict the performance of different baseband modulation techniques. |
| CO3 | Acquire Knowledge on inter symbol interference and its solutions. |
| CO4 | Design and simulate various passband modulation schemes. |
| CO5 | Compare and resolve various error control coding techniques for the encoding and decoding of digital data streams for their reliable transmission over noisy channels. |
| CO6 | Discern various spreading techniques and its applications. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 22EC2018 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SIGNAL PROCESSING AND ITS APPLICATIONS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the number of zeros to be padded to perform circular convolution between 𝑥1(𝑛) and 𝑥2(𝑛). Length of 𝑥1(𝑛) 𝑖𝑠 7 and the length of 𝑥2(𝑛) 𝑖𝑠5. |  | CO1 | U | 1 |
| 2. | Define Nyquist interval. |  | CO1 | R | 1 |
| 3. | Name the property that makes the 2-D DCT attractive for image coding. |  | CO2 | R | 1 |
| 4. | Estimate the number of butterfly stages needed for a 32-point radix-2 DIT-FFT. |  | CO2 | U | 1 |
| 5. | Interpret the magnitude response of Chebyshev type I LPF. |  | CO3 | U | 1 |
| 6. | Illustrate the IIR filter design technique that provides a many-to-one mapping of frequencies from analog to digital domains. |  | CO3 | U | 1 |
| 7. | Define window function in FIR filter design. |  | CO4 | R | 1 |
| 8. | State Gibb’s phenomenon. |  | CO4 | R | 1 |
| 9. | Define adaptive filters. |  | CO5 | R | 1 |
| 10. | List the primary color models used in image processing. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Evaluate the linear convolution of the sequences x1(n) = {-1,2,1,1,1,1} and x2(n) = {1,2,2}. |  | CO1 | An | 3 |
| 12. | Compare radix-2 DIT-FFT with radix-2 DIF-FFT algorithms. |  | CO2 | An | 3 |
| 13. | Compare analog filters with digital filters. |  | CO3 | An | 3 |
| 14. | Summarize the properties of rectangular and hamming window functions. |  | CO4 | U | 3 |
| 15. | Write the weight update equation of the LMS algorithm. |  | CO5 | A | 3 |
| 16. | Differentiate 8-adjacency from 4-adjacency in pixel relation. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Apply overlap add method to perform sectioned convolution for the given sequences, x(n) = {-2,1,-2,1,2,1,3,1,1} and h(n) = {-1,1,1}. | CO1 | A | 12 |
| 18. |  | Compute the 8-point DFT of x[n] = {4, 4, 4, 4, 4, 4, 4, 4} using radix-2 DIT-FFT, show the butterfly outputs per stage. | CO2 | A | 12 |
| 19. |  | Estimate a Butterworth filter for the following specifications using the bilinear transformation technique. Assume T = 1 sec. 0.707≤Hejω≤1; for 0≤ω≤π/2 H(ejω)≤0.2; for 3π/4≤ω≤π | CO3 | E | 12 |
| 20. |  | Determine a linear phase FIR digital filter’s coefficient for the given specifications using Hanning window of length N=7. Hd(ω)= e-j5ω; for ω≤π/20; for π/2<ω≤π | CO4 | A | 12 |
| 21. |  | Explain the basic wiener filter theory in detail and state the steps of Least Mean Square algorithm. | CO5 | A | 12 |
| 22. | a. | Determine whether the FIR filter with the coefficients h[n]={0.25,0.5,0.75,0.5,0.25} is a linear phase filter, and draw its linear phase realization structure. | CO4 | A | 6 |
|  | b. | Develop the linear phase realization structure for the following system function. H(z) = 0.45 + 0.32z-1 + 0.21z-2 + 0.25 z-3 + 0.21z-4 + 0.32z-5 +0.45z-6 | CO4 | A | 6 |
| 23. |  | Sketch the direct form-I and direct form – II realization for the LTI system Hz=1 + 0.75z-1 + 0.5z-21 - z-1+ 0.2z-2 | CO3 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the smoothing and sharpening spatial filters in detail. | CO6 | A | 6 |
|  | b. | Summarize the speech spectrogram with an illustration. | CO6 | U | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define signals and system mathematically in discrete time domain. |
| CO2 | Formulate the Discrete-Fourier Transform (DFT) and the FFT algorithms. |
| CO3 | Explain the various transformations for digital IIR filter design procedures. |
| CO4 | Design FIR digital filters for various applications. |
| CO5 | Outline the concepts of adaptive filtering algorithms. |
| CO6 | Make use of signal processing techniques for speech and image applications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC2023 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # IoT SECURITY AND TRUST | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define symmetric encryption. |  | CO1 | R | 1 |
| 2. | State one advantage of Elliptic Curve Cryptography. |  | CO1 | R | 1 |
| 3. | Define edge security. |  | CO2 | R | 1 |
| 4. | List two communication networks used in IoT. |  | CO2 | R | 1 |
| 5. | Define threat modeling. |  | CO3 | R | 1 |
| 6. | Identify one effect of default passwords in IoT devices. |  | CO3 | U | 1 |
| 7. | Define trust management lifecycle. |  | CO4 | R | 1 |
| 8. | State the purpose of the web of trust model. |  | CO4 | R | 1 |
| 9. | Define lightweight cryptography. |  | CO5 | R | 1 |
| 10. | Define cyber forensics. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the goals of cryptography. |  | CO1 | U | 3 |
| 12. | Describe the CIA triad with reference to IoT systems. |  | CO2 | U | 3 |
| 13. | Differentiate ‘user-centric’ from ‘device-centric’ identity management models in IoT. |  | CO3 | U | 3 |
| 14. | Explain the phases involved in mutual trust establishment. |  | CO4 | U | 3 |
| 15. | Compare identity-based with identity-driven access control in IoT. |  | CO5 | U | 3 |
| 16. | Explain the role of incident response in handling cybercrime. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Analyze the structure and working of the Advanced Encryption Standard algorithm. | CO1 | An | 12 |
| 18. | a. | Analyze the IIoT security framework with respect to hardware security, boot process, operating system, kernel, application, runtime environment, and containers. | CO2 | An | 6 |
|  | b. | Compare internet, intranet, LAN, and wireless cellular networks for securing IoT communication. | CO2 | An | 6 |
| 19. | a. | Analyze the common vulnerabilities present in IoT devices. | CO3 | An | 8 |
|  | b. | Evaluate the key elements of threat modeling for a smart home IoT system. | CO3 | E | 4 |
| 20. | a. | Analyze the trust management lifecycle in IoT. | CO4 | An | 8 |
|  | b. | Apply an identity management framework to a smart healthcare IoT system. | CO4 | A | 4 |
| 21. | a. | Analyze the need and methods of lightweight cryptography. | CO5 | An | 6 |
|  | b. | Compare capability-based, identity-based, and identity-driven access control schemes used in IoT. | CO5 | An | 6 |
| 22. | a. | Compute the shared secret key in Diffie–Hellman key exchange for given values of p, g, a, and b. | CO1 | A | 6 |
|  | b. | Evaluate the advantages of AES over DES for securing IoT communication. | CO1 | E | 6 |
| 23. | a. | Analyze the roles of white hat, black hat, and grey hat hackers in cyber security. | CO6 | An | 6 |
|  | b. | Evaluate the measures used to deal with the rise of cybercrimes in IoT systems. | CO6 | E | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze the types of cybercrime and the methods for preventing it. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design and implement cryptography algorithms using C programs |
| CO2 | Solve network security problems in various networks |
| CO3 | Build security systems using elementary blocks |
| CO4 | Build Trustable cloud based IoT systems |
| CO5 | Solve IoT security problems using light weight cryptography |
| CO6 | Appreciate the need for cyber security laws and methods. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC2024 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # COGNITIVE IOT | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the network that marked the beginning of the Internet’s evolution in the late 1960s. |  | CO1 | R | 1 |
| 2. | Examine the CIoT framework and interpret the significance of the mnemonic “LADA.” |  | CO1 | A | 1 |
| 3. | Interpret the role performed by the “Act” phase in the cognitive cycle. |  | CO2 | U | 1 |
| 4. | Select the most appropriate technology that supports intelligent spectrum reuse in CIoT. |  | CO2 | R | 1 |
| 5. | Identify the university computing setup and determine the computing era it represents. |  | CO3 | A | 1 |
| 6. | Infer the type of data predicted by regression. |  | CO3 | U | 1 |
| 7. | Identify the cloud computing model used in CIoT. |  | CO4 | U | 1 |
| 8. | Define DoS attack. |  | CO4 | R | 1 |
| 9. | Stata the concept of CNN training. |  | CO5 | R | 1 |
| 10. | Define GPU. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the fundamental traits of Cognitive Radio. |  | CO1 | U | 3 |
| 12. | Describe the importance of edge computing in IoT. |  | CO2 | U | 3 |
| 13. | List the key characteristics of Second Wave in IoT development. |  | CO3 | R | 3 |
| 14. | Summarize distributed computing. |  | CO4 | U | 3 |
| 15. | Describe the need for security in Cognitive IoT. |  | CO5 | U | 3 |
| 16. | Compare GPU with CPU architectures. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Compare and analyze CIoT and traditional computing paradigms in terms of architecture, data processing, scalability, and real-time decision-making. | CO1 | A | 8 |
|  | b. | Explain the applications of CIoT in various domains. | CO1 | A | 4 |
| 18. |  | Explain the enabling technologies of CIoT and analyze their roles in supporting intelligent and autonomous system operations. | CO1 | A | 12 |
| 19. |  | Explain ANN based classification for sensing remote health care applications. | CO2 | A | 12 |
| 20. | a. | Explain the role of data analytics in Cognitive IoT. | CO2 | A | 6 |
|  | b. | Explain how regression analytics can be applied in an IoT-based smart agriculture system. | CO2 | A | 6 |
| 21. | a. | Differentiate the key features of Fog computing from Cloudlets in IoT. | CO3 | An | 6 |
|  | b. | Describe cloud service models and examine their real-world applications. | CO3 | A | 6 |
| 22. |  | Explain the architecture of GPUs and analyze their role in parallel computing. | CO4 | A | 12 |
| 23. |  | Explain the concept of digital twin and evaluate its applications in IoT. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Discuss the major security issues in IoT at the device level and network level. | CO6 | U | 6 |
|  | b. | Identify the importance of security in Cognitive IoT. | CO6 | U | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Integrate the aspects of human cognitive processes in the system design. |
| CO2 | Comprehend the underlying cognitive process can have many abstractions of a cognitive cycle such as ‘Sense’, ‘Understand’, ‘Decide’ and ‘Act’. |
| CO3 | Detect any failures of system components and re-configure itself, which provides a graceful degradation through self-healing. |
| CO4 | Accomplish knowledge about the application, system architecture, resources, system state and behavior. |
| CO5 | Incorporate recent advancements in the machine learning including deep learning in IOT. |
| CO6 | Analyze security issues in IoT applications. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 22EC2027 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # BRAIN COMPUTER INTERFACE | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Infer the distinct rhythmic EEG activity frequency bands in the human brain. |  | CO1 | An | 1 |
| 2. | Define magnetoencephalography. |  | CO1 | R | 1 |
| 3. | Relate the cognitive processes and neural mechanisms associated with the P300 component. |  | CO2 | U | 1 |
| 4. | Predict the common sources of artefacts in BCI. |  | CO2 | A | 1 |
| 5. | Identify the key techniques employed for performing frequency domain analysis in Brain-Computer Interfaces (BCIs). |  | CO3 | R | 1 |
| 6. | Define the Phase-Locking Value (PLV). |  | CO3 | R | 1 |
| 7. | Define the term "margin" used in Support Vector Machines. |  | CO4 | R | 1 |
| 8. | Infer the underlying idea behind ensemble methods in machine learning. |  | CO4 | An | 1 |
| 9. | Indicate the importance of feature extraction in brain signal analysis. |  | CO5 | U | 1 |
| 10. | Explain the role of signal processing in BCI systems. |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare the signal acquisition methods in EEG-based and ECoG-based BCIs. Which has better spatial resolution and why? |  | CO1 | U | 3 |
| 12. | Determine the characteristics of slow cortical potentials. |  | CO2 | A | 3 |
| 13. | Calculate covariance for the following data x = {6,5,3,4,2}, y = {12,10,8,6,4} |  | CO3 | A | 3 |
| 14. | Differentiate between multilayer neural networks and single-layer neural networks in terms of structure, learning capacity, and practical applications. |  | CO4 | U | 3 |
| 15. | Estimate the recall value for a classifier network that has TP = 5 and FP = 10, TN = 10, FN = 5. |  | CO5 | E | 3 |
| 16. | Explain the structure of a Radial Basis Function (RBF) network. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the role of EEG electrodes in capturing brain signals and describe a commonly used system for analyzing these signals. | CO1 | U | 12 |
| 18. |  | Analyze the structure of a Brain-Computer Interface (BCI) system and evaluate its applications in various fields such as healthcare, communication, and entertainment. | CO2 | An | 12 |
| 19. |  | Evaluate how Independent Component Analysis (ICA) can be applied to complex signal processing scenarios with suitable example. | CO3 | E | 12 |
| 20. |  | Determine the steps involved in developing and implementing a perceptron neural network. | CO4 | A | 12 |
| 21. |  | Illustrate the procedure used in Graph Theoretical Functional Connectivity Analysis and its effectiveness in understanding brain network dynamics. | CO4 | A | 12 |
| 22. | a. | Discuss the advantages and limitations of using Support Vector Machines (SVM) in Brain-Computer Interface (BCI) applications. | CO4 | U | 6 |
|  | b. | Articulate the ethical implications of using Brain-Computer Interfaces (BCIs) in vulnerable populations, such as patients with neurological disorders. | CO5 | A | 6 |
| 23. |  | Brain-Computer Interfaces (BCIs) can be utilized to detect and classify emotions. Justify your answer. | CO5 | E | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Illustrate the architecture of the BCILAB toolbox and its plug-in concepts. | CO6 | A | 8 |
|  | b. | Determine the steps involved in recording Electrocorticography. | CO1 | A | 4 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Comprehend the components of BCI system. |
| CO2 | Apply concepts to assess the brain activation. |
| CO3 | Select appropriate feature extraction methods. |
| CO4 | Use machine learning algorithms for BCI. |
| CO5 | Explore the applications of BCI. |
| CO6 | Execute BCI using BCILAB toolbox. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC3009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # 5G AND BEYOND COMMUNICATIONS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Explain the goals and vision of next generation communication systems. | CO1 | A | 10 |
|  | b. | Interpret the key efforts towards creating “Green and Soft” networks focusing on reducing the environmental footprint of 5G. | CO1 | U | 10 |
|  |  | (OR) |  |  |  |
| 2. |  | Differentiate D2D communication with traditional cellular communication. Explain how does the integration of D2D and machine type communication (MTC) facilitate the realization of massive IoT (mIoT) framework? | CO2 | An | 20 |
| 3. |  | Compare point-to-point MIMO, multi-user MIMO, and Massive MIMO systems in terms of capacity, complexity, and scalability. Provide a critical evaluation of their suitability for 5G/6G systems. | CO3 | An | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | Compute the challenges of mmWave communication, including propagation loss, blockage, and channel estimation. Derive solutions using advanced beamforming techniques. | CO4 | A | 20 |
| 5. |  | Develop a machine learning-based framework for spectrum sharing in mmWave systems. Justify the choice of algorithms and evaluate performance metrics. | CO5 | A | 20 |
|  |  | (OR) |  |  |  |
| 6. | a. | Examine the 5G requirements that supports next generation application and connectivity. | CO1 | A | 10 |
|  | b. | Predict the emerging trends in mobile applications and services. | CO1 | U | 10 |
| 7. |  | Examine the role of machine learning in channel modeling and estimation. Compare traditional model-based approaches with data-driven approaches and evaluate their effectiveness. | CO5 | A | 20 |
|  |  | (OR) |  |  |  |
| 8. |  | Evaluate the vision of 6G networks, including terahertz communication, visible light communication, and AI integration. Determine a futuristic architecture addressing ethical, energy, and performance challenges. | CO6 | An | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Energy Harvesting (EH) is a key enabler for zero-power IoT. Compare the efficiency and implementation challenges of RF Energy Harvesting versus Energy Harvesting from Light in the context of 6G networks. | CO6 | An | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the trends, goals and vision of next generation wireless communication systems. |
| CO2 | Apply the basic understandings of key technologies to solve the existing problems of the 5G communications. |
| CO3 | Identify the state-of-the-art problems and apply the basic knowledge of massive MIMO for finding out the solutions. |
| CO4 | Analyse the performance of various physical layer technologies like mm-wave and multiple access techniques from physical layer perspective. |
| CO5 | Familiarise with 6G key enabling technologies like millimetre-wave communications, terahertz communications, and visible light communications. |
| CO6 | Acquire basic knowledge on 5G based machine learning applications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC3010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # MACHINE LEARNING FOR ENGINEERS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Evaluate the performance of a learning algorithm using cross-validation and discuss its advantages over simple train-test split. | CO1 | E | 20 |
|  |  | (OR) |  |  |  |
| 2. | a. | Illustrate the working of logistic regression for binary classification. | CO1 | A | 10 |
|  | b. | Differentiate between supervised, unsupervised, and reinforcement learning based on data, objective, and application. | CO1 | An | 10 |
| 3. | a. | Analyze the functional similarities between a biological neuron and an artificial neuron with a neat mathematical representation. | CO2 | An | 10 |
|  | b. | Differentiate between feedforward and feedback ANN architectures based on structure and information flow. | CO2 | An | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Evaluate the impact of different activation functions on the learning capability of an artificial neuron. | CO2 | E | 10 |
|  | b. | Examine the role of the human brain’s organization in inspiring artificial neural network design. | CO2 | A | 10 |
| 5. |  | Analyze optimization in backpropagation by deriving weight update equations and discussing convergence issues. | CO3 | An | 20 |
|  |  | (OR) |  |  |  |
| 6. | a. | Examine the stability in Adaptive Resonance Theory networks under varying vigilance parameters. | CO3 | A | 10 |
|  | b. | Illustrate clustering behavior in a Kohonen network for input vectors in 2D space using competitive learning. | CO3 | A | 10 |
| 7. | a. | Justify the need of residual connections in ResNet using mathematical representation of identity mapping. | CO4 | E | 10 |
|  | b. | Compute the output feature map size of a convolution layer for the following parameters: - Input size = 32 × 32 - Kernel size = 5 × 5 - Stride = 2 - Padding = 1 | CO4 | A | 10 |
|  |  | (OR) |  |  |  |
| 8. |  | Examine the layer-wise training mechanism of a Deep Belief Network using Restricted Boltzmann Machines with energy-based formulation. | CO5 | A | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Analyze the effectiveness of deep learning models (CNN) for satellite image classification compared to traditional ML techniques. | CO6 | An | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Outline the basic principles of machine learning techniques |
| CO2 | Understand the technical operation of artificial neural networks |
| CO3 | Describe the various neural network architectures and algorithms |
| CO4 | Understand the basic concepts of deep convolutional methods |
| CO5 | Analyze various deep learning architectures and algorithms |
| CO6 | Explore the various engineering applications using ML techniques |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 22EC3013 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # WIRELESS SENSOR NETWORKS AND IOT | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Evaluate the performance of Bluetooth, ZigBee, and Wi-Fi in terms of power consumption, latency, and scalability for IoT deployments. | CO1 | An | 10 |
|  | b. | Compare MiWi, nRF24, and Wireless LAN technologies, and infer their applicability in short-range communication systems. | CO1 | An | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Design an IoT solution using industry-standard boards and assess its scalability and cost-effectiveness. | CO2 | C | 10 |
|  | b. | Design a multi-node IoT network using heterogeneous boards and evaluate interoperability challenges. | CO2 | C | 10 |
| 3. | a. | Distinguish between WSN and Adhoc networks and infer their suitability for dynamic environments. | CO3 | E | 10 |
|  | b. | Evaluate different modulation schemes for IoT applications and justify their selection under varying conditions. | CO3 | E | 10 |
|  |  | (OR) |  |  |  |
| 4. |  | Examine the fundamentals of MAC protocols, construct a communication strategy for a dense Wireless Sensor Network environment, and distinguish between S-MAC and B-MAC protocols. | CO4 | A | 20 |
| 5. |  | Explain data dissemination and data gathering techniques in Wireless Sensor Networks and compare flooding and gossiping methods across different sensor network scenarios. | CO5 | A | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Analyze the architectural differences between SoC boards and Single Board Computers and examine their performance in IoT applications with respect to processing capability, power consumption, and scalability. | CO2 | An | 20 |
| 7. | a. | Examine the Traffic-Adaptive Medium Access (TRAMA) protocol and assess its effectiveness in dynamic traffic conditions. | CO4 | A | 10 |
|  | b. | Illustrate the IEEE 802.15.4 MAC protocol and explain its significance in IoT applications. | CO4 | A | 10 |
|  |  | (OR) |  |  |  |
| 8. |  | Explain the architectures of circuit switching and packet switching techniques, and evaluate their performance in Wireless Sensor Networks in terms of energy efficiency, latency, scalability, and reliability. | CO6 | A | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Illustrate the architecture of 6LoWPAN-based Wireless Sensor Networks and discuss its applications. | CO6 | A | 10 |
|  | b. | Develop the architecture of the AllJoyn network and explain its role in enabling M2M communication. | CO6 | A | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Interpret the behavior of mobile ad hoc networks and correlate with the infrastructure based networks. |
| CO2 | Infer and develop hardware-based set-up for the required IoT applications. |
| CO3 | Interpret the routing protocols and their implications on data transmission delay and bandwidth. |
| CO4 | Design networks with an attempt to reduce issue of broadcast and flooding techniques. |
| CO5 | Develop algorithms to improve existing or new wireless sensor network applications. |
| CO6 | Identify different protocol, design requirements, algorithms to meet the industrial requirement. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC1004 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # C++ AND DATA STRUCTURES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define the concept of objects in C++. |  | CO1 | R | 1 |
| 2. | List two String functions used in Object-Oriented Programming. |  | CO1 | R | 1 |
| 3. | Describe unary operator overloading in C++. |  | CO2 | U | 1 |
| 4. | Describe file pointer in C++. |  | CO2 | U | 1 |
| 5. | Define polymorphism. |  | CO3 | R | 1 |
| 6. | List types of constructors. |  | CO3 | R | 1 |
| 7. | Describe function template. |  | CO4 | U | 1 |
| 8. | Explain file modes. |  | CO4 | U | 1 |
| 9. | List linear data structures. |  | CO5 | R | 1 |
| 10. | Name a searching technique. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare structure and class. |  | CO1 | An | 3 |
| 12. | Write program for addition using functions. |  | CO2 | A | 3 |
| 13. | Explain the concept of destructor in C++ with syntax and example. |  | CO3 | U | 3 |
| 14. | Explain the working of seekp() in file handling with a suitable program. |  | CO4 | A | 3 |
| 15. | Compare array and linked list. |  | CO5 | An | 3 |
| 16. | Explain selection sort. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Evaluate the combination of arithmetic and relational operators to process patient heart rate data. Consider the following: - Use arithmetic operators to convert raw pulse sensor values (0–200) into beats per minute (BPM). - Use relational operators to check if the BPM is within the normal range of 60 to 100. If the value falls outside the normal range, display an alert message. | CO1 | E | 12 |
| 18. | a. | Develop a C++ program to overload the ‘+’ operator for adding two complex numbers. | CO2 | A | 6 |
|  | b. | Explain in detail about the different types of inheritance with necessary syntax. | CO2 | A | 6 |
| 19. |  | Explain friend functions and friend classes in C++ with suitable examples. Also discuss the concept of static functions and their uses. | CO3 | A | 12 |
| 20. | a. | Construct a C++ program to demonstrate the use of a recursive function. | CO4 | A | 8 |
|  | b. | Explain the concept of function templates in C++ with suitable examples. | CO4 | A | 4 |
| 21. |  | Construct a C++ program to create a singly linked list and display the elements {5, 15, 25}. | CO5 | A | 12 |
| 22. | a. | Analyze the key steps involved in the heap sort algorithm. | CO6 | An | 4 |
|  | b. | Analyze and illustrate the step-by-step execution of insertion sort for the list: 7, 1, 4, 5, 8, 6, 9. | CO6 | An | 8 |
| 23. |  | Construct a C++ program to demonstrate multilevel inheritance using a Student → Marks → Result hierarchy, where student details are inherited to calculate and display the result. | CO3 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the working of the bubble sort algorithm. | CO6 | A | 8 |
|  | b. | Construct a program to search an item in the list. | CO6 | A | 4 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Solve complex mathematical problems using OOP. |
| CO2 | Develop object initialization methods using constructors. |
| CO3 | Develop object-oriented programs utilizing encapsulation and modularity features. |
| CO4 | Develop inheritance models to solve intricate object- oriented design challenges in C++. |
| CO5 | Evaluate exception handling mechanisms to build fault- tolerant and reliable applications. |
| CO6 | Develop OOP programs by incorporating data structures. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC2002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # COMPUTER VISION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | A small drone used for agricultural monitoring captures low-resolution images of crops to quickly detect unhealthy areas. One such image has a resolution of 25 × 25 pixels. Determine the total number of pixels present in the captured image. |  | CO1 | A | 1 |
| 2. | List GLCM feature for texture analysis. |  | CO1 | R | 1 |
| 3. | Identify the following set of neighbor pixel coordinates to a pixel p at coordinate (x, y). (x+1, y+1), (x+1, y-1), (x-1, y+1), (x-1, y-1) |  | CO2 | R | 1 |
| 4. | An image has a resolution of 1500 × 1500 pixels and was scanned at 300 dpi. Calculate the physical dimensions of the image in inches. |  | CO2 | A | 1 |
| 5. | Write the Circle Hough Transform for detecting circles in a digital image. |  | CO3 | A | 1 |
| 6. | Define parameter space. |  | CO3 | R | 1 |
| 7. | Two cameras are placed 10 cm apart. The focal length is 5 cm. A point in the scene has a disparity of 2 mm. Calculate the depth. |  | CO4 | A | 1 |
| 8. | Calculate the Cartesian equation of a line corresponding to an accumulator peak at ρ=2, and θ=90o using Hough Transform. |  | CO4 | A | 1 |
| 9. | Identify a challenge in motion analysis. |  | CO5 | R | 1 |
| 10. | List the applications of computer vison. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | In a robotic path planning system, two grid locations are identified at pixel coordinates p(3,5) and s(2,3) on a warehouse floor map. Apply appropriate distance measures to find the Manhattan distance and Euclidean distance between these two pixels. |  | CO1 | A | 3 |
| 12. | Calculate the chain codes for the given boundary. |  | CO2 | A | 3 |
| 13. | In a 3D computer graphics scenario, consider an object represented by a point P = (2, 2, 1). Translate the object 3 units along the positive X-axis and Y axis in 3D space. Calculate the new coordinates of point P after the translation. |  | CO3 | A | 3 |
| 14. | A 3D surface reconstruction system is used to model surfaces based on gradient information. At a particular region, the system estimates the surface gradients as: p=4 (slope along the x-direction), q=5 (slope along the y-direction).Construct the surface equation z(x,y) from the given gradients. |  | CO4 | A | 3 |
| 15. | A drone monitoring system uses two cameras placed at different locations to track a flying drone in 3D space. Camera 1 is located at position (1,0,0). Camera 2 is located at position (5,0,0). Both cameras observe the same drone. From Camera 1, the drone appears along the direction (1,2,2) From Camera 2, the drone appears along the direction (−2,2,2) Determine the 3D position of the flying drone. |  | CO5 | A | 3 |
| 16. | List the importance of computer vision techniques in smart surveillance systems. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Apply the Harris Interest Point Operator to detect stable feature points in a real-time object tracking system under varying illumination conditions. | CO1 | A | 8 |
|  | b. | Describe any two statistical features derived from GLCM and their significance in analyzing image texture. | CO1 | R | 4 |
| 18. |  | A satellite imaging system captures a grayscale elevation map of a small region represented by the matrix A. Engineers use a structuring element B to perform grayscale dilation and erosion. Apply grayscale dilation and erosion on the given image A using the structuring element 𝐵 and show the resulting images. A=4822623373664423 B=121221111 | CO2 | A | 12 |
| 19. | a. | Apply the RANSAC algorithm to detect a straight line from a set of 2D edge points extracted from a noisy road image containing outliers | CO3 | A | 4 |
|  | b. | Apply the Hough Transform to verify whether the image points (0,1), (1,2), and (2,3) lie on a single straight line in a digital image. | CO3 | A | 8 |
| 20. |  | Apply the concept of photometric stereo to estimate surface normal from multiple images with varying lighting, and interpret the surface shape reconstruction. | CO4 | A | 12 |
| 21. |  | In an autonomous driving system, two cameras mounted at different positions capture images of a moving object from different viewpoints. The system uses 3D motion analysis and triangulation to estimate the position and trajectory of the object in space. Interpret triangulation and its use in determining the 3D position of a point from two camera views. | CO5 | A | 12 |
| 22. |  | In an image enhancement system, both spatial domain filtering and frequency domain-filtering techniques are used to improve image quality and remove noise. Interpret spatial filtering and frequency domain filtering in terms of implementation, efficiency, and applications. | CO1 | A | 12 |
| 23. |  | Interpret the role of boundary extraction in identifying object edges in a binary image. | CO2 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | An intelligent in-vehicle vision system is designed to assist drivers by analyzing road scenes in real time. The system detects roadways, identifies lane markings, recognizes traffic signs, and locates pedestrians to improve safety and navigation. Interpret the role of computer vision techniques in detecting roadways and lane markings. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Implement fundamental image processing techniques required for computer vision. |
| CO2 | Perform shape analysis and to implement boundary tracking techniques. |
| CO3 | Apply chain codes and other region descriptors. |
| CO4 | Apply Hough Transform for line, circle, and ellipse detections. |
| CO5 | Apply 3D vision techniques and implement motion related techniques. |
| CO6 | Develop applications using computer vision techniques. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC2004 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SOFTWARE ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Distinguish the reason of software deterioration over time. |  | CO1 | U | 1 |
| 2. | List two AI-based software. |  | CO1 | R | 1 |
| 3. | Name the IEEE standard used for Software Requirements Specification. |  | CO2 | R | 1 |
| 4. | Describe functional requirements in software development. |  | CO2 | R | 1 |
| 5. | Explain data flow diagram. |  | CO3 | U | 1 |
| 6. | List two software quality guidelines. |  | CO3 | R | 1 |
| 7. | Explain the need for reducing user memory load in interface design. |  | CO4 | U | 1 |
| 8. | Describe interface construction. |  | CO4 | U | 1 |
| 9. | Explain validation testing. |  | CO5 | U | 1 |
| 10. | List the elements of the management spectrum. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare system software and application software. |  | CO1 | A | 3 |
| 12. | Explain requirement engineering. |  | CO2 | U | 3 |
| 13. | Evaluate the role of information hiding in software design concepts. |  | CO3 | An | 3 |
| 14. | Describe the term breadcrumbs used in WebApp navigation. |  | CO4 | U | 3 |
| 15. | Deduce the purpose of software validation testing. |  | CO5 | An | 3 |
| 16. | Infer the need for software measurement. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Evaluate the need for prescriptive process model for software development with necessary diagrams. | CO1 | An | 12 |
| 18. | a. | Examine the different requirements engineering process stages | CO2 | A | 6 |
|  | b. | Explain the different types of requirement engineering. | CO2 | U | 6 |
| 19. | a. | Explain architectural context diagrams for software design. | CO3 | A | 6 |
|  | b. | Compare data centered and data flow architectures. | CO3 | An | 6 |
| 20. |  | Examine in detail the four activities of the user interface design process. | CO4 | A | 12 |
| 21. | a. | Discuss validation testing and its types. | CO5 | U | 6 |
|  | b. | Explain different strategic testing approaches with examples. | CO5 | A | 6 |
| 22. |  | Analyze the Agile software development process using the Scrum methodology. | CO1 | An | 12 |
| 23. | a. | Demonstrate the role of the Software Requirement Specification (SRS) in defining and managing the requirements of a software project. | CO2 | A | 6 |
|  | b. | Explain the importance of debugging and describe the debugging process. | CO5 | U | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Apply the elements of the management spectrum in a software project and explain their roles with examples. | CO6 | A | 6 |
|  | b. | Analyze the process of risk management and explain each stage with suitable examples. | CO6 | An | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the software engineering and agile development processes. |
| CO2 | Formulate requirements and validate them. |
| CO3 | Analyze different software design methodologies. |
| CO4 | Determine suitable tests for testing and debugging a software. |
| CO5 | Apply appropriate methods to manage and maintain a software. |
| CO6 | Ensure that the software meets the required standards. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC2006 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ARTIFICIAL NEURAL NETWORKS AND DEEP LEARNING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List any two applications of supervised learning methodology. |  | CO1 | R | 1 |
| 2. | Differentiate weights from layers in artificial neural networks. |  | CO1 | U | 1 |
| 3. | Distinguish delta rule from winner take-all rule in ANN. |  | CO2 | U | 1 |
| 4. | Infer the drawbacks of back propagation neural network. |  | CO2 | U | 1 |
| 5. | List the parameters used in deep neural networks. |  | CO3 | R | 1 |
| 6. | Illustrate the L1 regularization method used in deep neural networks. |  | CO3 | U | 1 |
| 7. | Interpret the concept of filters in convolution layer of CNN. |  | CO4 | U | 1 |
| 8. | Interpret ResNet and VGG-16. |  | CO4 | U | 1 |
| 9. | List the deep learning models used for signal processing applications. |  | CO5 | R | 1 |
| 10. | Distinguish between feature extraction and feature selection |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare classification with clustering. |  | CO1 | An | 3 |
| 12. | “Perceptron suffers from linear separable problem”. Justify this statement. |  | CO2 | E | 3 |
| 13. | Illustrate the operation of sigmoidal function with a graphical sketch. |  | CO3 | U | 3 |
| 14. | “Convolution is used for feature extraction in CNN”. Justify this statement. |  | CO4 | E | 3 |
| 15. | “The number of neurons in the input layer of auto encoder must be same as the neurons in the output layer”. Justify this statement. |  | CO5 | E | 3 |
| 16. | Illustrate the process of speaker identification using deep neural network with a block diagram. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Compare supervised with unsupervised learning methodologies. | CO1 | An | 8 |
|  | b. | Determine the weights of perceptron neural network after 2 iterations with following specifications: Inputs: [2 1], Initial weights: [0.3 0.5], learning rate:0.5, Target=1 | CO1 | E | 4 |
| 18. |  | Illustrate the forward pass and reverse pass of back propagation neural network with architecture and training algorithm. | CO2 | U | 12 |
| 19. |  | Illustrate the characteristic features of optimization methods used in deep neural networks in terms of speed and accuracy. | CO3 | U | 12 |
| 20. |  | “LSTM networks are widely preferred for continuous data applications”. Justify this statement with neat architecture and diagrams. | CO4 | E | 12 |
| 21. | a. | Compare Generative Adversarial Networks with Deep Belief Networks. | CO5 | An | 6 |
|  | b. | “Autoencoders are widely preferred for dimensionality reduction application”. Justify this statement with neat architecture. | CO5 | E | 6 |
| 22. | a. | Compare Hopfield neural network with Perceptron neural network. | CO5 | An | 6 |
|  | b. | “Kohonen neural networks employ unsupervised training methodology for the training process”. Justify this statement. | CO5 | E | 6 |
| 23. |  | Develop a deep neural network based automated system for retinal disease identification with 3 output classes and 8 input features. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Illustrate the functions of different layers of CNN with neat architecture and mathematical equations. | CO5 | U | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Interpret the pros and cons of supervised and unsupervised machine learning algorithms. |
| CO2 | Evaluate the performance of supervised and unsupervised networks for classification applications. |
| CO3 | Interpret the effect of hyperparameters on deep neural networks. |
| CO4 | Evaluate the performance of CNN for classification applications. |
| CO5 | Evaluate the performance of deep neural networks for regression and classification applications. |
| CO6 | Solve the problems in pattern recognition applications using deep learning models. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC2007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # VLSI DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the operating region of MOS when VGS<Vth​. |  | CO1 | U | 1 |
| 2. | State the expression for dynamic power dissipation. |  | CO1 | R | 1 |
| 3. | List 2 advantages of CMOS logic over Ratioed logic. |  | CO2 | U | 1 |
| 4. | State the drawback of pass transistor logic. |  | CO2 | U | 1 |
| 5. | Differentiate latch from Flip-flop. |  | CO3 | An | 1 |
| 6. | Sketch 2:1 MUX based negative latch. |  | CO3 | A | 1 |
| 7. | List the advantage of carry look-ahead adder. |  | CO4 | U | 1 |
| 8. | Mention the advantage of Booth multiplier over binary multiplier. |  | CO4 | U | 1 |
| 9. | Define volatile memory. |  | CO5 | R | 1 |
| 10. | List any 2 steps in CMOS fabrication process. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Illustrate MOS transistor operation in different regions. |  | CO1 | An | 3 |
| 12. | Justify the statement “pMOS FET always sends strong 1 but weak 0”. |  | CO2 | E | 3 |
| 13. | Design a MUX based negative latch with neat timing diagram. |  | CO3 | An | 3 |
| 14. | Describe carry propagate adder with diagram. |  | CO4 | U | 3 |
| 15. | Illustrate DRAM cell operation. |  | CO5 | An | 3 |
| 16. | Explain CMOS fabrication steps. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Illustrate CMOS inverter characteristics with VTC and its different regions. | CO1 | U | 12 |
| 18. | a. | Design a 2-input CMOS NAND gate and explain its operation. | CO2 | A | 6 |
|  | b. | Compare static CMOS and dynamic logic styles. | CO2 | U | 6 |
| 19. |  | Explain the design of edge-triggered register using master-slave configuration. | CO3 | An | 12 |
| 20. |  | Design carry look-ahead adder and derive Boolean expressions. | CO4 | A | 12 |
| 21. |  | Design a Mask Programmable NOR ROM to implement the given function table. R1; R2; R3; R4; C1; C2; C3; C4. 1; 0; 0; 0; 0; 1; 0; 1. 0; 1; 0; 0; 0; 0; 1; 1. 0; 0; 1; 0; 1; 0; 0; 1. 0; 0; 0; 1; 0; 1; 1; 0. | CO5 | A | 12 |
| 22. |  | Differentiate domino logic from np CMOS logic with neat diagram. | CO2 | An | 12 |
| 23. |  | Design CMOS full adder with schematic and transistor count and explain its operation. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Interpret the stick diagram for a CMOS inverter with proper labeling and explanation. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the MOS transistor structure and its characteristics. |
| CO2 | Design CMOS combinational circuits using various logic styles. |
| CO3 | Design sequential circuits using static and dynamic styles. |
| CO4 | Design subsystems like adders, multipliers and shifters. |
| CO5 | Analyze and design different types of memories. |
| CO6 | Understand layout design rules and fabrication techniques. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC2009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # PRINCIPLES AND DESIGN OF IoT SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List the key characteristic that differentiates embedded systems from general-purpose computers. |  | CO1 | R | 1 |
| 2. | Compare hard real-time systems with soft real-time systems. |  | CO1 | R | 1 |
| 3. | Identify the type of cache memory organization that separates instruction and data caches in embedded systems. |  | CO2 | R | 1 |
| 4. | A computer system has a cache memory with the following access pattern: Total memory accesses = 200 Number of cache hits = 150 Compute the cache hit ratio. |  | CO2 | A | 1 |
| 5. | Identify the basic storage mechanism that allows flash memory to retain data even when power is switched OFF. |  | CO2 | U | 1 |
| 6. | Name the most relevant RTOS scheduling concept used in airbag deployment system. |  | CO3 | R | 1 |
| 7. | Identify any two IoT messaging protocols used for communication. |  | CO4 | U | 1 |
| 8. | Name the type of programming language that is hardware-dependent in embedded systems. |  | CO5 | R | 1 |
| 9. | State any one cloud service model used in cloud computing. |  | CO5 | R | 1 |
| 10. | State one benefit of real-time data monitoring in IoT applications. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate the types of embedded systems in the following scenarios: a) An airbag control system that must deploy within milliseconds during a collision. b) An online video streaming application where occasional delays are acceptable. c) An industrial robotic arm used for precision assembly where delayed response reduces product quality but does not cause system failure. |  | CO1 | An | 3 |
| 12. | Illustrate the role of DMA in embedded systems. |  | CO2 | A | 3 |
| 13. | Interpret the roles of compiler, linker, and debugger in an Integrated Development Environment. |  | CO3 | An | 3 |
| 14. | Examine IoT communication patterns and justify the selection of a suitable messaging protocol for a smart home application. |  | CO4 | A | 3 |
| 15. | Compare assembly language with high-level languages with respect to their features, advantages, and limitations. |  | CO5 | An | 3 |
| 16. | Explain the sequence of operations involved in reading data from flash memory. |  | CO2 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Illustrate the characteristics and classification of embedded systems. | CO1 | A | 6 |
|  | b. | Describe the product design life cycle of an embedded system. | CO1 | U | 6 |
| 18. |  | Use the concept of cache memory to analyze how system performance can be improved in an embedded system. | CO2 | A | 12 |
| 19. | a. | Interpret the role of an RTOS in OS-based embedded system design. | CO3 | A | 6 |
|  | b. | A smart home embedded system performs multiple tasks such as temperature monitoring, motion detection, and sending alerts to a mobile application. These tasks must run simultaneously without affecting system performance. Explain how multithreading in an RTOS can be used to handle this scenario effectively. | CO3 | An | 6 |
| 20. |  | Illustrate the architecture of Internet of Things (IoT) and relate its layered structure with reference to the OSI model. | CO4 | A | 12 |
| 21. |  | A smart campus solution is developed to streamline attendance and monitoring processes. Students gain access using RFID/NFC cards, with data processed by a microcontroller. For transmitting information to a central server, long-range communication is utilized. The implementation incorporates Arduino IDE with ESP8266 and Thonny IDE with Raspberry Pi, supported by LoRa communication technology. Devise a complete implementation plan for this system, including hardware, software, communication, data flow, and any real-time monitoring features.. | CO4 | An | 12 |
| 22. |  | Construct an embedded C/Arduino program using Arduino IDE for the ESP8266 to blink an LED connected to a GPIO pin. In your program: - Initialize the GPIO pin - Implement delay using appropriate functions Explain how the program differs from an equivalent Assembly language implementation in terms of readability and execution. | CO5 | A | 12 |
| 23. |  | Describe the architecture of ESP8266 and its interfacing with GPS in an embedded system. | CO5 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Examine the architecture, data flow, functionalities, and the role of real-time data monitoring using cloud platforms in IoT-based Electric Vehicle systems. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate a comprehensive understanding of Embedded System Architecture. |
| CO2 | Analyze and apply knowledge of memory systems in Embedded Systems. |
| CO3 | Utilize IDE tools effectively and gain insight into (RTOS) for embedded systems. |
| CO4 | Understand the basics of IoT, Network Topologies and IoT communication protocols. |
| CO5 | Develop proficiency in IoT programming techniques and gain familiarity with Cloud Computing technologies for IoT applications. |
| CO6 | Apply IoT concepts and methodologies to solve real-world problems in various domains, demonstrating the ability to integrate IoT solutions into real-time applications effectively. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC2012 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # NETWORK THEORY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Calculate the current flow across the terminal ‘ab’. |  | CO1 | A | 1 |
| 2. | Determine the voltage V in the given circuit. |  | CO1 | A | 1 |
| 3. | Predict the condition for maximum power transfer from a source to the load. |  | CO2 | A | 1 |
| 4. | Identify the duality of Thevenin’s theorem. |  | CO2 | U | 1 |
| 5. | Write the formula for reactive power in a 3-phase star connected system. |  | CO3 | A | 1 |
| 6. | Write the formula for Time constant of a series RC circuit. |  | CO3 | A | 1 |
| 7. | Apply the Laplace Transform to e-6t and predict its result. |  | CO4 | A | 1 |
| 8. | Define Resonance frequency. |  | CO5 | R | 1 |
| 9. | Analyze the impact of bandwidth on the performance of communication systems. |  | CO5 | An | 1 |
| 10. | Choose a filter that allows all frequencies up to the cutoff frequency fc to pass without attenuation while attenuating frequencies above fc. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Calculate the current through 3 ohm resistor. |  | CO1 | A | 3 |
| 12. | Construct the dual network for the following circuit. |  | CO2 | C | 3 |
| 13. | Evaluate the power factor for a balanced mesh load of 20∠40⁰ is connected across a 400V, 3 – Ø balanced supply. |  | CO3 | E | 3 |
| 14. | Write the expressions for the following properties of the Laplace Transform: (i) Linearity (ii) Time Differentiation (iii) Time Integration |  | CO4 | A | 3 |
| 15. | Identify the critical frequencies for the network function V(s) = 4(s+9)/(s+2) (s+5) |  | CO5 | U | 3 |
| 16. | Determine the open circuit input impedance Z22 for the given circuit. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Compute the nodal voltage using nodal analysis. | CO1 | A | 12 |
| 18. |  | Apply mesh analysis method to determine the mesh currents in the circuit shown. | CO1 | A | 12 |
| 19. |  | Apply Norton’s theorem to determine the Norton’s equivalent circuit and the current through 4 Ω resistor for the given circuit. | CO2 | A | 12 |
| 20. |  | Determine the current through 3Ω resistor in the circuit shown using superposition theorem. | CO2 | A | 12 |
| 21. |  | Evaluate the line current, phase current, line voltage, power consumed by the load, and power factor for the following system: A three-phase balanced delta-connected load with an impedance of (3 + j4) Ω is supplied by a 440V, 3Ø balanced source with an RYB phase sequence. | CO3 | E | 12 |
| 22. |  | Determine the steady state response of series RL and series RC circuit using Laplace Transform if R= 4 Ω, C= 1/16 F, L= 0.2 H and V= 3u(t). | CO4 | A | 12 |
| 23. |  | Estimate the resonant frequency, Q factor, lower cut off frequency, upper cut off frequency and bandwidth of the following circuit specification. A parallel RLC circuit consists of 40 Ω resistor 0.3 H inductance and 12 μF capacitor with an applied voltage of 30V. | CO5 | E | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Determine the short circuit admittance parameters of the network given below. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design basic electrical circuits with nodal and mesh analysis. |
| CO2 | Apply various electrical network theorems to analyze circuits and networks. |
| CO3 | Evaluate the performance metrics in three phase circuits. |
| CO4 | Apply Laplace Transform for steady state and transient analysis. |
| CO5 | Analyze the frequency domain techniques. |
| CO6 | Determine different network functions and design filter circuits. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC2014 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # CONTROL SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define transfer function. |  | CO1 | R | 1 |
| 2. | Identify the electrical analogous quantity for frictional co-efficient (B) in the force-voltage analogy. |  | CO1 | R | 1 |
| 3. | List any two standard test signals used in control systems. |  | CO2 | R | 1 |
| 4. | Compare the transient response and steady-state response of a system. |  | CO2 | U | 1 |
| 5. | Define stability of a system. |  | CO3 | R | 1 |
| 6. | Identify the number of branches in root locus plot, if the number of poles and zeros of the open loop transfer function are 3 and 2. |  | CO3 | U | 1 |
| 7. | Identify the representation of the circles in a polar graph sheet. |  | CO4 | U | 1 |
| 8. | Predict the coordinates used in a polar plot. |  | CO4 | U | 1 |
| 9. | List the types of compensators in control systems. |  | CO5 | R | 1 |
| 10. | Interpret the concept of controllability in state variables. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate between open-loop and closed-loop systems. |  | CO1 | U | 3 |
| 12. | Define the following time domain specifications. i) Delay time ii) Rise time |  | CO2 | R | 3 |
| 13. | Determine the range of K for the stability of the system whose characteristic equation is S4+S3+11S2+S+K=0. |  | CO3 | A | 3 |
| 14. | Calculate the centroid of the root locus of a system with open loop transfer function KS(S+3)(S+6) |  | CO4 | A | 3 |
| 15. | Develop the transfer function of PID controller. |  | CO5 | A | 3 |
| 16. | List the advantages of state space model. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Determine the transfer function for the block diagram shown below | CO1 | A | 6 |
|  | b. | Determine the overall transfer function of the system whose signal flow graph is shown in figure | CO1 | A | 6 |
| 18. | a. | Evaluate the static error constants Kp, Kv, and Ka for the unity feedback system with the following open-loop transfer function: G(s)= 10S(S+2) | CO2 | An | 6 |
|  | b. | The damping ratio of system is 0.6 and the natural frequency of oscillation is 8 rad/sec. Determine the rise time(tr) and peak time (tp). | CO2 | A | 6 |
| 19. |  | Construct the root locus for the following system whose open loop transfer function is G(s)= KS(S+2)(S+4) | CO3 | A | 12 |
| 20. |  | Sketch the bode plot for the following transfer function and determine the gain margin and phase margin. G(s)= 2S(1+0.5S)(1+0.05S) | CO4 | A | 12 |
| 21. |  | Sketch the block diagrams of P, PD, PI, and PID controllers, and explain their characteristics with transfer functions. | CO5 | A | 12 |
| 22. | a. | Evaluate the stability of the system using the Routh–Hurwitz criterion for the characteristic equation: S4+2s3+3s2+4s+5=0 | CO3 | An | 6 |
|  | b. | Identify the compensator depicted in the following figure and outline its characteristics, advantages and disadvantages. | CO5 | U | 6 |
| 23. |  | Determine phase margin and gain margin by drawing polar plot for the system whose open loop transfer function G(s)= 1S1+S(1+2S) | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain any two properties of state transition matrix φ(t). | CO6 | A | 6 |
|  | b. | Determine the state space model for the system having the transfer function Y(s)U(s)= 1s2+s+1 | CO6 | A | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Develop mathematical model of physical systems. |
| CO2 | Analyze the various linear models in time domain. |
| CO3 | Examine the stability of systems. |
| CO4 | Analyze the various linear models in frequency domain. |
| CO5 | Design appropriate controller for the given specifications. |
| CO6 | Outline the basics of state space representation of systems |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC2017 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SEMICONDUCTOR AND CHIP DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define crystal lattice. |  | CO1 | R | 1 |
| 2. | State the number of atoms per unit cell in a simple cubic lattice. |  | CO1 | R | 1 |
| 3. | Define PN junction. |  | CO2 | R | 1 |
| 4. | Differentiate Zener and avalanche breakdown based on doping concentration. |  | CO2 | U | 1 |
| 5. | Define channel in MOSFET. |  | CO3 | R | 1 |
| 6. | Define load line in transistor operation. |  | CO3 | R | 1 |
| 7. | Define forward blocking state. |  | CO4 | R | 1 |
| 8. | State the concept of forward blocking state in a p–n–p–n device. |  | CO4 | R | 1 |
| 9. | Define FPGA used in high-density memory systems. |  | CO5 | R | 1 |
| 10. | Cite about ‘Hardware Description Language (HDL)’. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Describe intrinsic and extrinsic semiconductor. |  | CO1 | U | 3 |
| 12. | Differentiate Schottky contact from Ohmic contact. |  | CO2 | U | 3 |
| 13. | Interpret the output and transfer characteristics of a MOSFET. |  | CO3 | U | 3 |
| 14. | Differentiate SCR from IGBT based on structure and operation |  | CO4 | U | 3 |
| 15. | Compare adders with multipliers based on their functionality in subsystem design. |  | CO5 | U | 3 |
| 16. | Describe Gate level modeling using Verilog with suitable example. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Illustrate the Hall effect in semiconductors with carrier concentration and mobility. | CO1 | A | 12 |
| 18. |  | Analyze the operation of a PN junction under forward and reverse bias conditions with energy band diagrams. | CO2 | An | 12 |
| 19. | a. | Describe the structure and operation of GaAs MESFET. | CO3 | A | 8 |
|  | b. | Interpret the advantages of GaAs MESFET over silicon MOSFETs in high-frequency applications. | CO3 | A | 4 |
| 20. |  | Describe the V–I characteristics of a tunnel diode and the negative resistance region with suitable diagrams. | CO4 | A | 12 |
| 21. |  | Illustrate the read and write operations of a 6T SRAM cell in data path design. | CO5 | A | 12 |
| 22. |  | Design the structure logic of 4*4 Wallace Tree Multiplier and illustrate its function. Assume A=1011 and B=1110 | CO5 | C | 12 |
| 23. |  | Design a Full Subtractor using gate-level modelling in Verilog with truth table. | CO6 | C | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze Verilog-based HDL techniques for designing data path and controller architectures in gate level modelling. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply the concepts of semiconductor physics in chip design. |
| CO2 | Differentiate various types of junctions and its effect in semiconductors. |
| CO3 | Analyze the characteristics of MOSFET and MISFET. |
| CO4 | Design various diodes used in high frequency and high-power devices. |
| CO5 | Develop subsystems for chip design. |
| CO6 | Simulate electronic circuits using Hardware Description Language. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC2020 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # PRINTED CIRCUIT BOARD DESIGN AND ARDUINO PROGRAMMING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the key Industry 5.0 principle, where human operators and collaborative robots work together in an automobile industry. |  | CO1 | R | 1 |
| 2. | List any two applications of Cyber-Physical Systems (CPS) in smart manufacturing industries. |  | CO1 | R | 1 |
| 3. | Identify the sensor type most suitable for detecting transparent bottles positioned at a 20 cm distance on a conveyor belt. |  | CO2 | R | 1 |
| 4. | Identify the key characteristics of an Infrared (IR) sensor used for object detection in industrial automation. |  | CO2 | R | 1 |
| 5. | List any two intelligent machines used in a warehouse. |  | CO3 | R | 1 |
| 6. | Identify the most suitable intelligent sensor used in smart wearables for tracking body temperature |  | CO3 | R | 1 |
| 7. | List the characteristics of an embedded system. |  | CO4 | R | 1 |
| 8. | Compare embedded systems with general computing systems. |  | CO4 | R | 1 |
| 9. | Define netlisting. |  | CO5 | R | 1 |
| 10. | Infer the importance of stepper motor in real time applications. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Calculate the yield percentage, defective count and value loss for an intelligent packaging line in a smart industry from the given data and brief the sensor accuracy and cost efficiency in the systems. Target = 2,000 items; Sensor count = 1,950; Manual audit = +12 missed, −8 false detections and each product costs ₹50. |  | CO1 | A | 3 |
| 12. | Compare analog sensors with digital sensors. |  | CO2 | An | 3 |
| 13. | A collaborative robot assists a human operator in assembling 200 parts/hour. If automation adds 30% speed improvement and consider an error reduction of 5% due to safety constraints. 1. Calculate the net productivity gain. 2. Analyze its effect on human machine synergy. |  | CO3 | An | 3 |
| 14. | Compare Assembly language with High level language based program development for embedded systems. |  | CO4 | An | 3 |
| 15. | Summarize the importance of PCB Layout design. |  | CO5 | U | 3 |
| 16. | Sketch the block diagram of LED & Switch interfacing with Arduino. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Develop a circuit model of a smart manufacturing line using an Industry 5.0 framework and explain its sustainability and adaptability. | CO1 | C | 12 |
| 18. |  | Develop an ultrasonic object detection system to measure distance from moving objects in a conveyor. Determine the detection accuracy under varying distances and angles. | CO2 | C | 12 |
| 19. | a. | Analyze the structural components and execution flow of the Arduino microcontroller system. | CO3 | An | 8 |
|  | b. | Sketch the block diagram of an intelligent machine. | CO3 | A | 4 |
| 20. | a. | Interpret the different “C” program elements used in embedded software design with examples. | CO4 | A | 6 |
|  | b. | Explain the software/firmware development tool kit used in host and target machines. | CO4 | A | 6 |
| 21. | a. | Classify Printed Circuit Boards and explain the significance of multi-layered PCB. | CO5 | An | 8 |
|  | b. | Classify IC packages. | CO5 | An | 4 |
| 22. | a. | Interpret the role of digitalization in a smart precision quality control system. Explain its contribution to the system accuracy and efficiency. | CO1 | A | 6 |
|  | b. | Evaluate the impact of compelling forces and challenges for Industry 4.0 initiatives in emerging economies. | CO1 | E | 6 |
| 23. |  | Design a temperature control system using Arduino for a furnace maintaining 1200 °C. Interpret the controller response and discuss the operation. | CO2 | C | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Design a cloud-enabled intelligent robotic arm used in an automobile industry and explain its operation. | CO6 | C | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Comprehend the importance of Industry 4.0 and its challenges. |
| CO2 | Interpret the various sensors for embedded systems. |
| CO3 | Understand the specifications of Arduino and its design flow. |
| CO4 | Evaluate embedded systems using embedded C programming. |
| CO5 | Apply the knowledge of electronic component packages, types and history of PCBs to design prototypes. |
| CO6 | Develop and program Arduino controllers with different peripheral devices for embedded applications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EC3003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # HARDWARE SECURITY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Apply the architecture and hardware implementation concepts of the Advanced Encryption Standard (AES) in digital systems. | CO1 | A | 10 |
|  | b. | Illustrate the significance of the RSA Algorithm in cryptography through its practical applications. | CO1 | A | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Illustrate how different PUF architectures can be implemented in hardware authentication systems using Physical Unclonable Function (PUF). | CO2 | A | 10 |
|  | b. | Analyze the implementation of different Physical Unclonable Function (PUF) architectures and their effectiveness in hardware authentication systems. | CO2 | An | 10 |
| 3. | a. | Estimate techniques for resilience against Side-Channel Attacks (SCA) in digital systems using advanced hardware and algorithmic countermeasures. | CO3 | E | 10 |
|  | b. | Examine Kocher’s Timing Attack in the context of the Data Encryption Standard (DES) and its impact on cryptographic hardware. | CO3 | A | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Estimate the effectiveness of design techniques used to prevent IP and IC piracy, focusing on hardware obfuscation, watermarking, and split manufacturing. | CO4 | An | 10 |
|  | b. | Analyze design techniques used to prevent IP and IC piracy, focusing on strategies that enhance security and prevent unauthorized duplication and tampering. | CO4 | An | 10 |
| 5. | a. | Estimate the effectiveness of Hardware Trojans detection using logic testing and side-channel analysis, and assess challenges in distinguishing Trojan-induced anomalies from process variations and environmental noise. | CO5 | An | 10 |
|  | b. | Explain the classification of Hardware Trojans based on structure, activation mechanism, and payload, and list the operating modes that enable stealthy operation in modern VLSI systems. | CO5 | A | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Explain the Data Encryption Techniques, focusing on methods used to secure data during storage and transmission. | CO1 | A | 10 |
|  | b. | Estimate techniques to protect a cryptographic devices against advanced fault injection attacks in VLSI systems | CO3 | A | 10 |
| 7. | a. | Analyze the role of Physical Unclonable Functions (PUFs) in hardware security and differentiate the design challenges in implementing PUF-based authentication and encryption. | CO4 | An | 10 |
|  | b. | Analyze the impact of different Physical Unclonable Functions (PUFs) design techniques on reliability, uniqueness, and randomness of PUF responses | CO4 | An | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Explain Cache-based attacks, focusing on how modern processors’ cache systems can be exploited to leak sensitive information. | CO6 | A | 10 |
|  | b. | Develop security mechanisms to prevent Spectre and Meltdown attacks data leakage caused by speculative execution and side-channel exploits | CO6 | C | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Evaluate attacks against branch prediction attacks that exploit speculative execution vulnerabilities, and estimate countermeasures to mitigate risks such as Spectre and Branch Target Injection (BTI). | CO6 | E | 10 |
|  | b. | Develop a framework to smart-grid security systems that mitigates threats and enhances reliability, confidentiality, and integrity in smart-grid infrastructures. | CO6 | C | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Interpret and optimize the process of implementing cryptographic algorithms on hardware. |
| CO2 | Classify various Hardware Security Primitives. |
| CO3 | Determine the different kinds of attacks that can be mounted against cryptographic algorithms and Physical Unclonable Functions and make them resilient to attacks. |
| CO4 | Design Modern IC and Manufacturing Practices and Their Implications. |
| CO5 | Analyze the different kinds of Trojans, their impact and learn the effective counter measures for defending against them. |
| CO6 | Infer the different kinds of threats at the micro architectural level and their corresponding Counter measures. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC2003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ANALOG CIRCUITS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Identify the circuit used in the final stage of a DC power supply to maintain a constant DC output. |  | CO1 | U | 1 |
| 2. | Define a clipper circuit. |  | CO1 | R | 1 |
| 3. | A transistor amplifier in a communication circuit shows variation in operating point due to changes in transistor gain (β). Identify a suitable biasing method to improve its stability. |  | CO2 | U | 1 |
| 4. | Calculate the stability factor for a fixed-bias BJT circuit with a current gain (β) of 50. |  | CO2 | A | 1 |
| 5. | A three-stage amplifier has individual voltage gains of 10, 20, and 5. Calculate the overall voltage gain of the multistage amplifier. |  | CO3 | A | 1 |
| 6. | A multistage amplifier used in an audio system requires proper signal transfer between stages without distortion or loss of information. List the coupling components used between different stages in the amplifier. |  | CO3 | R | 1 |
| 7. | Interpret the design features of a power transistor that aid in heat dissipation. |  | CO4 | A | 1 |
| 8. | Define the operating cycle of a Class C power amplifier. |  | CO4 | R | 1 |
| 9. | A voltage-shunt feedback amplifier has an open-loop gain Av=100​ and a feedback factor β=0.05.Calculate the closed-loop voltage gain of the amplifier. |  | CO5 | A | 1 |
| 10. | Define Barkhausen criterion. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Construct an inductor filter circuit to remove ripple from the output of a rectifier and explain its operation to obtain a smooth DC output. |  | CO1 | A | 3 |
| 12. | Determine the collector current IC flowing in the given fixed bias network. [Assume VBE = 0.7 V] |  | CO2 | An | 3 |
| 13. | Sketch the hybrid equivalent circuit of a common-emitter BJT amplifier. |  | CO3 | A | 3 |
| 14. | Justify the occurrence of crossover distortion with a suitable method for its elimination. |  | CO4 | E | 3 |
| 15. | In a communication system requiring stable gain and low distortion, justify the use of a negative feedback amplifier by stating its advantages. |  | CO5 | E | 3 |
| 16. | A single tuned amplifier uses a tank circuit with inductance L=100 μH and capacitance C=100 pF. Calculate the resonant frequency of the amplifier. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | A company, Powertech Electronics, is developing a low-cost DC power supply for simple electronic devices using a rectifier circuit that utilizes only one-half cycle of the input signal. Design a suitable circuit for this requirement, explaining its operation, deriving the rectifier efficiency, and discussing its performance. | CO1 | A | 12 |
| 18. | a | In the DC analysis of a BJT with voltage divider biasing, calculate the Thevenin equivalent voltage and resistance for the input loop. | CO2 | A | 6 |
|  | b. | Explain the procedure for establishing the biasing point in a fixed-bias JFET circuit. | CO2 | U | 6 |
| 19. |  | A multistage amplifier is used in an audio system to amplify weak signals to a higher level. Explain its function in the circuit. | CO3 | A | 12 |
| 20. |  | A studio audio system demands low distortion despite high power loss. Explain the principle of operation of a Class A power amplifier and assess its efficiency and power dissipation. | CO4 | A | 12 |
| 21. | a. | Justify the use of a differential amplifier in reducing common-mode signals. | CO5 | E | 6 |
|  | b. | Describe the voltage-series feedback configuration, analyzing its effect on circuit performance. | CO5 | U | 6 |
| 22. | a. | A full-wave rectifier is supplied with an AC input of 20 V (rms) and is connected to a load resistance of 1 kΩ. Calculate the DC output voltage, DC output current and rectifier efficiency. | CO1 | A | 6 |
|  | b. | Illustrate the transistor connections in a cascode amplifier. | CO3 | A | 6 |
| 23. |  | Sketch the Single Tuned Amplifier circuit and derive the expression for its resonant frequency. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | "A company is developing a low-cost audio signal generator for testing speakers and amplifiers. The design must operate efficiently at low frequencies and avoid the use of inductors to reduce size and cost". Evaluate the performance of the designed oscillator for the given application. | CO6 | E | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Evaluate the application of diodes. |
| CO2 | Interpret and classify the characteristics of BJT and JFET amplifiers. |
| CO3 | Design single stage and multistage amplifier circuits. |
| CO4 | Analyse the function of power amplifier. |
| CO5 | Construct differential amplifier for a given specification. |
| CO6 | Demonstrate sinusoidal and non-sinusoidal oscillators. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC2005 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # LINEAR INTEGRATED CIRCUITS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List any two ideal characteristics of an Op-Amp. |  | CO1 | R | 1 |
| 2. | Determine the input offset current, if IB+ = 10 mA and IB- = 8 mA. |  | CO1 | A | 1 |
| 3. | In a battery monitoring system, a non-inverting comparator has a reference voltage Vref =4V. If the non-inverting terminal input voltage is 6V, predict the output of the comparator. |  | CO2 | A | 1 |
| 4. | A differentiator receives a square wave input. Identify the output waveform. |  | CO2 | U | 1 |
| 5. | State the Barkhausen criterion for sustained oscillation. |  | CO3 | R | 1 |
| 6. | Indicate the output voltage of a 7915 IC regulator. |  | CO3 | U | 1 |
| 7. | Sketch the frequency response of a wide-band-pass filter. |  | CO4 | A | 1 |
| 8. | Identify the filter used to eliminate frequencies around 50 Hz while allowing other frequencies to pass. |  | CO4 | U | 1 |
| 9. | List the modes of the 555 timer. |  | CO5 | R | 1 |
| 10. | Indicate the use of switched capacitor circuits. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | In a sensor interface circuit, an inverting amplifier has Rin =10kΩ, Rf =100kΩ, and an input of 0.2 V. Calculate the output voltage and gain. |  | CO1 | A | 3 |
| 12. | An Op-Amp inverting adder has three inputs: 1 V, 2 V, and 3 V with equal resistors. Calculate the output voltage. |  | CO2 | A | 3 |
| 13. | An RC phase shift oscillator uses three identical RC sections with R=10 kΩ and =0.01 μF. Determine the frequency of oscillation. |  | CO3 | A | 3 |
| 14. | Sketch the op-amp-based circuit diagram of a wide-bandstop filter. |  | CO4 | A | 3 |
| 15. | Summarize the function of the Phase Locked Loop (PLL). |  | CO5 | U | 3 |
| 16. | A 4-bit weighted resistor DAC has a reference voltage of 10 V. The digital input is 1101. Determine the analog output voltage. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Construct an Op-Amp-based non-inverting amplifier circuit to amplify a low-level sensor signal and justify your design. | CO1 | A | 8 |
|  | b. | Analyze the performance of an Op-Amp considering DC characteristics in a real-time signal processing system. | CO1 | An | 4 |
| 18. | a. | Construct an Op-Amp-based waveform shaping circuit using an integrator for signal processing applications. | CO2 | A | 8 |
|  | b. | Explain the working of an Op-Amp-based full-wave rectifier with a waveform. | CO2 | U | 4 |
| 19. |  | Develop an Op-Amp-based square wave generator (astable multivibrator) for clock generation and determine the frequency of the signal generated. | CO3 | A | 12 |
| 20. | a. | Examine the mathematical model of a first-order low-pass filter and determine its cut-off frequency and gain. | CO4 | A | 8 |
|  | b. | Determine the component values required to design a second-order high-pass Butterworth filter with a cut-off frequency of 2 kHz. | CO4 | A | 4 |
| 21. |  | Explain the working of a 555 timer in monostable mode with a diagram and waveform. Provide the necessary steps to calculate its time period. | CO5 | U | 12 |
| 22. | a. | Develop the circuit of a differential amplifier using an Op-Amp and derive the output voltage expression for given inputs. | CO2 | A | 8 |
|  | b. | Illustrate and explain the working of a Wien Bridge Oscillator with a circuit diagram and waveform. | CO3 | U | 4 |
| 23. |  | Explain the basic planar process steps involved in IC fabrication with a neat diagram and discuss their significance. | CO1 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the working principle of a Successive Approximation ADC with an example. | CO6 | U | 6 |
|  | b. | Compare the weighted resistor DAC with the R–2R ladder DAC in terms of construction and real-world application. | CO6 | An | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Demonstrate the characteristics of operational amplifier. |
| CO2 | Design application circuits using linear ICs. |
| CO3 | Analyse the operational characteristics of op-amp for signal generation. |
| CO4 | Apply operational amplifier filter concepts for signal processing applications. |
| CO5 | Develop pulse generation circuits using PLL and 555 Timer. |
| CO6 | Implement data converters using op-amp in digital communication systems. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC2006 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DIGITAL SIGNAL PROCESSING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Determine the number of zeros to be padded with h(n) if linear convolution has to be performed using circular convolution. Let the number of samples in x(n) be 5 and the number of samples in h(n) be 6 . |  | CO1 | A | 1 |
| 2. | In a digital temperature monitoring system, the output at any time depends on both the current sensor reading and the previous recorded values stored in memory. Determine whether this system is static or dynamic based on its input–output relationship. |  | CO1 | A | 1 |
| 3. | Predict the number of complex multiplications required to compute an 𝑁-point DFT using direct computation |  | CO2 | A | 1 |
| 4. | The number of stages in the computation of DFT by radix-2 FFT is 5. Calculate the number of samples in x(n). |  | CO2 | A | 1 |
| 5. | In the design of a digital low-pass filter using frequency transformation methods, the analog prototype must be converted into a digital filter. Write the equation used in the bilinear transformation technique for digital filter design. |  | CO3 | A | 1 |
| 6. | In an audio signal processing application, a Chebyshev Type-I low-pass filter is selected due to its sharper roll-off characteristics. Sketch the magnitude response of the Chebyshev Type-I low-pass filter. |  | CO3 | A | 1 |
| 7. | State the condition on the impulse response coefficients h[n] of an FIR filter that ensures it has symmetric coefficients for linear phase response. |  | CO4 | R | 1 |
| 8. | List the disadvantages of the rectangular window method in FIR filter design. |  | CO4 | R | 1 |
| 9. | Identify the effect of truncation on signal representation. |  | CO5 | R | 1 |
| 10. | State the concept of signal processing in quantum systems. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | In a digital filtering application using circular buffers, two finite-length sequences x1[n]={2,1,1} and x2​[n]={2,2,2} are processed. Determine the circular convolution of these two sequences. |  | CO1 | A | 3 |
| 12. | List the steps involved to find FFT of a 2D signal. |  | CO2 | R | 3 |
| 13. | Compare one-to-one mapping with many-to-one mapping in digital filter design. |  | CO3 | An | 3 |
| 14. | List the characteristics of the FIR filter. |  | CO4 | R | 3 |
| 15. | State the effects of finite word length in digital filter design. |  | CO5 | R | 3 |
| 16. | Compare Harvard architecture with and Von Newman architecture |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | In a real-time signal processing system, a long input sequence x[n]= {2,3,4,-1,0,1,3,2,0,1,2,1,2} is filtered using h[n] = {1,1,1} through block processing. Determine the output using the overlap-save method of sectioned convolution. | CO1 | A | 12 |
| 18. | a. | In an OFDM-based communication system, an 8-point sequence x[n] = {8,0,0,0,0,0,0,0} must be transformed into the frequency domain efficiently. Calculate the 8-point DFT using the Radix-2 DIT-FFT algorithm. | CO2 | A | 8 |
|  | b. | Sketch the signal flow graph of 4-point DFT using the Radix-2 DIF-FFT algorithm | CO2 | A | 4 |
| 19. |  | In implementing a digital controller, the system is governed by the difference equation: 𝑌(𝑛) = −0.5𝑦(𝑛 − 1)+ 0.22𝑦(𝑛 − 2)+ 0.45𝑥(𝑛) + 0.3 𝑥(𝑛 − 1)+ 0.6 𝑥(𝑛 −2) Sketch the Direct Form-I and Direct Form-II realization of this LTI system suitable for hardware implementation. | CO3 | A | 12 |
| 20. |  | Determine the transfer function of digital FIR filter using Hamming window with the length N=7. The desired response of the filter is Hejω= e-3jω ; -3π4≤ω≤3π40 ;otherwise | CO4 | A | 12 |
| 21. | a. | Illustrate the effect of quantization in filter coefficients with an example. | CO5 | A | 6 |
|  | b. | Discuss the types of number representation with necessary examples. | CO5 | A | 6 |
| 22. |  | In a communication receiver, a digital low-pass filter must satisfy the following specifications: 0.7≤Hejω≤1 ; 0≤ω≤0.2π Hejω≤0.3 ; 0.6π≤ω≤π Using impulse invariant transformation, Determine the Butterworth digital IIR filter for these specifications. | CO3 | A | 12 |
| 23. |  | Determine the 8 point DFT of a sequence x[n]= {1,2,3,4,4,3,2,1} using radix-2 DIF-FFT algorithm. | CO2 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the implementation of the Least Mean Squares adaptive algorithm and its role in noise cancellation for audio signals. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze discrete time signals for LTI systems. |
| CO2 | Apply DFT for 1D and 2D signal processing. |
| CO3 | Design IIR filters using transformation methods. |
| CO4 | Design FIR filters for digital audio processing. |
| CO5 | Evaluate the performance measures of quantization techniques. |
| CO6 | Formulate procedures to improve signal quality using DSP processor. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC3004 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ANALOG VLSI DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Calculate the output voltage of the given circuit for R = 16 kΩ. Also, find the maximum allowable value of R that ensures M1 operates in the saturation region, using the typical NMOS process parameters provided in Table 1. Table 1 Typical Process Parameter for 3µ NMOS and CMOS processes Parameter; n-channel; p-channel; units. K'; 24; 8; µA/V2. VTOE; 0.75; -0.75; V. VTOD; -3; 3; V. Ƴ(CMOS); 0.8; 0.4; V1/2. Ƴ(NMOS); 0.4; -; V1/2. Ф; 0.6; 0.6; V. λ; 0.01; 0.02; V-1. | CO1 | An | 12 |
|  | b. | Discuss the small-signal MOSFET model, explaining its equivalent circuit, key parameters, and the impact of channel length modulation on circuit performance. | CO1 | U | 4 |
| 2. | a. | Explain the design and operation of a Voltage Scaling DAC with neat circuit diagrams, specifying its architecture, signal conversion process, and performance characteristics. | CO2 | A | 12 |
|  | b. | Determine the DNL (codes 011,100,101,110,111) for the 3-bit non-ideal DAC whose transfer function is shown below.(Fig.1) Fig.1 3-bit non-ideal DAC function | CO2 | A | 4 |
| 3. | a. | Explain resistor emulation in parallel switched capacitor circuits | CO3 | A | 12 |
|  | b. | Show the value of the capacitor C that will emulate a 1.2 MΩ resistor, if the clock frequency of a Parallel switched capacitor circuit is 150kHz. | CO3 | U | 4 |
| 4. | a. | Determine the output voltage expression of a switched capacitor integrator and illustrate the operating modes of the switched capacitor integrator during ϕ1 and ϕ2 clock phases. | CO4 | A | 10 |
|  | b. | Illustrate the design of a CMOS current amplifier using a current mirror configuration, and explain how it enhances current gain and efficiency in analog circuits. | CO4 | A | 6 |
| 5. |  | Evaluate the design constraints, phase noise characteristics, and tuning range of a Voltage-Controlled Oscillator (VCO), considering the effects of varactor tuning, supply voltage variations, and the influence of inductor (L) and capacitor (C) values on frequency stability. | CO5 | E | 16 |
| 6. | a. | Design a CMOS differential amplifier incorporating a current-mirror load with its performance characteristics. | CO4 | C | 8 |
|  | b. | Distinguish Sah’s and Shichman-Hodges model with neat description and obtain the equivalent circuits for MOSFET in Ohmic and Saturation region. | CO1 | An | 8 |
| 7. |  | Illustrate the operation of a Class B power amplifier and derive its maximum Power efficiency using appropriate expressions and circuit diagrams. | CO4 | A | 16 |
| PART – B (1 X 20 = 20 MARKS) [Compulsory Question] |  |  |  |  |  |
| 8. | a. | Analyze the operation of a two-stage CMOS comparator in comparison with a single-stage comparator, highlighting the trade-offs in speed, power, and accuracy. | CO6 | An | 12 |
|  | b. | Illustrate the track-and-latch stage in a latched comparator and discuss its significance in achieving fast decision-making. | CO6 | A | 8 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze the characteristics of MOS transistors. |
| CO2 | Design data converters for analog systems. |
| CO3 | Examine the different types of switched capacitor circuits. |
| CO4 | Assess the performance characteristics of CMOS amplifiers. |
| CO5 | Illustrate the tuning characteristics of oscillators. |
| CO6 | Design different types of CMOS comparators. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC3005 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # HARDWARE DESIGN VERIFICATION TECHNIQUES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Evaluate the importance of functional verification in detecting design flaws prior to fabrication, with appropriate examples. | CO1 | U | 8 |
|  | b. | Compare and contrast Directed Verification and Constrained-Random Verification (CRV) by examining their methodologies, advantages, and limitations. | CO1 | U | 8 |
| 2. | a. | Design and implement a System Verilog example showcasing the effective use of a weighted constraint in random stimulus generation. | CO2 | A | 8 |
|  | b. | Explain the purpose and operation of each layer within the testbench, including the major components associated with each layer. | CO2 | A | 8 |
| 3. | a. | Evaluate the concept of Inheritance in System Verilog and describe how it supports code reuse and extensibility in verification environments. | CO3 | E | 8 |
|  | b. | Explain how virtual methods enable dynamic binding and runtime method resolution in System Verilog classes. | CO3 | E | 8 |
| 4. | a. | Evaluate the significance of Coverage-Driven Verification in ensuring verification completeness and reliability in complex digital designs. | CO4 | An | 8 |
|  | b. | Illustrate the step-by-step procedure for constructing a layered test bench for the given FIFO. | CO4 | An | 8 |
| 5. | a. | Define the concept of a UVM Agent and explain its role within a UVM-based verification environment. | CO5 | An | 8 |
|  | b. | Compare and contrast OVM (Open Verification Methodology) and UVM by analyzing their features, libraries, and verification approaches. | CO5 | E | 8 |
| 6. | a. | Analyze the role of constructors in System-Verilog classes and evaluate how they contribute to object initialization, reuse, and flexibility in testbench design. | CO2 | A | 8 |
|  | b. | Explain the concept of Transaction-Level Modeling (TLM) communication in the Universal Verification Methodology (UVM) and its role in component interaction. | CO5 | E | 8 |
| 7. | a. | Persuade the role and importance of the sampling point in concurrent assertions and its impact on assertion evaluation and correctness. | CO4 | E | 8 |
|  | b. | Describe the structure and anatomy of a covergroup in System-Verilog, identifying its key components. | CO4 | An | 8 |
| PART – B (1 X 20 = 20 MARKS) [Compulsory Question] |  |  |  |  |  |
| 8. | a. | Explain the key challenges involved in verifying modern System-on-Chip (SoC) designs where hardware and software are tightly interdependent. | CO6 | E | 10 |
|  | b. | Evaluate why a combination of these techniques is essential for achieving comprehensive verification coverage in complex SoC designs. | CO6 | E | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Develop layered test bench for functional design verification. |
| CO2 | Develop system Verilog codes for logic circuits. |
| CO3 | Develop System Verilog codes for real time applications. |
| CO4 | Design logic circuits using Open Verification Methodology. |
| CO5 | Examine logic circuits using Universal Verification Methodology. |
| CO6 | Practice the hardware verifications technique for practical applications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC3005 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # HARDWARE DESIGN VERIFICATION TECHNIQUES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Estimate the verification process and the importance of functional design verification in digital systems. | CO1 | E | 10 |
|  | b. | Analyze the verification process in digital design with a neat block diagram. | CO1 | An | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Analyze the use of System Verilog concepts in illustrating the connection between the test bench and the design. | CO2 | An | 10 |
|  | b. | Apply different data types available in SystemVerilog with suitable examples. | CO2 | A | 10 |
| 3. | a. | Analyze the application of inheritance in SystemVerilog with suitable examples. | CO3 | An | 10 |
|  | b. | Analyze the role of downcasting, virtual methods, abstract classes, and parameterized classes in modern verification environments. | CO3 | An | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Analyze functional coverage and different coverage types in SystemVerilog. | CO4 | An | 10 |
|  | b. | Estimate the behavior and effectiveness of asynchronous assertions in SystemVerilog by examining their timing, responsiveness, and verification impact. | CO4 | E | 10 |
| 5. | a. | Analyze the major components of the Universal Verification Methodology (UVM) and determine their specific functions in a verification environment. | CO5 | An | 10 |
|  | b. | Analyze UVM agents, their architecture, and their role in the Universal Verification Methodology (UVM) environment. | CO5 | An | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Apply basic object-oriented concepts in SystemVerilog to develop modular and reusable verification components. | CO2 | A | 10 |
|  | b. | Evaluate the importance of OOP concepts in building reusable verification environments using SystemVerilog. | CO2 | E | 10 |
| 7. | a. | Analyze methodology basics used in hardware verification with reference to structured frameworks like Universal Verification Methodology (UVM). | CO1 | An | 10 |
|  | b. | Construct a layered testbench architecture with a neat example using Universal Verification Methodology (UVM). | CO1 | C | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Analyze the differences between Open Verification Methodology (OVM) and Universal Verification Methodology (UVM) in terms of architecture, features, and industry adoption. | CO5 | An | 10 |
|  | b. | Analyze Voice over IP (VoIP) Network SoC verification with respect to functionality, protocols, and system-level challenges. | CO6 | An | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Create a detailed framework addressing the verification challenges in CPU (soft core processor) verification using Universal Verification Methodology (UVM). | CO6 | C | 10 |
|  | b. | Analyze the verification of cache memory subsystems in detail using structured methodologies such as Universal Verification Methodology (UVM). | CO6 | An | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Develop layered testbench for functional design verification. |
| CO2 | Construct System Verilog codes for logic circuits. |
| CO3 | Develop System Verilog codes for real time applications. |
| CO4 | Design logic circuits using Open Verification Methodology. |
| CO5 | Examine logic circuits using Universal Verification Methodology. |
| CO6 | Demonstrate hardware verification techniques for practical applications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC3010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SYSTEM-ON-CHIP DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. |  | Design a system specification requiring low power and moderate performance, give the SoC design flow to identify the key activities in RTL design, verification, synthesis, and physical. | CO1 | E | 16 |
| 2. | a. | Evaluate a pipelined processor used in SoC: 30% instructions are branches Branch misprediction rate is 8% Branch penalty is 3 cycles Base CPI = 1 (a) Calculate effective CPI. (b) Analyse methods to reduce pipeline delay and branch cost | CO2 | E | 8 |
|  | b. | Analyse the role of FPGA in SoC design. Compare it with ASIC in terms of flexibility, cost, and performance. | CO2 | An | 8 |
| 3. | a. | Evaluate different processor architectures VLIW, Superscalar, Vector and justify which processor core should be selected. Discuss performance, power, area, and scalability trade-offs. | CO3 | E | 8 |
|  | b. | Analyze different processor architectures (RISC, CISC, VLIW, Superscalar) for SoC design. Justify the selection of a suitable processor for a low-power embedded application. | CO3 | E | 8 |
| 4. | a. | Illustrate AMBA-based interconnect architecture can be implemented in an SoC. Explain bus arbitration ensures efficient communication. | CO4 | U | 8 |
|  | b. | A 4×4 Mesh Network-on-Chip (NoC) is designed using switch-based routers. Each router introduces 2 clock cycles delay, and each link introduces 1 clock cycle delay. Tasks: a) Calculate the total latency for data transfer from node (0,0) to (3,3). b) Design the router switch control logic using basic gates for packet forwarding (use arbitration logic). c) Analyse how pipelining at gate level reduces latency. | CO4 | E | 8 |
| 5. | a. | Articulate the SoC memory hierarchy (cache, scratchpad, DRAM), to design a memory subsystem for high-speed applications. | CO5 | A | 8 |
|  | b. | Explain the SoC design validation flow. Analyze the importance of testbenches, simulation, and co-simulation in verifying complex systems. | CO5 | A | 8 |
| 6. |  | Evaluate various power reduction techniques (clock gating, voltage scaling, power gating). Justify their impact on performance and energy efficiency. | CO5 | E | 16 |
| 7. |  | Evaluate the role of GPU in SoC for multimedia applications. Discuss architecture, parallelism, and performance benefits. | CO6 | E | 16 |
| PART – B (1 X 20 = 20 MARKS) [Compulsory Question] |  |  |  |  |  |
| 8. | a. | A modern multi-core System-on-Chip (SoC) employs a Network-on-Chip (NoC) with switch-based interconnects to enable communication between processing elements, memory, and peripherals. - Explain the architecture of a NoC with switch interconnects, including topology and components. - Describe the function of switches/routers in packet-based communication. | CO6 | A | 10 |
|  | b. | Analyse the Modern multimedia and gaming systems integrate a 3D Graphics Processor (GPU) within a System-on-Chip (SoC) to handle complex rendering tasks efficiently. Based on this context, answer the following: - Explain the role of a 3D Graphics Processor in an SoC. - Describe the architecture of a GPU within an SoC, including major components such as shader cores, memory interface, and interconnect. - Discuss a case study of a GPU-based SoC and highlight its features. | CO6 | An | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Formulate address complex issues in System-on-Chip (SoC) design from a comprehensive perspective. |
| CO2 | Evaluate the performance of SoC-based designs by selecting and comparing various processors. |
| CO3 | Apply SoC design methodologies to implement practical system designs. |
| CO4 | Create interconnection structures for an SoC-based system design. |
| CO5 | Assess SoC validation techniques for handling moderately complex systems. |
| CO6 | Analyse the issues related to System-on-Chip design and implementation. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC3012 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # NANOSCALE DEVICES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | A nanoscale MOSFET exhibits DIBL (Drain Induced Barrier Lowering) of 80 mV/V. The threshold voltage at VDS=0.05Vis 0.4 V. (i) Calculate the threshold voltage at VDS=1V (ii) Analyze the impact of DIBL on off-state leakage current. | CO1 | AN | 12 |
|  | b. | Explain leakage current mechanisms in nanoscale CMOS devices. | CO1 | U | 8 |
|  |  | (OR) |  |  |  |
| 2. | a. | A FinFET has the following parameters: Fin height = 30 nm, Fin width = 8 nm, Number of fins = 3 (i) Calculate the total effective channel width. (ii) Compare with a planar MOSFET of width 90 nm. | CO2 | AN | 12 |
|  | b. | Design a nanowire MOSFET structure using multi-gate device concepts for low-power applications. | CO2 | C | 8 |
| 3. |  | For a double-gate MOSFET, the scale length is given by λ=√(εsi/εox). tsi. tox Given:tsi=5 nm, tox=1 nm, εsi/εox=3 (i) Compute the scale length. (ii) Analyze its implication on short-channel effects. | CO3 | An | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | A quantum well has width L=5 nm. Assume an electron confined in an infinite potential well En=n2h28mL2 (i) Calculate the ground state energy E1. (ii) Determine the energy difference between first two levels. | CO4 | A | 20 |
| 5. | a. | Apply voltage scaling techniques to estimate power reduction in nanoscale CMOS circuits. | CO5 | A | 8 |
|  | b. | A CMOS inverter drives a load capacitance of 20 fF at frequency 1 GHz with VDD=1V. (i) Calculate dynamic power consumption. (ii) If switching activity = 0.2, recompute power. (iii) Analyze power reduction strategies. | CO5 | AN | 12 |
|  |  | (OR) |  |  |  |
| 6. | a. | Analyze short-channel effects in nanoscale MOSFETs. | CO3 | AN | 8 |
|  | b. | A nanoscale transistor operates in ballistic transport regime with current 120μAand drift-diffusion current 80μA: (i) Compute percentage improvement in current. (ii) Evaluate whether achieving fully ballistic transport is practical, considering fabrication limits. | CO3 | E | 12 |
| 7. |  | A nanoscale MOSFET experiences Random Telegraph Noise (RTN) with fluctuation amplitude 20 mV. Recommend mitigation techniques at device and circuit levels. | CO5 | E | 20 |
|  |  | (OR) |  |  |  |
| 8. |  | A TFET has slope 40 mV/dec vs MOSFET 60 mV/dec - Estimate current improvement over 3 decades. - Analyze low-power advantage. | CO6 | AN | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Develop a nanoscale device for low-power operation under the constraint VDD=0.5V, Leakage current 10 nA | CO6 | A | 12 |
|  | b. | Explain CNTFET (Carbon Nanotube FET) operation. | CO6 | U | 8 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Evaluate the leakage current reduction techniques in CMOS devices. |
| CO2 | Analyze FinFETs and nanowire MOSFETs for low-power applications. |
| CO3 | Analyze two-dimensional scaling in single and multi-gate MOSFETs. |
| CO4 | Develop low-power and voltage scaling strategies for nanoscale CMOS. |
| CO5 | Analyze heterostructure and nanowire-based devices for nanoelectronic applications. |
| CO6 | Apply spintronics, RTDs, and graphene-FETs in circuit design. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC3013 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # VLSI TECHNOLOGY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. |  | Explain in detail about the Czochralski crystal growth technique with a neat sketch | CO1 | A | 16 |
| 2. |  | Differentiate the kinetics of silicon dioxide growth and correlate the mechanisms governing thick and thin oxide formation | CO2 | An | 16 |
| 3. | a. | Examine the process of e-beam lithography with a neat sketch. | CO3 | A | 8 |
|  | b. | Interpret the dry oxidation process with a neat schematic diagram and elaborate the technique with its features and applications. | CO2 | A | 8 |
| 4. |  | Analyze in detail about the diffusivities of Boron, Phosphorous, Arsenic and Antimony. | CO4 | An | 16 |
| 5. | a. | Summarize the diffusion enhancement and retardation mechanisms in SiO2. | CO4 | E | 8 |
|  | b. | Design the metallization choices using metals and alloys based on their properties in IC fabrication | CO5 | C | 8 |
| 6. |  | Examine the Physical Vapour Deposition technique, analyze its types and factors influencing thin film formation in detail with suitable diagrams. | CO5 | A | 16 |
| 7. |  | Illustrate the NMOS IC fabrication process and highlight its significance on nanoscale device scaling. | CO6 | An | 16 |
| PART – B (1 X 20 = 20 MARKS) [Compulsory Question] |  |  |  |  |  |
| 8. |  | Implement VLSI assembly technologies and illustrate their role in IC packaging with suitable diagrams. | CO6 | A | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design process flow for the fabrication of semiconductor devices. |
| CO2 | Examine the oxidation techniques in fabrication process. |
| CO3 | Demonstrate the lithographic fabrication process. |
| CO4 | Analyze the performance of various diffusion models. |
| CO5 | Evaluate the metallization techniques in VLSI technology. |
| CO6 | Apply the various IC technologies for chip design. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EC3014 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # QUANTUM CIRCUIT DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Differentiate superposition from entanglement. Represent the qubit ¦ψ⟩ = 1/√3 ¦0⟩ + √(2/3) ¦1⟩ on the Bloch sphere and interpret its meaning. | CO1 | AN | 10 |
|  | b. | Compute the final state using matrix operations for a qubit initially in state ¦0⟩ and passed through a Hadamard gate followed by a Pauli-X gate.. | CO1 | A | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Evaluate the decomposition of SWAP gate into elementary gates using matrix multiplication. | CO2 | AN | 10 |
|  | b. | Compute the resulting states for ¦ψ⟩ = (1/√2)(¦0⟩ + ¦1⟩) using Pauli-Z and Phase gates. | CO2 | A | 10 |
| 3. |  | For a 3-qubit input ¦101⟩: (i) Construct QFT circuit (ii) Show sequence (iii) Compute final output. | CO3 | A | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | For 3-qubit phase-flip code ¦+++⟩: (i) Draw syndrome measurement circuit using ancilla qubits. (ii) Detect phase-flip on second qubit. | CO4 | AN | 20 |
| 5. |  | Compare superconducting qubits, ion traps, and photonic systems with respect to coherence time, fidelity, and scalability. Justify which is most suitable for large-scale implementation. | CO5 | E | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Consider a 3-qubit quantum error correction code with stabilizer generators S1 = Z⊗Z⊗I, S2 = I⊗Z⊗Z - Determine the code space (Valid encoded states) satisfying the stabilizer conditions. - Verify that the logical states belong to the +1 eigenspace of the stabilizers. | CO4 | A | 20 |
| 7. | a. | Examine the Toffoli gate using single-qubit and CNOT gates. Apply to ¦110⟩ and analyze complexity. | CO2 | A | 10 |
|  | b. | Explain quantum gate universality with examples. | CO2 | U | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Given a hybrid quantum-classical loop with parameter update θ_new = θ − η(dC/dθ), Examine optimization steps and convergence. | CO6 | A | 10 |
|  | b. | Explain a quantum machine learning circuit for classification. | CO6 | U | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | A quantum circuit consists of the following sequence of gates on a single qubit H → Z → H → X → X. (i)Simplify the circuit using gate identities (ii)Estimate the reduction in gate count. | CO6 | E | 10 |
|  | b. | Analyze the role of classical optimizers (gradient descent vs gradient-free methods) in the variational quantum algorithms and compare their convergence. | CO6 | AN | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply quantum mechanics concepts to represent quantum states and gates. |
| CO2 | Evaluate the performance of quantum circuits using simulation tools. |
| CO3 | Develop quantum circuits for implementing algorithms like Grover’s and Shor’s. |
| CO4 | Employ error correction codes to mitigate quantum noise and decoherence. |
| CO5 | Evaluate the performance of quantum circuits for their suitability in real-world hardware implementations. |
| CO6 | Construct advanced quantum circuit designs in applications like cryptography and optimization. |

END SEMESTER EXAMINATION – APRIL/MAY 2026

| # Course Code | # 24EC3022 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # MACHINE LEARNING IN VLSI DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Analyze the various machine learning algorithms (Linear Regression, Decision Trees, Neural Networks) for solving VLSI design problems and justify its suitability for different stages of chip design. | CO1 | An | 20 |
|  |  | (OR) |  |  |  |
| 2. |  | Design a machine learning framework for VLSI applications integrating supervised and reinforcement learning approaches for adaptive chip optimization. | CO2 | C | 20 |
| 3. | a. | Evaluate the role of machine learning in Electronic Design Automation (EDA) tools with respect to design space exploration and high-level synthesis. | CO3 | E | 10 |
|  | b. | Interpret the effectiveness of ML-based static timing analysis (STA) compared to traditional timing analysis methods. | CO3 | A | 10 |
|  |  | (OR) |  |  |  |
| 4. |  | Develop a machine learning-based circuit simulation model using neural networks for fast and accurate prediction of circuit behavior. | CO4 | C | 20 |
| 5. |  | Evaluate the application of machine learning techniques in optimizing Power, Performance and Area (PPA) in physical design stages. | CO5 | E | 20 |
|  |  | (OR) |  |  |  |
| 6. | a. | Analyze the machine learning approaches for floor planning, placement and routing congestion prediction in VLSI design. | CO1 | An | 10 |
|  | b. | Assess predictive models for power estimation using ML and compare their performance with analytical models. | CO4 | E | 10 |
| 7. |  | Apply machine learning techniques for fault detection and reliability estimation in VLSI circuits and evaluate their effectiveness. | CO5 | A | 20 |
|  |  | (OR) |  |  |  |
| 8. | a. | Evaluate the role of machine learning in predictive maintenance of integrated circuits and reliability enhancement. | CO6 | E | 10 |
|  | b. | Analyze the different ML-based fault prediction models and their impact on VLSI testing methodologies. | CO3 | An | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Design a hardware-aware machine learning architecture for VLSI implementation, incorporating FPGA/ASIC-based acceleration and discuss its extension to 3D IC and quantum VLSI design. | CO6 | C | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define reconfigurable computing hardware |
| CO2 | Demonstrate reconfigurable computing architectures and systems |
| CO3 | Construct reconfigurable processing fabric architectures |
| CO4 | Inspect the Operating System for Reconfigurable Computing |
| CO5 | Determine the mapping and routing for reconfigurable platforms |
| CO6 | Build the various logic in reconfigurable architectures |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25EC205 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # TRANSFORMS AND NUMERICAL METHODS FOR ELECTRONICS ENGINEERS | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | The current in a linear electrical system is represented by the time-domain functionit=e-3t-sin2t. Identify the Laplace transform of the current i(t). | 1a | R | 1 |
| 2. | The transfer functions of an RC filter in a communication receiver is given by Hs=(s+3)(s+3)2+16. Using the inverse Laplace transform, identify the time-domain impulse response h(t). | 1d | R | 1 |
| 3. | Predict the time-domain signal for the frequency-domain function X(ω)=2π δ(ω−ω0) using Fourier transform. | 2a | U | 2 |
| 4. | State any two properties of Fourier transform. | 2b | R | 2 |
| 5. | An alternating disturbance input is applied to the controller as R(n)=(−1)n . Predict the Z-transform of R(n). | 3a | U | 3 |
| 6. | In a discrete-time control system, the Z-transform of a sampled system response is given by F(z)=zeaTzeaT-cosbTz2e2aT-2zeaTcosbT+1. Using the Initial Value Theorem of the Z-transform, identify the initial value f(0) of the system response. | 3a | R | 3 |
| 7. | The frequency response magnitude of a control system is measured at equal intervals as shown below: Frequency (rad/s); 0; 1; 2; 3; 4; 5; 6. Magnitude; 1.0; 2.5; 3.8; 5.2; 6.0; 7.1; 8.3. Predict the area under the frequency response curve using Simpson’s 1/3rd rule. | 4d | U | 4 |
| 8. | Identify the conditions to apply Simpson’s 1/3rd rule and Simpson’s 3/8th rule for finding the area under the curve. | 4d | U | 4 |
| 9. | In an RC circuit, the capacitor voltage satisfies dVdt=-V, V(0)=1. Using Euler's method with step size h=0.01, predict the voltage at t=0.01 seconds. | 5b | U | 5 |
| 10. | In a first-order electrical system, the output signal y(x) varies with input x according to the differential equation: dydx=x-y, y(0)=0. Using Modified Euler's method with h=0.1, estimate the output voltage y at x=0.1. | 5b | U | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | The input excitation in a linear electrical system is represented by time-weighted functions. Compute the Laplace transform of the input signal itassuming zero initial conditions. (i) i(t)=te-3t (ii) i(t)=e-at-e-btt | 1b | A | 1 |
| 12. | To Model the signal propagation for a ramp input v(t)=t, t≥0 . Determine its Fourier sine transform. | 2e | A | 2 |
| 13. | In a discrete-time control system, the Z-domain representation of the system output is given by X(z)=z(z-1)(z-2). To analyze the time-domain behavior of the system response, it is necessary to obtain the corresponding discrete-time sequence. Find the inverse Z-transform of X(z) and hence determine the output sequence x[n] of the system. | 3b | A | 3 |
| 14. | The step response of a control system is recorded at equal time intervals as shown below: Time(s); 0.0; 0.1; 0.2; 0.3; 0.4. Output response; 2.0; 2.6; 3.4; 4.5; 6.0. Estimate the system response at time t=0.15 s using Newton’s Forward Interpolation method. | 4c | An | 4 |
| 15. | In a signal processing circuit, the output voltage y(x) evolves with time according to dydx=x+y, where x represents time (in seconds). Initially, the system output is y(0)=1. Using the fourth-order Runge-Kutta method with step size h=0.1, determine the output voltage at x= 0.1 seconds. | 5d | A | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | In a signal conditioning unit, the output voltage y(t) of an electronic sensor varies with time according to the differential equation dydt-y=2. At the instant the sensor is switched on, at t=0, the initial output voltage is y(0)=2. Determine the expression for the sensor output y(t). | 1e | A | 1 |
| (OR) |  |  |  |  |
| 17 | A periodic signal f(t) is defined over one period as: f(t)=t,0<t<ππ-t,π<t<2π The signal repeats periodically with period T=2π. Estimate the Laplace transform of the periodic signal f(t). | 1c | An | 1 |
| 18 | - A signal is defined as f(x)=sin(4πx), 0≤x≤1. Estimate its Finite Fourier Transform. Fn=01sin4πxe-j2πnxdx ,Identify the frequency components present in the signal. - Evaluate the time-domain signal corresponding to the frequency-domain representation X(ω)=π[δ(ω−ω0)+δ(ω+ω0)] using fourier transform. | 2b | E | 2 |
| (OR) |  |  |  |  |
| 19 | - The voltage along a semi-infinite RC circuit is v(t)=V0e-at, t≥0, Evaluate its Fourier cosine transform. - Estimate the finite fourier transform of the finite-duration ramp signal f(x)=x, 0≤x≤1. | 2c,2e | E | 2 |
| 20 | A discrete-time system used in a digital filter is described by yn+2-5yn+1+6yn=1 with y[0]=0 and y[1]=1. Solve the system using the Z-transform method and obtain the output response. | 3c | A | 3 |
| (OR) |  |  |  |  |
| 21 | - In a discrete-time control system, the output sequence is f[n]=2-nn+7, n≥0. Using the Final Value Theorem of the Z-transform, predict the steady-state response of the system output. - A gradually increasing input with exponential weighting is applied as r(n)=nan . Identify the Z-transform of r(n) . | 3a | U | 3 |
| 22 | - A linear time-invariant system has the characteristic equation P(s)=s2 – 7. Estimate the root of P(s) with the initial guess x0=3 using Newton–Raphson method. - The frequency response magnitude of a control system is measured at equal frequency intervals as shown below: Frequency (rad/s); 0; 1; 2; 3; 4; 5; 6. Magnitude; 1; 2.5; 3.8; 5.2; 6; 7.1; 8.3. Evaluate the area under the frequency response curve using the Trapezoidal rule. | 4b, 4c | E | 4 |
| (OR) |  |  |  |  |
| 23 | The real part of the characteristic equation of a closed-loop system is f(x)=x3−4x−9.Using the Bisection Method, compute one of the roots of this equation to assess the stability of the system. | 4a | A | 4 |
| Compulsory Question: |  |  |  |  |
| 24 | In an electronic system, the output voltage y(x) varies with time according to dydx=x2-y where x represents time (in seconds). Initially, the system output is y(0)=1. Using the Taylor series method, estimate the output voltage at x=0.1 and x=0.2. | 5a | An | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25EC206 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # AI FOR ELECTRONICS ENGINEERS | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | A wireless channel has an estimated gain of 0.5. Calculate the received signal power if the transmitted signal power is 20 W. | 1e | A | 1 |
| 2. | List any four digital modulation techniques used in communication systems. | 1c | R | 1 |
| 3. | Identify the role of data preprocessing in AI-assisted design. | 2a | U | 2 |
| 4. | Two blocks are located at points A (2, 3) and B (10, 7) on a grid. Calculate the wire length required to connect block A to block B, assuming straight-line distance. | 2b | A | 2 |
| 5. | Distinguish k-NN from Naïve Bayes classifier. | 3a | An | 3 |
| 6. | State the purpose of converting Float32 to Int8 in neural networks. | 3b | R | 3 |
| 7. | List the key stages in feature matching pipeline. | 4b | R | 4 |
| 8. | Summarize the principle of YOLO detection. | 4c | U | 4 |
| 9. | Define time-domain features used in signal classification. | 5a | R | 5 |
| 10. | Summarize the segmentation criteria in audio signal processing. | 5d | U | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Analyze the latent features extracted by an autoencoder for anomaly detection in network traffic. | 1f | An | 1 |
| 12. | A machine learning classifier is used to identify faults in a logic circuit. The classifier predicts three classes: Fault-Free, Stuck-At-0 (SA0), and Stuck-At-1 (SA1). The confusion matrix obtained is as follows: Actual \ Predicted; Fault-Free; SA0; SA1. Fault-Free; 9; 1; 0. SA0; 2; 6; 0. SA1; 0; 1; 7. Calculate the classification accuracy of the model and examine the classifier’s performance. | 2d | A | 2 |
| 13. | Develop a sensor interfacing pipeline using Raspberry Pi for environmental monitoring with data logging. | 3c | A | 3 |
| 14. | Examine the convergence behavior of Reinforcement Learning in motion control system. | 4f | A | 4 |
| 15. | Analyze the sequence modeling capability of RNN in speech recognition tasks. | 5e | An | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Develop a machine learning model for modulation classification in a noisy wireless environment and justify the choice of features and algorithm. | 1c | A | 1 |
| (OR) |  |  |  |  |
| 17 | Explain the AI/ML integration in the physical and network layers on latency and throughput in a 5G system with given traffic load parameters. | 1a | A | 1 |
| 18 | A dataset contains area, delay, and power values for multiple circuits. Analyze the effectiveness of regression models for PPA prediction using error metrics. | 2c | An | 2 |
| (OR) |  |  |  |  |
| 19 | A deep learning-based ATPG system generates patterns with 98% fault coverage. Analyze its limitations in detecting bridging faults and sequential faults. | 2f | An | 2 |
| 20 | Analyze the role of quantization in optimizing machine learning models for TinyML deployment on resource-constrained microcontrollers. | 3b | An | 3 |
| (OR) |  |  |  |  |
| 21 | Develop a TinyML pipeline for a home automation system integrating sensor interfacing, data preprocessing, and deployment. | 3e | A | 3 |
| 22 | Explain the CNN architecture for large-scale scene recognition in an intelligent surveillance system operating under varying lighting conditions. | 4d | A | 4 |
| (OR) |  |  |  |  |
| 23 | Examine the limitations of stereo vision depth estimation in an autonomous driving system under low illumination and adverse weather conditions. | 4e | A | 4 |
| COMPULSORY QUESTION: |  |  |  |  |
| 24 | Evaluate the performance of a CNN model for denoising signals with appropriate loss functions and optimization strategy. | 5c | E | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25EC207 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # C++ AND DATA STRUCTURES FOR ELECTRONICS ENGINEERS | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | Related CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | List 4 features of constructors. | 1b | R | 1 |
| 2. | Identify four common string operations in C++ for handling text data. | 1f | U | 1 |
| 3. | List the types of operator overloading in C++. | 2a | R | 2 |
| 4. | Explain the role of access specifiers in inheritance. | 2c | U | 2 |
| 5. | State the purpose of virtual functions in C++. | 3a | R | 3 |
| 6. | Explain how try, catch, and throw handle runtime errors in C++. | 3f | U | 3 |
| 7. | List the basic operations of a circular linked list. | 4a | R | 4 |
| 8. | Explain the role of recursion in image processing applications. | 4e | U | 4 |
| 9. | List two types of collision resolution techniques used in hashing. | 5c | R | 5 |
| 10. | Explain the difference between linear search and binary search with respect to time complexity. | 5d | U | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Determine the output of the following C++ program: #include <iostream> using namespace std; class Sensor { public: int reading; Sensor(int r) { reading = r; } }; int main() { Sensor temp(27); cout << "Reading: " << temp.reading; return 0; } | 1a | A | 1 |
| 12. | Use pointers and dynamic memory allocation in a C++ program for storing and displaying sensor readings in an industrial system. | 2g | A | 2 |
| 13. | Use ‘this’ pointer to develop a solution for a smart energy meter system where multiple device objects update and display their own power consumption values accurately. | 3d | An | 3 |
| 14. | Calculate the time efficiency of enqueue operations in queues implemented using arrays for storing pixel data in an image processing system. | 4c | An | 4 |
| 15. | Distinguish between Depth-First Search (DFS) and Breadth-First Search (BFS) for traversing a communication network topology, considering memory usage and traversal efficiency. | 5b | An | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Find the errors in the following C++ program that uses a structure and a class for storing and displaying signal strength in a communication system: #include <iostream> using namespace std; struct SignalData { int strength;}; class Signal { SignalData *data; public: Signal(int s) { data->strength = s; } void show() { cout << "Signal Strength: " << data.strength << endl; }}; int main() { Signal s1; Signal s2; s2.show(); s1.show(); cout << s2.strength; return 0; } | 1c | E | 10 |
| (OR) |  |  |  |  |
| 17 | Develop a C++ program using a copy constructor to model a signal processing system that stores a signal value, creates a duplicate object, and displays both values. | 1d | A | 10 |
| 18 | Explain the concepts of base class and derived class using a multilevel inheritance model in the context of a smart industrial monitoring system with hierarchical layers for device management, sensor data acquisition, and signal processing. | 2c | An | 10 |
| (OR) |  |  |  |  |
| 19 | Compare different approaches for designing a class hierarchy for a temperature sensor system that records multiple temperature readings, stores at least five readings in a file named sensor.txt, and retrieves the stored values for display. | 2h | An | 10 |
| 20 | Write a C++ program using a friend function for a secure wireless communication system where multiple transmitter devices share their signal strength values, and an external function compares and validates these values to ensure reliable communication. | 3b | A | 10 |
| (OR) |  |  |  |  |
| 21 | Write a C++ program using function templates and class templates to model a signal processing system that performs operations on different types of signal data (e.g., integer and floating-point values). | 3e | A | 10 |
| 22 | Choose the most suitable stack implementation using arrays or linked lists for buffering operations in an image processing system. | 4b | E | 10 |
| (OR) |  |  |  |  |
| 23 | Distinguish recursive approaches from iterative approaches for solving image-processing tasks in terms of performance and memory usage. | 4d | E | 10 |
| COMPULSORY QUESTION: |  |  |  |  |
| 24 | Use a Binary Search Tree (BST) to develop a solution for efficient data retrieval in a signal processing system that stores and processes frequency values. | 5a | A | 10 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25EC208 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # PYTHON PROGRAMMING FOR ELECTRONICS ENGINEERS | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | Related CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | Identify the given Python data type that is most suitable for storing a set of fixed sensor calibration constants that should never be altered during runtime: (0.78, 1.02, 0.95). | 1b | U | 1 |
| 2. | Differentiate between int and float data types with an example. | 1c | U | 1 |
| 3. | Determine the output of the following Python code. signal = "DIGITAL" print(signal * 2) | 2a | A | 2 |
| 4. | Determine the output of the following Python code. values = [1, 2, 3, 4, 5] print(values[1:4]) | 2b | A | 2 |
| 5. | Differentiate between pop() and popitem() methods in Python with an example. | 3a | U | 3 |
| 6. | State any one significance of the tuple datatype in Python. | 3b | R | 3 |
| 7. | Differentiate between a parameter and an argument in Python with an example. | 4a | U | 4 |
| 8. | Give an example of a non-void function. | 4a | U | 4 |
| 9. | Name the Python library commonly used for image processing. | 5a | R | 5 |
| 10. | Define a module in Python. | 5d | R | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | You are testing an LED circuit. The supply voltage is 9 Volts and the LED requires 3 Volts to operate. Write a Python program to: a. Create a variable supply_voltage and assign it the value 9. b. Create a variable led_voltage and assign it the value 3. c. Calculate the remaining voltage using the formula: remaining_voltage = supply_voltage - led_voltage, and display the result. | 1c | A | 1 |
| 12. | In a wireless communication system, a transmitter sends a message ID in the form of a string: message_id = "Tx_msg_2024_Sent" Write a Python program to: a. Convert the message ID to uppercase and display it. b. Check whether the transmission status "SENT" is present in the message ID. The status is defined as: If the message ID contains "SENT", display "Message transmitted successfully"; otherwise, display "Transmission failed". c. Extract the year from the message ID and display it. | 2b | A | 2 |
| 13. | An IoT smart agriculture system stores soil sensor readings (Moisture %, Temperature °C, and pH level) as a tuple: soil_data = (45, 27.5, 6.8) Write a Python program to: i) Display the tuple in reverse order. ii) Extract and display the first two sensor readings using slicing. iii) Concatenate the given tuple with another tuple new_data = (50,) and display the updated tuple. | 3b | A | 3 |
| 14. | In an embedded system, a processor needs to compute factorial values for certain calculations. Write a Python program to: a. Define a recursive function factorial(n) to calculate the factorial of a number. b. Call the function with a value (e.g., n = 5) and compute the factorial. c. Display the result in the format: "The factorial of 5 is 120" | 4c | A | 4 |
| 15. | An image is processed in a Python program using OpenCV for basic enhancement. Write a Python program to: a. Read an image and display it. b. Apply a median filter to reduce noise in the image. c. Detect edges in the image using a simple method and display the result. | 5b | A | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | An electronic protection system monitors the temperature of a high-power amplifier to ensure safe operation. The system status is defined as follows: Temperature 0°C to 40°C: Display "Low Temperature: System Idle" Temperature 41°C to 60°C: Display "Normal Operation" Temperature 61°C to 75°C: Display "Moderate Heating" Temperature 76°C to 90°C: Display "Warning: Overheating" Temperature above 90°C: Display "Critical: System Shutdown" Develop a Python program using nested if statements to check the temperature value and display the appropriate system status. | 1d | A | 1 |
| (OR) |  |  |  |  |
| 17 | In a power control system, the current in a circuit increases gradually during operation. The system continuously monitors the current until it reaches a predefined safe limit. Write a Python program to: a) Initialize the current value as 2A. b) Use a while loop to increase the current by 2A in each step. c) Display the current value at each step. d) Stop the loop when the current reaches 10A. e) Display the message "Safe limit reached" | 1e | A | 1 |
| 18 | In a communication system, packet sizes are recorded in a list for analysis and optimization of network performance. Write a Python program to: a) Create a list packets = [120, 150, 200] (b) Add a new packet of size 180 to the list (c) Remove the packet of size 150. d) Find the maximum packet size (e) Display the updated list. | 2c | A | 2 |
| (OR) |  |  |  |  |
| 19 | In a digital communication system, message identifiers are stored as strings containing information such as transmission type, data type, and status. Write a Python program to: a) Create a string msg = "TX_DATA_2025_OK". b) Convert the string to lowercase. c) Check whether the status "OK" is present in the string. d) Extract the year from the string. e) Display the message details using string formatting. | 2b | A | 2 |
| 20 | An IoT-based smart system stores sensor data in the form of key-value pairs, where each key represents a sensor type and the value represents its reading. Write a Python program to: a) Create a dictionary sensors = {"temp": 30, "hum": 60}. b) Add a new key-value pair for pressure. c) Update the temperature value. d) Access and display the humidity value. e) Display all the keys in the dictionary. | 3d | A | 3 |
| (OR) |  |  |  |  |
| 21 | An IoT network maintains a collection of unique device IDs to track active devices. Since duplicate entries are not allowed, the IDs are stored in a set. Write a Python program to: a) Create a set devices = {101, 102, 103} (b) Add a new device ID to the set. (c) Remove an existing device ID (d) Perform a union operation with another set {104, 105}. (e) Display the final set of device IDs. | 3c | A | 3 |
| 22 | An electronics engineer frequently needs to calculate the output voltage of a standard two-resistor voltage divider circuit. Write a Python program to: a. Define a function called calculate_v_out(v_in, r1, r2) that takes three parameters (input voltage, resistor 1, and resistor 2). b. Inside the function, create a local variable called v_out and calculate the output voltage using the formula: v_in * (r2 / (r1 + r2)). c. Return v_out as a single return value. d. Test the function by passing 5.0 Volts, 1000 Ohms, and 2000 Ohms. e. Store the result in a variable and print it formatted to two decimal places. | 4b | A | 4 |
| (OR) |  |  |  |  |
| 23 | In a signal processing application, scaling and attenuating the signal are often performed based on system requirements. Functions can be used to perform these operations efficiently. Write a Python program to perform the following tasks. a) Define scale(x) → multiply by 3. b) Define attenuate(x) → divide by 3. c) Call functions. d) Display results. e) Print formatted message. | 4d | A | 4 |
| Compulsory Question: |  |  |  |  |
| 24 | An image is processed to identify object boundaries using edge detection. Write a Python program to: a) Read an image. b) Convert it to grayscale. c) Apply edge detection using Canny method. d) Display the detected edges. e) Display the original image for comparison. | 5c | A | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25EC209 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # PROGRAMMING FOR CIRCUIT DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | List the steps to test a power supply circuit using SPICE. | 1a | R | 1 |
| 2. | Define transient response in automotive circuits. | 1d | R | 1 |
| 3. | List the components of a mobile charger rectifier. | 2a | R | 2 |
| 4. | Define ripple voltage in power supplies. | 2b | R | 2 |
| 5. | Define VI in industrial automation. | 3a | R | 3 |
| 6. | State use of Sub VI. | 3b | R | 3 |
| 7. | State LPF use in biomedical signals. | 4a | R | 4 |
| 8. | Define gain in amplifiers. | 4d | R | 4 |
| 9. | Identify digital modulation in mobile networks. | 5a | U | 5 |
| 10. | List the MATLAB tools for programming communication systems. | 5b | R | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Calculate the current through 3 ohm resistor of a linear circuit. | 1a | A | 1 |
| 12. | Sketch the digital logic for the equation Y = I0 * S0' * S1' + I1 * S0' * S1 + I2 * S0 * S1' + I3 * S0 * S1. | 2b | A | 2 |
| 13. | Develop a Virtual Instrument (VI) for LED control using LabVIEW and explain its working. | 3a | A | 3 |
| 14. | Develop a start/stop button to control stopwatch operation in a LabVIEW VI. | 4b | A | 4 |
| 15. | Develop a code for BPSK modulation in MATLAB for a real-time wireless communication system. | 5d | A | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Determine Kirchhoff’s laws in a complex industrial electrical network using SPICE. Assume the source voltage (VAB) is 15 V. | 1c | A | 1 |
| (OR) |  |  |  |  |
| 17 | Evaluate the frequency response of an RL circuit used in industrial filtering applications. | 1e | E | 1 |
| 18 | Develop a Full adder circuit for embedded processor applications using SPICE. | 2d | A | 2 |
| (OR) |  |  |  |  |
| 19 | Develop a full-wave rectifier for SMPS applications and analyze its performance using SPICE. | 2c | A | 2 |
| 20 | Develop a LabVIEW Sub VIs for a voltage to current conversion modular automation system using LabVIEW. | 3d | A | 3 |
| (OR) |  |  |  |  |
| 21 | Develop a LabVIEW VI for a real-time degree to Fahrenheit conversion for industrial monitoring system. | 3f | A | 3 |
| 22 | Develop a LabVIEW VI for gain control circuit for amplifying weak signals in measurement systems. | 4d | A | 4 |
| (OR) |  |  |  |  |
| 23 | Develop a LabVIEW VI for signal conditioning circuit using a low-pass filter for noise reduction in sensor systems. | 4c | A | 4 |
| COMPULSORY QUESTION: |  |  |  |  |
| 24 | Perform AM and FM modulation for a broadcasting system using MATLAB and examine the waveform. | 5b | A | 5 |