SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 18EE2002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # NETWORK THEORY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State Kirchhoff’s Voltage Law. |  | CO1 | R | 1 |
| 2. | Predict the number of linearly independent mesh equations (M), if the circuit has ‘B’ branches and ‘N’ nodes including the reference node. |  | CO1 | A | 1 |
| 3. | Identify the duality of Norton’s theorem. |  | CO2 | U | 1 |
| 4. | State the condition for maximum power transfer from a source to the load. |  | CO2 | R | 1 |
| 5. | Write the relation between line voltage and phase voltage in a 3-phase star connected system. |  | CO3 | A | 1 |
| 6. | Predict the laplace transform of u(t). |  | CO3 | U | 1 |
| 7. | Identify the transform impedance of the inductor. |  | CO4 | U | 1 |
| 8. | Predict the effect of quality factor (Q) of a resonant circuit, when the bandwidth is low. |  | CO5 | U | 1 |
| 9. | Indicate the number of links in a co-tree, if there are 4 branches and 3 nodes in an electric circuit. |  | CO6 | U | 1 |
| 10. | Choose a filter that passes frequencies between two designated cut-off frequencies and attenuates all other frequencies. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Calculate the voltage drop across 10Ω resistor in the given circuit. |  | CO1 | A | 3 |
| 12. | State Superposition theorem. |  | CO2 | R | 3 |
| 13. | A three phase balanced star-connected load having an impedance (15+j10)Ω per phase is connected to a 440 V, 50 Hz three-phase supply. Determine the line currents. |  | CO3 | An | 3 |
| 14. | List any three properties of Laplace Transform. |  | CO4 | U | 3 |
| 15. | Identify the scale factor, zeros and poles for the network function. |  | CO5 | U | 3 |
| 16. | Distinguish low pass filter from high pass filter used in communication system. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Apply mesh analysis method to determine the mesh currents in the circuit shown. | CO1 | A | 12 |
| 18. | a. | Determine the voltage across each current source using nodal analysis for the given circuit. | CO2 | A | 12 |
| 19. | a. | A three-phase balanced delta connected load of (3+j4)Ω is connected across a 400V, 3Ø balanced supply. Determine the phase currents, line currents and power factor. Assume the phase sequence to be RYB. Also calculate the power drawn by the load. | CO3 | A | 12 |
| 20. | a. | Determine the steady state response of a series RL and a series RC circuit using Laplace Transform. | CO4 | A | 12 |
| 21. | a. | Estimate the resonant frequency, Q factor, lower cut off frequency, upper cut off frequency and bandwidth of the following circuit specification. A series RLC circuit consists of 40 Ω resistor 0.3 H inductance and 10 μF capacitor with an applied voltage of 30V. | CO5 | E | 12 |
| 22. | a. | Analyze the given circuit to calculate the voltage e0. | CO1 | An | 6 |
|  | b. | Compute the equivalent resistance between the terminals A and B shown in the circuit. | CO1 | A | 6 |
| 23. | a. | Apply thevenin theorem to determine the thevenin equivalent circuit across the terminal AB and the current through 4Ω resistor for the given circuit . | CO2 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Determine the open circuit impedance (Z) parameters of the given network. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand basics electrical circuits with nodal and mesh analysis |
| CO2 | Apply the various electrical network theorems to analyze the circuits and networks. |
| CO3 | Analyze three phase circuits |
| CO4 | Apply Laplace Transform for steady state and transient analysis |
| CO5 | Analyze the frequency domain techniques |
| CO6 | Determine different network functions and Design filter circuits to satisfy design specifications |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 18EE3018 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # POWER QUALITY ISSUES AND MITIGATION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Explain the concept of power quality and discuss its importance in modern industrial systems. | CO1 | U | 10 |
|  | b. | Differentiate between short interruptions and long interruptions in power systems. Discuss their causes and impact on industrial loads. | CO1 | An | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Describe the CBEMA and ITI curves in detail. How are these curves used to assess the tolerance of equipment to voltage disturbances? | CO1 | U | 10 |
|  | b. | Discuss various power quality issues faced by end users. Mention the typical effects of these issues on electrical and electronic equipment. | CO1 | U | 10 |
| 3. | a. | Explain the characteristics of voltage sags and discuss the key parameters used to define and classify them in power systems. | CO2 | U | 10 |
|  | b. | Evaluate various mitigation methods for voltage sags and overvoltage conditions, including lightning protection techniques, and justify the most effective solution for an industrial power system. | CO2 | E | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Analyze the causes and effects of utility capacitor switching transients and compare their impact on industrial equipment with other types of transients. | CO2 | An | 10 |
|  | b. | A three-phase system experiences a voltage sag due to a fault condition. Demonstrate how to calculate the magnitude and duration of the voltage sag using appropriate formulas and assumptions. | CO2 | A | 10 |
| 5. | a. | Analyze different methods used for harmonic distortion evaluation and compare their effectiveness in identifying power quality issues. | CO3 | An | 10 |
|  | b. | Explain waveform distortion in power systems and describe the common harmonic sources from both commercial and industrial loads. | CO3 | U | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Given a power system with nonlinear loads, illustrate the procedure to locate harmonic sources and assess their contribution using suitable techniques. | CO3 | A | 10 |
|  | b. | Evaluate the principles of harmonic control and compare active and passive filters in terms of performance, cost, and suitability for industrial applications. | CO3 | E | 10 |
| 7. | a. | Explain the working principle of DSTATCOM, Dynamic Voltage Restorer (DVR), and Unified Power Quality Conditioner (UPQC) with neat functional descriptions. | CO4 | U | 10 |
|  | b. | For an industrial distribution system experiencing voltage sag and reactive power issues, illustrate how DSTATCOM and DVR can be applied to improve power quality. | CO4 | A | 10 |
|  |  | (OR) |  |  |  |
| 8. |  | Explain the fundamentals of instantaneous reactive power (p–q) theory, instantaneous symmetrical component theory, SRF theory, and DFT theory used in power quality analysis. | CO5 | U | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Illustrate how power quality measurement equipment can be used to identify and record disturbances such as harmonics, flicker, and voltage variations in an industrial system. | CO6 | An | 10 |
|  | b. | Evaluate the process of planning, conducting, and analyzing a power quality survey, and justify the selection of appropriate instruments and methodologies for accurate assessment. | CO6 | E | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recognize the cause and source of power system disturbances. |
| CO2 | Calculate harmonic voltages and currents by analyzing all types of electrical systems loads and their power quality considerations. |
| CO3 | Suggest suitable mitigation scheme for some of the power quality issues. |
| CO4 | Examine the methods of reducing excessive harmonics using advanced modelling technique |
| CO5 | Analyze the power quality issues using the Power quality indices. |
| CO6 | Design load compensators useful for mitigating power quality problems. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 18EE3021 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SMART POWER GRID RENEWABLE ENERGY SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Describe the concept of a resilient and self-healing grid with suitable examples. | CO1 | U | 10 |
|  | b. | Define Smart Grid and discuss its key functions and benefits. | CO1 | A | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Compare conventional grid and smart grid in terms of operation, reliability, and efficiency. | CO1 | U | 10 |
|  | b. | Illustrate the feeder automation and its importance in distribution systems. | CO2 | An | 10 |
| 3. | a. | Describe smart substations and substation automation with suitable block diagrams. | CO2 | U | 10 |
|  | b. | Explain the role of technology drivers in the development of Smart Grid. | CO2 | A | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Describe the role of cloud computing and web services in Smart Grid applications. | CO3 | A | 10 |
|  | b. | Select the role of Wide Area Monitoring Systems (WAMS) in smart grid communication. | CO3 | A | 10 |
| 5. | a. | Illustrate the architecture and importance of LAN, HAN, and WAN in Smart Grid communication. | CO3 | A | 10 |
|  | b. | Paraphrase the working principle and features of smart meters. | CO4 | U | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | State the Advanced Metering Infrastructure (AMI), its functions and benefits. | CO4 | A | 10 |
|  | b. | Evaluate the role of Phasor Measurement Units (PMU) in Smart Grid. | CO4 | A | 10 |
| 7. | a. | State the application of Intelligent Electronic Devices (IEDs) in monitoring and protection. | CO4 | A | 10 |
|  | b. | Illustrate the anti-islanding protection techniques in Smart Grid. | CO5 | An | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Test the power quality conditioner and its role in Smart Grid. | CO5 | An | 10 |
|  | b. | Evaluate shunt compensation using D-STATCOM with suitable diagrams. | CO5 | An | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | List any two renewable energy resources and describe the integration into Smart Grid. | CO6 | E | 10 |
|  | b. | Distinguish Electric Vehicles (EVs) and Plug-in Hybrid Electric Vehicles (PHEVs) connected with Smart Grid. | CO6 | E | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Differentiate the information exchange in traditional grid and smart grid |
| CO2 | Assess the importance of the information security for smart grid |
| CO3 | Demonstrate different Smart Grid communication technologies |
| CO4 | Design the prototype model of the smart grid |
| CO5 | Evaluate the role of power electronic devices in the network |
| CO6 | Analyze the grid integration issues of renewable energy sources |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 18EE3022 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRIC AND HYBRID VEHICLES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Explain different types of Hybrid Electric Vehicles (HEVs) with suitable diagrams and discuss their merits and demerits. | CO1 | U | 10 |
|  | b. | Explain the construction, working principle, and chemical reactions involved in Lead-Acid and Lithium-Ion batteries. | CO1 | U | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Discuss the working principle of fuel cell vehicles and compare them with battery electric vehicles. | CO1 | An | 10 |
|  | b. | Describe the configuration of an electric vehicle with a neat block diagram and explain the function of each component. | CO1 | U | 10 |
| 3. | a. | Explain the construction and operation of a BLDC motor drive with suitable diagrams. | CO2 | A | 10 |
|  | b. | Explain the principle of DC motor drives and analyze their performance characteristics in electric vehicle applications. | CO2 | A | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Explain the construction and operation of a SRM drive with suitable diagrams. | CO2 | U | 10 |
|  | b. | Describe the working principle and steady-state performance of induction motor drives used in EVs. | CO2 | A | 10 |
| 5. | a. | Describe the architecture of electric vehicles and discuss different drivetrain layouts. | CO3 | U | 10 |
|  | b. | Explain the design considerations involved in EV chassis and body structure. | CO3 | E | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Discuss IC engine modeling in hybrid vehicles and its integration with electric drive systems. | CO4 | An | 10 |
|  | b. | Explain battery modeling techniques used in EV simulations. | CO4 | An | 10 |
| 7. | a. | Describe the modeling of DC motor drive and its controller for EV applications. | CO4 | An | 10 |
|  | b. | Explain induction motor drive modeling and controller design for electric vehicles. | CO4 | An | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Explain the hybrid architecture and performance of the BMW i8 | CO5 | U | 10 |
|  | b. | Explain the working and features of fuel cell vehicles using Honda FCX Clarity | CO5 | U | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Explain the procedure for importing and creating driving cycles for EV simulation. | CO6 | An | 10 |
|  | b. | Discuss the steps involved in range simulation of an electric vehicle using GM EV1 as a reference. | CO6 | An | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Realize the need of Hybrid Vehicles and Electric vehicles. |
| CO2 | State different types of drives used in Electric & Hybrid Vehicles. |
| CO3 | Use the energy on-board optimally. |
| CO4 | Understand the merits and demerits of various mathematical models of Electric and hybrid Vehicle. |
| CO5 | Design the EHV using the mathematical Model. |
| CO6 | Simulate and observe the behavior of the EHV. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 20EE2001 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRIC VEHICLE DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Specific power in a battery indicates its ability to deliver ______ efficiently. |  | CO1 | R | 1 |
| 2. | Li Ion battery’s specific energy is about ______times than that of lead acid batteries. |  | CO1 | R | 1 |
| 3. | The Hall sensor in a BLDC motor helps in detecting ______ for precise commutation. |  | CO2 | R | 1 |
| 4. | The synchronous speed of an AC machine, if the no. of poles (P) is 4 and frequency (f) is 50Hz is _____ rpm. |  | CO2 | R | 1 |
| 5. | Increasing the mass of an electric vehicle results in a ______ effect on its range and performance. |  | CO3 | R | 1 |
| 6. | Mention any one popular driving cycles developed. |  | CO3 | R | 1 |
| 7. | The aerodynamic drag coefficient for a hatchback car normally varies from __ to __. |  | CO4 | R | 1 |
| 8. | The force at the rims or the outer edges of the driving wheels of moving trains is called _________ effort. |  | CO4 | R | 1 |
| 9. | Transducers are sometimes referred to as ______ converters. |  | CO5 | R | 1 |
| 10. | _____ is a technique which enables machines to mimic human behavior. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | List the types of batteries used in EV. |  | CO1 | R | 3 |
| 12. | Distinguish BLDC Motor from Brushed DC Motor. |  | CO2 | U | 3 |
| 13. | Highlight the major components in the mechanical design aspects of EV. |  | CO3 | U | 3 |
| 14. | With respect to design of controls, sketch the stick controller. |  | CO4 | U | 3 |
| 15. | Differentiate RADAR from LIDAR. |  | CO5 | U | 3 |
| 16. | List out any four latest EV models. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Examine the Series, Parallel, and Series–Parallel hybrid vehicle configurations and analyze their structural differences, energy flow paths, and performance characteristics using appropriate diagrams. | CO1 | An | 12 |
| 18. | . | Analyze the switching sequence, energy conversion process, and performance characteristics of any one converter employed in a Switched Reluctance Motor drive using appropriate diagrams and waveforms | CO2 | An | 12 |
| 19. |  | Using appropriate diagrams, demonstrate the application of total tractive effort in evaluating the performance characteristics of an electric vehicle | CO3 | A | 12 |
| 20. |  | Examine the design considerations of Electric Vehicles, particularly focusing on aerodynamics, rolling resistance, transmission efficiency, and the influence of vehicle mass. | CO4 | An | 12 |
| 21. |  | Demonstrate the application of autonomous driving technologies by categorizing self-driving cars according to the various levels of automation and citing relevant examples | CO5 | A | 12 |
| 22. |  | Apply the principles of four-quadrant chopper control to illustrate the operation of a DC motor in an electric vehicle under motoring and braking modes, using appropriate diagrams and waveforms | CO2 | A | 12 |
| 23. | a. | Explain the historical evolution of electric vehicles, highlighting the major technological developments and milestones. | CO1 | U | 8 |
|  | b. | List the key characteristics of Lithium-ion and Lithium Polymer batteries | CO1 | R | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze the uniqueness of hybrid EV – Honda Insight with a case study. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Realize the need of Electric vehicles |
| CO2 | State different types of Electric & Hybrid Vehicles |
| CO3 | Use the energy on-board optimally |
| CO4 | Understand the design and mathematical modelling of EV and drives |
| CO5 | Analyze the latest control techniques for vehicle control |
| CO6 | Simulate and observe the behavior of the EV |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # EMBEDDED SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List the components of embedded systems. |  | CO1 | R | 1 |
| 2. | Differentiate unit cost and NRF cost. |  | CO1 | An | 1 |
| 3. | Identify the device that converts physical parameter into electrical signal. |  | CO2 | U | 1 |
| 4. | List the examples of embedded processor. |  | CO2 | R | 1 |
| 5. | Name the hardware device used to debug the software of an embedded system. |  | CO3 | R | 1 |
| 6. | Identify the unit that fetches instructions from memory. |  | CO3 | U | 1 |
| 7. | Differentiate compiler and assembler. |  | CO4 | An | 1 |
| 8. | Define embedded system. |  | CO4 | R | 1 |
| 9. | Name an input device used to control the operation of an output device. |  | CO5 | R | 1 |
| 10. | List two software tools of embedded systems. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Illustrate the building blocks of an embedded system. |  | CO1 | An | 3 |
| 12. | Describe the functions of RTOS. |  | CO2 | U | 3 |
| 13. | Explain the procedure of interfacing an LED with a microcontroller. |  | CO3 | An | 3 |
| 14. | Analyze the significance of software development kit. |  | CO4 | An | 3 |
| 15. | Explain the significant features of the target system board. |  | CO5 | An | 3 |
| 16. | Differentiate functional description and logic description. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | A temperature-controlled fan is designed to automatically adjust its speed based on room temperature using sensors and a microcontroller. Describe the development process of the embedded system used in this application. | CO1 | U | 6 |
|  | b. | A digital signal processing unit uses an ARM processor to fetch input data from memory and perform computations. Describe the ARM addressing modes involved in data access and processing. | CO1 | U | 6 |
| 18. | a. | A smart inverter uses an ARM processor for real-time monitoring and control of voltage and current parameters. Explain the ARM architecture that supports this application. | CO2 | An | 6 |
|  | b. | Analyze the modes of ARM Processor. | CO2 | An | 6 |
| 19. |  | A smart inverter uses an ARM processor to monitor voltage/current, move data between memory and registers, and control switching operations. Describe the instruction set of the ARM processor that supports these functions. | CO3 | U | 12 |
| 20. | a. | An ARM-based signal processing unit switches execution states to balance speed and memory efficiency. Describe the processor states used in this operation. | CO4 | U | 10 |
|  | b. | Differentiate data transfer instructions and control flow instructions. | CO4 | An | 2 |
| 21. | a. | Explain the memory management and virtual memory system of the ARM processor. | CO5 | An | 10 |
|  | b. | Analyze the architecture of cache memory in the ARM Processor. | CO5 | An | 2 |
| 22. | a. | Illustrate the key features of Free RTOS with relevant examples. | CO4 | An | 10 |
|  | b. | Explain the control bits of ARM Processor. | CO4 | A | 2 |
| 23. | a. | A smart energy meter is developed by dividing the system into functional blocks and also by integrating individual modules. Analyze the top-down and bottom-up approaches used in this system design. | CO5 | An | 6 |
|  | b. | Analyze the typical co-design process in embedded systems. | CO5 | An | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | An embedded system is designed to continuously monitor environmental temperature using an LM35 sensor and process the data using an ARM7 LPC2148 microcontroller. Illustrate the interfacing of the LM35 sensor with the ARM7 LPC2148 microcontroller in this system. | CO6 | An | 6 |
|  | b. | An industrial control system connects multiple sensors and controllers over a long distance in a noisy environment. Another system connects a single device over a short distance. Explain RS-232 and RS-485 suitable for these applications. | CO6 | An | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Manipulate the principles of embedded systems design to engineer innovative solutions for complex engineering challenges. |
| CO2 | Utilize practical experimentation and testing methodologies to evaluate the performance of embedded systems and ARM processors. |
| CO3 | Implement advanced programming techniques to develop software for embedded systems, employing real-time operating system (RTOS) concepts, task scheduling algorithms, and memory management. |
| CO4 | Demonstrate comprehensive analysis techniques to assess the interaction between hardware and software components in embedded systems. |
| CO5 | Employ a thorough understanding of communication protocols and interfaces to design embedded systems. |
| CO6 | Implement cutting-edge technologies and industry best practices to design embedded systems that address evolving. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2008 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRICAL MACHINES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List two applications of DC generators. |  | CO1 | R | 1 |
| 2. | Indicate the function of the commutator in a DC generator. |  | CO1 | U | 1 |
| 3. | Predict the change in speed of a DC motor when the flux is reduced. |  | CO2 | A | 1 |
| 4. | Indicate the torque equation of a DC motor. |  | CO2 | U | 1 |
| 5. | State the principle of operation of a transformer. |  | CO3 | R | 1 |
| 6. | Define voltage regulation. |  | CO3 | R | 1 |
| 7. | Indicate the formula for calculating synchronous speed. |  | CO4 | U | 1 |
| 8. | Define hunting in synchronous motors. |  | CO4 | R | 1 |
| 9. | Write the applications of a squirrel cage induction motor. |  | CO5 | A | 1 |
| 10. | Identify the motor used in robotics applications. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Discuss the change in terminal voltage when the load increases or decreases. |  | CO1 | U | 3 |
| 12. | Compute the torque when the armature current is doubled, and the flux constant. |  | CO2 | A | 3 |
| 13. | Determine the efficiency of a distribution transformer supplying power to residential loads when the input power is 1200 W, copper loss is 100 W, and iron loss is 50 W. |  | CO3 | A | 3 |
| 14. | List the conditions required for synchronization of an alternator. |  | CO4 | R | 3 |
| 15. | Illustrate the torque-slip characteristics of an induction motor |  | CO5 | U | 3 |
| 16. | Predict the performance of a servo motor in control systems with reference to its applications. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the constructional features, working principle, and EMF equation of a DC generator with a diagram. | CO1 | U | 12 |
| 18. |  | Evaluate testing methods (Swinburne’s and Hopkinson’s) and the efficiency of DC motors. | CO2 | E | 12 |
| 19. |  | Describe the construction and working principle of a single-phase transformer used in a power distribution system. | CO3 | U | 12 |
| 20. |  | Analyse the operation, starting methods, and characteristics of synchronous motors in applications such as industrial drives and power factor correction. | CO4 | An | 12 |
| 21. |  | Evaluate the starting and speed control methods of induction motors in applications like cranes and electric traction systems. | CO5 | E | 12 |
| 22. | a. | Analyze the different types of losses in a transformer and evaluate its efficiency under varying load conditions. | CO3 | An | 6 |
|  | b. | Evaluate the working principle and performance of special transformers in industrial systems. | CO3 | E | 6 |
| 23. | a. | Explain the working principle and construction of DC motor with a relevant diagram. | CO2 | U | 8 |
|  | b. | Examine the construction and working of slip ring induction motors used in industrial drive applications. | CO5 | U | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Assess the constructional features and working principles of stepper and BLDC motors, and justify their suitability for precision control applications with relevant diagrams. | CO6 | E | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Illustrate the operation of DC Machines. |
| CO2 | Identify the differences in operation of different DC machine configurations. |
| CO3 | Examine single phase and three phase transformers circuits. |
| CO4 | Outline the working of autotransformers. |
| CO5 | Analyse the effect of parameter variation on torque of Induction Motor and Identify suitable starting, speed control and braking methods for Induction Motor. |
| CO6 | Comprehend the operation of various types of induction motor and Synchronous motor |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # POWER ELECTRONICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Name the terminals of a TRIAC |  | CO1 | R | 1 |
| 2. | The device most suited for its high switching frequency operation is _______. |  | CO1 | R | 1 |
| 3. | The expression for average output voltage of single phase full-controlled converter for resistive load is given by Vavg = |  | CO2 | R | 1 |
| 4. | _______ is used in converters to remove the negative voltage in the output and to maintain load current continuous. |  | CO2 | R | 1 |
| 5. | Name any application of AC voltage controller. |  | CO3 | R | 1 |
| 6. | A cycloconverter is also termed as ______________ converter. |  | CO3 | R | 1 |
| 7. | ______ type of commutation is used in DC choppers. |  | CO4 | R | 1 |
| 8. | Abbreviate UPS. |  | CO4 | R | 1 |
| 9. | An Inverter Circuit converts ______ DC to _____ AC. |  | CO5 | R | 1 |
| 10. | A _______ converter directly converts AC to AC without an intermediate DC stage. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare the power electronic devices, SCR and TRIAC |  | CO1 | U | 3 |
| 12. | Differentiate single phase semi and full converter. |  | CO2 | U | 3 |
| 13. | Sketch the circuit diagram of single phase cycloconverter. |  | CO3 | U | 3 |
| 14. | Mention the control strategies of DC chopper circuits. |  | CO4 | U | 3 |
| 15. | Define PWM and mention its advantages. |  | CO5 | U | 3 |
| 16. | Mention the key features of a Super-Lift Luo Converter. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Interpret the reverse recovery characteristics of a Power Diode with neat diagram. | CO1 | An | 8 |
|  | b. | Compare Power MOSFET with Power BJT | CO1 | U | 4 |
| 18. |  | Investigate the working of a single-phase full converter with neat diagram and waveforms. | CO2 | An | 12 |
| 19. |  | Interpret the operation of a single-phase AC to AC Converter with voltage control using neat diagram and waveforms. | CO3 | An | 12 |
| 20. |  | Examine the operation of a four-quadrant chopper with appropriate diagrams and waveforms, and evaluate its impact on motor performance. | CO4 | An | 12 |
| 21. |  | Examine the 120-degree mode operation of an inverter with neat diagram and waveforms. | CO5 | An | 12 |
| 22. |  | Examine the switching characteristics of a Power BJT using neat diagrams. | CO1 | An | 12 |
| 23. | a. | List the applications of AC voltage controller and Cycoconverter | CO3 | U | 8 |
|  | b. | A DC chopper has input voltage of 24V and resistance of R = 20Ω. Find the average output voltage during step-down and step-up mode when duty ratio is 0.4. | CO4 | A | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Examine the role of power electronics in enhancing the efficiency of wind power generation system, with neat diagram. | CO6 | An | 8 |
|  | b. | Mention any 4 modern power converters used in the power sector. | CO6 | U | 4 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the switching characteristics of power devices and select a suitable power device for power conversion. |
| CO2 | Design a power converter with criteria (power, efficiency, ripple voltage and current, harmonic distortions, power factor). |
| CO3 | Implement and verify the performance characteristics of power converters. |
| CO4 | Interpret terminal characteristics of the components for designing the circuitry for power converters. |
| CO5 | Estimate the required converters for renewable based applications. |
| CO6 | Assess the quality of power by analyzing the factors such as harmonics, ripples, etc., |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # LINEAR INTEGRATED CIRCUITS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State the purpose of Pins 4 and 7 in op-amp IC 741. |  | CO1 | R | 1 |
| 2. | Define slew rate. |  | CO1 | R | 1 |
| 3. | Recall the relationship between output and input for a voltage follower. |  | CO2 | R | 1 |
| 4. | An inverting summing amplifier has Rf=10k, input resistors R1=R2=10kΩ. If V1=2V and V2=3V, find V0. |  | CO2 | U | 1 |
| 5. | Why is an monostable multivibrator called "one shot"? |  | CO3 | U | 1 |
| 6. | A Wien bridge oscillator has R=10kΩ, C=10nF in its lead-lag network. Calculate the frequency of oscillation. |  | CO3 | A | 1 |
| 7. | What is the relationship between the capacitor's impedance Xc and frequency in an active filter? |  | CO4 | U | 1 |
| 8. | Which pin is known as the reset pin in a 555 timer? |  | CO5 | R | 1 |
| 9. | Which regulator would you choose to step up a 5V input to 12V? |  | CO5 | A | 1 |
| 10. | Why is the R-2R ladder DAC preferred over the weighted resistor DAC for integrated circuits? |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | An op-amp has a differential gain of 105 and a common-mode gain of 0.2. Calculate the CMRR in dB. |  | CO2 | A | 3 |
| 12. | Describe the output waveform of a half-wave rectifier with a resistive load. |  | CO2 | U | 3 |
| 13. | Why an RC phase shift oscillator requires a three-stage RC network and an inverting amplifier to satisfy Barkhausen criteria? Explain. |  | CO3 | U | 3 |
| 14. | Draw a wide band pass filter using high pass filter and low pass filter. |  | CO4 | U | 3 |
| 15. | Compare 7805 voltage regulator and 7905 voltage regulator. |  | CO5 | An | 3 |
| 16. | Calculate the output voltage for a 4-bit DAC with a reference voltage of 5V and input (1011)2. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | List and explain the DC characteristics of an operational amplifier. | CO1 | U | 8 |
|  | b. | Compare the ideal and practical characteristics of IC741. | CO1 | An | 4 |
| 18. |  | Sketch the circuit diagram of inverting and non inverting amplifiers using op-amp and derive the expression for closed loop voltage gain. Draw the necessary waveforms. | CO2 | A | 12 |
| 19. | a. | Draw the circuit diagram and explain the operation of op-amp differentiator. Derive the expression for the output voltage V0 in terms of the input voltage Vi. | CO2 | U | 6 |
|  | b. | Explain the working of non inverting comparator with positive voltage reference. | CO2 | U | 6 |
| 20. | a. | With a neat circuit diagram, explain the operation of an astable multivibrator using op-amp and draw the output voltage waveforms. | CO3 | U | 6 |
|  | b. | Calculate the resistor value of wein bridge oscillator for the operating frequency of 200kHz and for the value of capacitor 10 pF. | CO3 | A | 6 |
| 21. | a. | Classify the filters based on their frequency response. | CO4 | U | 2 |
|  | b. | Construct a second-order band pass filter with a center frequency of 2kHz and a gain of 10. | CO4 | C | 10 |
| 22. |  | Explain the internal structure of voltage regulator IC 723. Also explain its working as low voltage regulator circuit. | CO5 | An | 12 |
| 23. | a. | Design a 4-bit Successive Approximation Register ADC circuit, including the necessary logic to implement the binary search algorithm. | CO6 | C | 10 |
|  | b. | Mention any 2 applications of PLL. | CO6 | R | 2 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | With neat diagram, explain the working principle of flash type ADC. | CO6 | U | 8 |
|  | b. | Define i) capture range in PLL ii) Lock in range in PLL. | CO6 | R | 4 |

CO – COURSE OUTCOME	BL – BLOOM’S LEVELM – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the fundamentals of OP-AMP and its characteristics. |
| CO2 | Use OP-AMP to design circuits such as Amplifiers, differentiator and Integrator. |
| CO3 | Infer the significance of OP-AMP in multivibrators and IC voltage regulators. |
| CO4 | Design active filters using OP-AMP and power amplifiers. |
| CO5 | Explore the Timer, pulse generation, and oscillator applications of IC555 timer. |
| CO6 | Understand the fundamentals of ADCs, DACs and Phase Locked Loops. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2011 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRIC POWER TRANSMISSION AND DISTRIBUTION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Select the suitable transmission system for delivering power over very long distances. |  | CO1 | U | 1 |
| 2. | Recognize the component responsible for delivering power directly to consumers. |  | CO1 | R | 1 |
| 3. | Identify the parameter responsible for power loss in transmission lines. |  | CO2 | R | 1 |
| 4. | State the factor that causes voltage drop along transmission lines. |  | CO2 | R | 1 |
| 5. | Identify the component used to control reactive power in transmission lines. |  | CO3 | U | 1 |
| 6. | Recognize the advantage of HVDC transmission for underwater cables. |  | CO3 | R | 1 |
| 7. | Name any two factors affecting voltage drop in distribution lines. |  | CO4 | U | 1 |
| 8. | State the situation that results in lower string efficiency in overhead transmission lines. |  | CO4 | R | 1 |
| 9. | Name the regulation governing electricity supply in India. |  | CO5 | R | 1 |
| 10. | State the unit of insulation resistance. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Distinguish between feeder and distributor using a practical distribution example. |  | CO1 | An | 3 |
| 12. | Relate resistance of a transmission line to power loss and efficiency. |  | CO2 | U | 3 |
| 13. | Distinguish between Ferranti effect and corona effect. |  | CO3 | An | 3 |
| 14. | Classify substations based on voltage level. |  | CO4 | U | 3 |
| 15. | Compare suspension and strain insulators. |  | CO5 | An | 3 |
| 16. | Identify the application of energy storage systems. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Compare radial, ring main, and interconnected distribution systems with respect to reliability, cost, and flexibility with suitable schematic diagram. | CO1 | U | 6 |
|  | b. | Investigate the key steps involved in transmission line construction, covering site selection, tower erection, conductor stringing, and safety considerations. | CO1 | An | 6 |
| 18. | a. | Explain the function of any three EHV AC Substation Components. | CO2 | U | 6 |
|  | b. | Describe the major constraints involved in planning and constructing overhead transmission lines. | CO2 | U | 6 |
| 19. | a. | Illustrate about Nominal T method and Nominal π method. | CO3 | A | 6 |
|  | b. | Describe the structure of pole mount substation with neat diagram. | CO3 | U | 6 |
| 20. |  | An overhead 3-phase transmission line delivers 5000 kW at 22 kV at 0·8 p.f. lagging. The resistance and reactance of each conductor is 4 Ω and 6 Ω respectively. Determine: (i) sending end voltage (ii) percentage regulation (iii) transmission efficiency. | CO4 | A | 12 |
| 21. | a. | Describe about Resonance Inductive Coupling method used in Wireless Power Transmission (WPT) with suitable block diagram | CO4 | U | 6 |
|  | b. | Analyze the performance differences between AC and DC transmission. | CO4 | An | 6 |
| 22. | a. | Assess the effectiveness of various insulation resistance testing methods, including spot testing, time-resistance testing, and polarization index testing, in diagnosing insulation health. | CO5 | E | 6 |
|  | b. | A 4-disc suspension insulator has voltages across each disc as 30 kV, 27 kV, 25 kV, and 24 kV (from top to bottom). Calculate the string efficiency. | CO5 | A | 6 |
| 23. | a. | Explain the structure and key component of a microgrid. | CO6 | U | 6 |
|  | b. | Analyze the classification and construction of underground cables. | CO5 | An | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Analyze the impact of large-scale solar and wind power penetration on the stability and reliability of conventional power systems. | CO6 | An | 6 |
|  | b. | Describe the components and operating mechanism of an SMES system with a suitable schematic representation. | CO6 | U | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recall the standard operation of electric transmission and distribution systems. |
| CO2 | Describe the components and operation of low voltage AC distribution systems. |
| CO3 | Analyze and interpret the functioning of extra high voltage transmission systems. |
| CO4 | Evaluate the reliability and performance of low voltage AC distribution systems |
| CO5 | Construct innovative approaches to enhance the reliability of transmission and distribution line components |
| CO6 | Recall the standard operation of electric transmission and distribution systems. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2023 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRIC AND HYBRID VEHICLE | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | A fuel cell is an electrochemical cell that converts _____ energy of a fuel and an oxidizing agent through a pair of redox reactions. |  | CO1 | R | 1 |
| 2. | Li-Ion battery’s specific energy is about ______times than that of lead acid batteries. |  | CO1 | R | 1 |
| 3. | State the power factor,in which Switched Reluctance motor operates. |  | CO2 | R | 1 |
| 4. | Is Regenerative braking possible in a series motor. |  | CO2 | R | 1 |
| 5. | State quadrant of electronic controller is used to control the motor, where the 1st quadrant for forward power and the 2nd for regenerative braking. |  | CO3 | R | 1 |
| 6. | State the expression for the effect of rolling resistance is (Frr) on a vehicle |  | CO3 | R | 1 |
| 7. | Cite the motor technology which can minimize or even eliminate the mechanical components of an electric powertrain system. |  | CO4 | U | 1 |
| 8. | Name the force at the rims or the outer edges of the driving wheels of moving trains. |  | CO4 | R | 1 |
| 9. | State the full form of the Popular Driving Cycle ‘FUDS’ |  | CO5 | R | 1 |
| 10. | State the technique which enables machines to mimic human behavior. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Distinguish Electric and IC Engine vehicles with any 3 points. |  | CO1 | U | 3 |
| 12. | Differentiate BLDC Motor with Induction Motor. |  | CO2 | U | 3 |
| 13. | An electric car of mass 1500 kg is moving on a level road at a speed of 100 km/h. If the coefficient of rolling resistance is 0.015, calculate the rolling resistance force acting on the vehicle and the power required to overcome it. Take g = 9.81 m/s². |  | CO3 | A | 3 |
| 14. | Mention the types of battery chargers for EV. |  | CO4 | U | 3 |
| 15. | Highlight any 3 popular sensors used in self-driving cars. |  | CO5 | U | 3 |
| 16. | List out any 3 popular 4-wheeler EV models. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Examine any two hybrid configurations of hybrid vehicles with necessary diagrams. | CO1 | An | 12 |
| 18. |  | Illustrate the operation of a converter for Switched Reluctance Motor (SRM) with neat diagram and waveforms. | CO2 | An | 12 |
| 19. |  | Illustrate the influence of aerodynamic drag, rolling resistance, transmission efficiency, and vehicle mass on the design considerations of an electric vehicle with necessary diagrams. | CO3 | An | 12 |
| 20. |  | Obtain the mathematical model (electrical and mechanical) of a DC motor with necessary diagrams and equations. | CO4 | An | 12 |
| 21. |  | Apply AI-based and conventional PI control techniques to an electric vehicle system and compare their performance with necessary diagrams. | CO5 | A | 12 |
| 22. |  | Explain the working of fuel-cell with necessary diagrams. Also mention its applications | CO1 | U | 12 |
| 23. | a. | Explain any electric drive train configuration with neat diagram. | CO4 | U | 6 |
|  | b. | Sketch the wireless charging infrastructure for EV | CO5 | U | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Evaluate the case study of the GM EV1 with respect to its electric architecture and performance. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Realize the need of Electric vehicles |
| CO2 | State different types of Electric & Hybrid Vehicles |
| CO3 | Use the energy on-board optimally |
| CO4 | Understand the design and mathematical modelling of EV and drives |
| CO5 | Analyze the latest control techniques for vehicle control |
| CO6 | Simulate and observe the behavior of the EV |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2024 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SOLAR AND WIND ENERGY SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Classify the components of solar radiation reaching the Earth’s surface. |  | CO1 | U | 1 |
| 2. | Define extraterrestrial solar radiation. |  | CO1 | R | 1 |
| 3. | State the function of Maximum Power Point Tracking (MPPT). |  | CO2 | U | 1 |
| 4. | Recognize the function of glazing in flat plate collectors. |  | CO2 | U | 1 |
| 5. | Identify the factor influencing wind power output most significantly. |  | CO3 | U | 1 |
| 6. | Define the Betz limit in wind energy systems. |  | CO3 | R | 1 |
| 7. | State the purpose of hybrid solar–wind systems. |  | CO4 | U | 1 |
| 8. | Identify the function of STATCOM in power quality improvement. |  | CO4 | R | 1 |
| 9. | Relate the function of inverter in grid-connected renewable systems. |  | CO5 | U | 1 |
| 10. | Indicate the role of energy storage in maintaining power quality. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Analyze the variation of solar radiation with latitude and atmospheric conditions. |  | CO1 | An | 3 |
| 12. | Classify wind turbines based on axis orientation. |  | CO2 | U | 3 |
| 13. | Compare flat plate collectors and concentrating collectors based on design and performance. |  | CO3 | An | 3 |
| 14. | Compare different types of generators used in wind energy systems. |  | CO4 | U | 3 |
| 15. | Analyze the economic considerations involved in hybrid renewable systems. |  | CO5 | An | 3 |
| 16. | Explain the major power quality issues in renewable energy systems. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | A wind–solar hybrid system is installed to supply power to a remote load. The wind turbine operates with an air density of 1.225 kg/m³, rotor swept area of 10 m², and wind speed of 6 m/s. At the same time, the solar PV array receives solar radiation of 800 W/m² over an effective panel area of 2 m², and the PV conversion efficiency is 15%. Using the wind power expression determine the power available from the wind turbine, the electrical power produced by the solar PV array, and the total renewable power available from the hybrid system. | CO1 | E | 12 |
| 18. | a. | Explain the construction and working of flat plate collectors with heat transfer analysis. | CO2 | U | 8 |
|  | b. | Evaluate the effectiveness of MPPT techniques in solar PV systems. | CO2 | E | 4 |
| 19. |  | Recommend and develop a simple hybrid solar–wind system configuration for rural electrification. | CO3 | E | 12 |
| 20. | a. | Compare line-focus and point-focus solar concentrators with respect to design and applications. | CO3 | R | 8 |
|  | b. | Describe the key challenges and limitations of wind energy systems. | CO4 | U | 4 |
| 21. | a. | Sketch and explain the components of a wind power plant. | CO5 | A | 8 |
|  | b. | Evaluate the role of energy storage in improving hybrid system reliability. | CO4 | U | 4 |
| 22. |  | A solar collector delivers a useful heat output of 1000 W when exposed to solar radiation of 750 W/m² over a surface area of 2.5 m². The temperature difference between the absorber plate and ambient air is 25°C, and the overall heat loss coefficient is 4 W/m²K. Determine the absorbed solar flux and collector efficiency. | CO4 | E | 12 |
| 23. | a. | Explain power quality issues in renewable energy systems. | CO5 | U | 6 |
|  | b. | Analyze the impact of power quality problems on distributed generation systems. | CO5 | An | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Case Study: A remote coastal village is powered by a hybrid renewable energy system consisting of solar photovoltaic panels, a small wind turbine, and battery storage. The system normally operates in a standalone mode as it is not connected to the main utility grid. Recently, a nearby feeder line has been extended, allowing occasional connection to the grid. However, due to frequent grid failures, the system often operates in an islanded mode. During operation, engineers observed issues such as voltage fluctuations, frequency deviations, and improper load sharing between sources when switching between grid-connected and islanded modes. Suggest suitable control strategies to ensure smooth transition between grid-connected and islanded modes and recommend methods to improve reliability and power quality in standalone operation. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basics of wind energy, wind turbines, solar energy and grid integration. |
| CO2 | Explain and classify wind turbines, instruments for measuring solar radiation, solar collectors, solar cell and solar MPPT techniques. |
| CO3 | Analyze different types of wind generators, solar cell and solar collectors. |
| CO4 | Understand about integration of solar and wind energy systems. |
| CO5 | Demonstrate the Solar Thermal and PV applications along with its storage systems. |
| CO6 | Evaluate and mitigate power quality issues associated with the integration of renewable energy resources in both stand-alone and grid-connected systems. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2025 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # FOUNDATION OF MACHINE LEARNING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | A matrix with n columns is having a rank r. Find the number of columns which are linearly dependent. |  | CO2 | U | 1 |
| 2. | Interpret singular value in the context of data matrix. |  | CO2 | U | 1 |
| 3. | Define overfitting in machine learning. |  | CO1 | U | 1 |
| 4. | State bias in the learning process. |  | CO1 | U | 1 |
| 5. | List the compression methods used in JPEG and PNG images. |  | CO3 | R | 1 |
| 6. | Encode the following black and white image. | CO3 | U | 1 |
| 7. | Define the classification task mathematically. |  | CO4 | U | 1 |
| 8. | Recall the purpose of threshold value in a threshold classifier. |  | CO4 | U | 1 |
| 9. | Define a leaf node in decision tree. |  | CO5 | U | 1 |
| 10. | Compare clustering with classification in terms of data labeling. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Motor vibration signals are measured using a sensor. The magnitude of the vibration in mV are given as {3.2, 3.5, 3.1, 3.3, 3.4, 3.2, 9.5}. Find IQR and identify the possible outliers. |  | CO2 | An | 3 |
| 12. | For each of the following tasks, identify whether the machine-learning problem is Regression, Classification, or Clustering. i) Predicting the remaining battery life (in hours) of an Electric Vehicle ii) Grouping solar panels in a Photovoltaic Power Plant based on their power output patterns iii) Determining whether a Wind Turbine is operating under normal or faulty condition |  | CO1 | An | 3 |
| 13. | Consider the three words related to electrical circuits: “Resistor”, “Capacitor”, and “Inductor”. Represent each word using one-hot encoding. Mention the usefulness of this encoding in machine learning. |  | CO3 | An | 3 |
| 14. | In an electronic system, a rectangular classifier is used to detect a valid signal based on two parameters: voltage V and current I. A signal is classified as VALID if 2 ≤ V ≤ 6 and 1 ≤ I ≤ 4 ; otherwise, it is INVALID. Classify the following signals: - (V=3V,I=2A) - (V=7V,I=3A) - (V=5V,I=5A) |  | CO4 | A | 3 |
| 15. | Derive the expression for the margin of a linear SVM hyperplane. |  | CO5 | An | 3 |
| 16. | Apply the Expectation-Maximization (EM) algorithm to cluster agricultural field data based on soil characteristics. Two clusters represent soil types in a farmland based on soil moisture level (%): - Cluster C1 (Dry Soil): Mean = 10, Variance = 4 - Cluster C2 (Wet Soil): Mean = 20, Variance = 4 A new soil sample has moisture value X=12. Assume equal prior probabilities for both clusters. Apply the EM clustering process to: - Determine which cluster has higher likelihood in the E-step - Assign the soil sample to the appropriate cluster - Interpret how the M-step updates the cluster parameters after assignment |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | An electrical power distribution company wants to predict transformer failures using sensor data such as temperature, load current, oil level, and vibration readings collected from substations. The company plans to build a Machine Learning pipeline to automate data processing, model training, and deployment for predictive maintenance. Propose a ML pipeline architecture for real-time monitoring of the transformer. | CO1 | C | 12 |
| 18. | a. | A dataset used for training the EV battery model is very large and cannot be processed at once. Apply suitable gradient-based optimization techniques and explain how Stochastic Gradient Descent or Mini-batch Gradient Descent can be used to train the model efficiently. | CO2 | A | 6 |
|  | b. | A wind farm maintenance team records the occurrence of blade faults and gearbox faults in 100 turbines as shown in the table. Find i) the joint probability that a turbine has both blade fault and gearbox fault. ii) the conditional probability that a turbine has a gearbox fault given that it has a blade fault. iii) Find the conditional probability that a turbine has a blade fault given that it has a gearbox fault. Gearbox Fault-yes; Gearbox Fault-no; Total. Blade Fault; 15; 25; 40. No Blade Fault; 10; 50; 60. Total; 25; 75; 100. | CO2 | An | 6 |
| 19. |  | With neat schematics, discuss the learning process adopted in supervised learning. Clearly indicate the learning stages. | CO1 | An | 12 |
| 20. |  | For the given documents, apply Bag of Words (BoW) method to find the numerical vectors. Document1: The motor consumes electrical power and the motor delivers power to the load Document2: The generator produces electrical power and generator supplies power to the grid. | CO3 | A | 12 |
| 21. |  | In a power electronics system, a matching prototype classifier is used to identify the operating condition of a converter based on two features: output voltage (V) and current (I). Four prototype classes are defined as follows: - P1=(5V,1A) → Normal operation - P2=(8V,2A) → Over-voltage condition - P3=(3V,0.5A) → Under-voltage condition - P4=(6V,3A) → Over-current condition A new observations are X1 = (7V, 1.5A) and X2 = (4V, 3A). Determine which prototype each observation matches. Analyze the result and identify the operating condition of the system for both the cases. | CO4 | An | 12 |
| 22. |  | A Naive Bayes classifier is used to predict whether an Electronic Component (EC) is Faulty or Non-Faulty based on three attributes. The dataset is: Temperature; Voltage Level; Inspection Score; Class. Low; High; Good; Non-Faulty. Low; Low; Good; Faulty. Medium; High; Poor; Non-Faulty. Medium; Low; Good; Non-Faulty. High; High; Poor; Non-Faulty. High; Low; Poor; Faulty. Low; High; Poor; Non-Faulty. Medium; High; Good; Non-Faulty. High; High; Good; Faulty. Low; Low; Poor; Faulty. Using Naive Bayes, classify a new EC with attributes: Temperature = Medium, Voltage Level = Low, Inspection Score=Poor Analyze which class is more likely and explain the reason. | CO5 | An | 12 |
| 23. |  | Apply the steps of the K-means clustering algorithm to explain how data points are grouped into clusters. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | A wind turbine monitoring system collects data on three attributes. The system records whether a turbine is in Fault or No Fault condition as shown below: Wind Speed; Rotor Vibration; Temperature; Class. Low; Low; Normal; No Fault. Low; High; Normal; Fault. Medium; Low; Normal; No Fault. Medium; High; High; Fault. High; High; High; Fault. High; Low; Normal; No Fault. Using Gini Index, determine the best attribute for the root node. Show all calculations. Construct the first split of the decision tree using the selected attribute. Analyze why this attribute provides the purest split compared to the other attributes. | CO5 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand different types of machine learning algorithms |
| CO2 | Apply probability, linear algebra and optimization concepts for machine learning. |
| CO3 | Encode text and image data for machine learning applications |
| CO4 | Analyze simple machine learning algorithms. |
| CO5 | Explain the underlying concepts of supervised machine learning algorithms |
| CO6 | Solve data grouping problems using clustering algorithms |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2026 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # MODERN ELECTRICAL MACHINES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define electric propulsion in EVs. |  | CO1 | R | 1 |
| 2. | Infer the term regenerative braking. |  | CO1 | U | 1 |
| 3. | Recall the step angle in a stepper motor. |  | CO2 | R | 1 |
| 4. | Mention one application of stepper motors. |  | CO2 | R | 1 |
| 5. | State the basic principle of operation of an SRM. |  | CO3 | U | 1 |
| 6. | Name any two types of permanent magnets used in BLDC motors. |  | CO3 | R | 1 |
| 7. | Explain speed control in BLDC motors. |  | CO4 | R | 1 |
| 8. | Tell about synchronous speed. |  | CO5 | R | 1 |
| 9. | Identify the function of a power controller in PMSM drives. |  | CO5 | U | 1 |
| 10. | Write about magnetic levitation (Maglev). |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare BLDC motor with induction motor used in EVs. |  | CO1 | U | 1 |
| 12. | Sketch the torque characteristics of a stepper motor. |  | CO2 | U | 2 |
| 13. | Elucidate the concept of sensorless operation in SRM and its advantages. |  | CO3 | A | 3 |
| 14. | Discuss the characteristics of BLDC motors. |  | CO4 | U | 4 |
| 15. | Draw the phasor diagram of PMSM. |  | CO5 | U | 5 |
| 16. | Examine the concept of Halbach array motors. |  | CO6 | A | 6 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Discuss the different types of electric motors used in electric vehicles with their advantages and limitations. | CO1 | U | 12 |
| 18. |  | Explain the construction and principle of operation of a stepper motor with a neat diagram. | CO2 | U | 12 |
| 19. | a. | Explore the drive circuits used for stepper motors and their operation. | CO2 | U | 6 |
|  | b. | Illustrate the concept of closed-loop control in stepper motors and its advantages. | CO2 | U | 6 |
| 20. |  | A SRM with 6 stator poles and 4 rotor poles has a stator pole arc of 30o and rotor pole arc is 32o. The aligned inductance is 10.7mH and unaligned inductance is 1.5mH. Saturation can be neglected. Calculate the instantaneous torque when the rotor is 30o before the aligned position and phase current is 6A. What is the maximum energy conversion for one stroke, if the current is limited to 7A? Determine the average torque corresponding to this energy conversion. | CO3 | A | 12 |
| 21. |  | Examine the role of power controllers and describe the control techniques used in Switched Reluctance Motor (SRM) drives. | CO3 | A | 12 |
| 22. |  | Derive the EMF and torque equations of a BLDC motor. | CO4 | A | 12 |
| 23. |  | Demonstrate the construction and principle of operation of a Permanent Magnet Synchronous Motor with a neat diagram. | CO5 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Analyze the regulatory and policy implications for the development and adoption of future motor technologies. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Select an energy efficient linear or rotary motor based on the characteristics of the load & application. |
| CO2 | Incorporate the correct control technique to the Stepper motor for efficient operation. |
| CO3 | Analyze the behavior of the SRM Motors for the applied control technique. |
| CO4 | Improve the performance of the PMBLDC by enhancing the motor suitably. |
| CO5 | Explain the theory of traveling magnetic field and applications of PMSM. |
| CO6 | Understand the significance of electrical motors for Future drives. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23EE2031 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # OBJECT ORIENTED PROGRAMMING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | The operator used to extract input from the keyboard is ________. |  | CO1 | R | 1 |
| 2. | The header file must be included for basic input/output operations in C++ is _____ |  | CO1 | R | 1 |
| 3. | In C++, _______ is a process in which a function calls itself to solve a problem. |  | CO2 | R | 1 |
| 4. | A function defined inside another class and allowed to access private members is called a ________ function. |  | CO2 | R | 1 |
| 5. | The process of defining more than one function with the same name but different parameters in C++ is called _______ |  | CO3 | R | 1 |
| 6. | The keyword used to declare a constant object is ________. |  | CO3 | R | 1 |
| 7. | ______ is the process by which new classes called derived classes are created from existing classes called base classes. |  | CO4 | R | 1 |
| 8. | The process of redefining operators for user-defined types is called ________. |  | CO4 | R | 1 |
| 9. | The pointer that refers to the current object is called ________. |  | CO5 | R | 1 |
| 10. | The class used to write data into a file is ________. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate between signed and unsigned data types in C++. |  | CO1 | U | 3 |
| 12. | Mention the branching and looping statements in C++ |  | CO2 | U | 3 |
| 13. | List the key features of constructors in C++ |  | CO3 | U | 3 |
| 14. | Highlight the difference between Operator Functions and Normal Functions |  | CO4 | U | 3 |
| 15. | Differentiate between arrays and pointers in C++. |  | CO5 | U | 3 |
| 16. | Explain the need for exception handling in C++. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Construct a C++ program to accept two integer values from the user and perform arithmetic operations, compare the values using relational operators, and display the result of logical conditions based on the inputs. | CO1 | A | 12 |
| 18. |  | Write a C++ program to find the factorial of a given number using both for and while loops. | CO2 | A | 12 |
| 19. |  | Develop a C++ program to create a simple class for storing and displaying book details using objects. | CO3 | A | 12 |
| 20. |  | Write a C++ program to implement public inheritance for displaying student and exam details. | CO4 | A | 12 |
| 21. |  | Write a C++ program to calculate the area of different shapes using virtual functions. | CO5 | A | 12 |
| 22. |  | Write a C++ program to find the sum of even numbers from 1 to NNN using a for loop and if-else statement. | CO2 | A | 12 |
| 23. | a. | Describe the types of inheritance in C++. | CO4 | U | 6 |
|  | b. | Write a C++ program to swap two numbers using pointers | CO5 | U | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Write a C++ program using file handling to store and display the names and marks of five students. Your program should: - Create a text file named students.txt - Write the name and mark of five students entered by the user into the file - Read the same data from the file and display it on the screen | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Define the object-oriented programming concepts. |
| CO2 | Select the relevant object-oriented concepts to implement a real time application. |
| CO3 | Demonstrate the application of polymorphism in various ways. |
| CO4 | Illustrate the use of advanced features of C++ such as templates, exceptions, and multiple inheritances. |
| CO5 | Create applications using inheritance in C++. |
| CO6 | Explain file management and string manipulation. |

END SEMESTER EXAMINATION – APRIL/MAY 2026

| # Course Code | # 23EE2035 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # BATTERY MANAGEMENT SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Name any two secondary batteries used in EV. |  | CO1 | R | 1 |
| 2. | Define working voltage. |  | CO1 | R | 1 |
| 3. | List any three functional uses of BMS. |  | CO2 | R | 1 |
| 4. | Justify the need for BMS in lead acid battery. |  | CO2 | E | 1 |
| 5. | Describe any two key components of a Battery Management System (BMS). |  | CO3 | R | 1 |
| 6. | Write the advantages of Coulomb counting method. |  | CO3 | U | 1 |
| 7. | List any one advantage of CAN over LIN. |  | CO4 | R | 1 |
| 8. | Draw the network structure of CAN. |  | CO4 | A | 1 |
| 9. | Define prismatic cell. |  | CO5 | R | 1 |
| 10. | Name any two EV manufacturing companies in India. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the basic charge and discharge process in lithium-ion batteries with a graph of voltage vs specific capacity. |  | CO1 | U | 3 |
| 12. | Describe in brief about the key functions of a Battery Management System. |  | CO2 | U | 3 |
| 13. | Write a brief note on State of Health (SoH) of LIB. |  | CO3 | U | 3 |
| 14. | Discuss the importance of cell balancing in multi-cell battery packs. |  | CO4 | U | 3 |
| 15. | Write a short note on the role of communication protocols in BMS. |  | CO5 | U | 3 |
| 16. | Explain the types of LIB cells based on design aspects. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Describe the construction and working of Lithium ion battery. | CO1 | U | 12 |
| 18. |  | Compare different types of battery management systems implemented in electric vehicles. | CO2 | A | 12 |
| 19. |  | Estimate the SoC by voltage based method, algorithm involved, and its applications. | CO3 | An | 12 |
| 20. |  | Evaluate active and passive cell balancing with examples. | CO4 | E | 12 |
| 21. |  | Analyze the role of heat pipe based on its structure in thermal management of lithium ion battery. | CO5 | An | 12 |
| 22. |  | Differentiate air and liquid cooling in thermal management of LIB. | CO5 | A | 12 |
| 23. |  | Compare CAN with LIN communication protocols in detail. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Evaluate the case study of the Nissan Leaf and Tesla electric vehicles based on different parameters such as line origin, type of battery, thermal management, and other relevant factors. | CO6 | E | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Evaluate the functions and importance of a Battery Management System. |
| CO2 | Estimate the State of Charge (SoC) and State of Health (SoH) of batteries. |
| CO3 | Identify and mitigate safety risks in battery systems. |
| CO4 | Design and simulate BMS for various applications such as electric vehicles and grid storage. |
| CO5 | Implement battery balancing techniques and communication protocols. |
| CO6 | Apply emerging trends and technologies, including AI and ML, in the field of BMS. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 23EE2073 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DIGITAL FORENSICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Name the first city in USA that used photography in criminal investigation. |  | CO1 | U | 1 |
| 2. | State law of progressive change. |  | CO1 | R | 1 |
| 3. | Identify two examples of utility software. |  | CO2 | Ap | 1 |
| 4. | Differentiate Compiler from Linker with respect memory requirements. |  | CO2 | An | 1 |
| 5. | Mention the four components of documenting a crime scene. |  | CO3 | U | 1 |
| 6. | Diagrammatically represent one indoor technique used for measuring the evidence. |  | CO3 | Ap | 1 |
| 7. | Define file carving. |  | CO4 | R | 1 |
| 8. | Name the feature that helps windows to go back in time. |  | CO4 | R | 1 |
| 9. | Identify the secret message in the image given below. |  | CO5 | Ap | 1 |
| 10. | Recall the network analysis tool used in digital forensics. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Name the instrument used for the comparison of tool marks. |  | CO1 | An | 3 |
| 12. | Convert 43110 to Binary, Hexadecimal and Octal number system. |  | CO2 | U | 3 |
| 13. | Differentiate physical from biological evidence with one example each. |  | CO3 | An | 3 |
| 14. | Compare hibernate with sleep mode with respect to resumption. |  | CO4 | An | 3 |
| 15. | Give an example for chaffing and winnowing. |  | CO5 | An | 3 |
| 16. | Identify any 3 features of SINTELIX forensic tool. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Enumerate in detail the seven principles of forensic science. | CO1 | U | 12 |
| 18. |  | With a pyramid diagram interpret the hierarchy of memory organization of a computer. | CO2 | U | 12 |
| 19. |  | Demonstrate the step-by-step procedure to maintain a chain of custody log in order to secure and present evidence in court. | CO3 | AP | 12 |
| 20. |  | Articulate the importance of metadata in terms of creation, modification and accessing. | CO4 | U | 12 |
| 21. |  | Illustrate with an example the application of message digest used for verification of digital evidence. | CO5 | An | 12 |
| 22. |  | Identify the information to be mentioned when seizing the Digital device and name any 6 digital evidences that could be found at a crime scene. | CO3 | Ap | 12 |
| 23. |  | Enumerate the different forms of applying information warfare. | CO5 | Ap | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Summarize the basic steganography model with a block diagram and compare it with combined crypto-steganography in terms of its efficiency. | CO6 | An | 6 |
|  | b. | Interpret the three types of steganography with one example each. | CO6 | Ap | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Illustrate Forensic science and Digital Forensic concepts |
| CO2 | Determine various digital forensic operandi and motive behind cyber-attacks. |
| CO3 | Interpret the cyber pieces of evidence, digital forensic process model and their legal perspective. |
| CO4 | Demonstrate various forensic tools to investigate the cybercrime. |
| CO5 | Categorize the digital pieces of evidence. |
| CO6 | Analyze the digital evidence used to commit cyber offences. |

END SEMESTER EXAMINATION – APRIL/MAY 2026

| # Course Code | # 23EE2079 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # PYTHON PROGRAMMING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1 | Name the basic building blocks of an algorithm. |  | CO1 | R | 1 |
| 2 | Identify the founder of Python programming language. |  | CO1 | R | 1 |
| 3 | Recognize the function used to display output in Python. |  | CO2 | R | 1 |
| 4 | State the operator used for exponentiation. |  | CO2 | R | 1 |
| 5 | Identify an immutable data type in Python. |  | CO3 | R | 1 |
| 6 | Name any one tuple function in Python. |  | CO3 | R | 1 |
| 7 | Recognize the correct syntax for list declaration. |  | CO4 | R | 1 |
| 8 | State the purpose of tuples in Python. |  | CO4 | R | 1 |
| 9 | List any two image filter operations. |  | CO5 | R | 1 |
| 10 | State the purpose of event handling in GUI. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11 | Recall and write the Python program to calculate the sum and Average of n numbers. |  | CO1 | R | 3 |
| 12 | Interpret the role of interpreter in Python. |  | CO2 | R | 3 |
| 13 | Define string and give examples. |  | CO3 | R | 3 |
| 14 | Differentiate between list and tuple. |  | CO4 | U | 3 |
| 15 | Interpret the concept of image enhancement techniques. |  | CO5 | U | 3 |
| 16 | Classify different widgets of GUI with examples. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Illustrate flowchart symbols and their functions, rules for drawing flow chart. | CO1 | U | 12 |
| 18. | a. | Explain arithmetic, logical and relational operators in detail with examples. | CO2 | U | 6 |
|  | b. | Compute the output of:(i) 17 / 3 + 2 ** 3. (ii) (3 – 9) / 3 + 77 * (3 – 1) = -6/3 + 77 * (2) | CO2 | A | 6 |
| 19. | a. | Explain List operations and various list functions with examples. | CO3 | U | 6 |
|  | b. | Describe the different string functions with examples. | CO3 | U | 6 |
| 20. |  | Explain Python dictionary datatype Methods and functions with examples. | CO4 | U | 12 |
| 21. |  | Develop a program to implement image manipulation techniques such as resize, crop and rotate. | CO5 | A | 12 |
| 22. | a. | Compute the output of list slicing: L = [10,20,30,40,50], L[1:4]. | CO3 | A | 2 |
|  | b | Examine types of errors in Python and debugging techniques. | CO2 | An | 10 |
| 23. |  | Analyze and develop a GUI-based Python application integrating multiple widgets and justify its design. | CO6 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Examine and elaborate the syntax of conditionals with necessary flowcharts. | CO1 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze different strategies for developing algorithms. |
| CO2 | Identify errors in Python code with the help of an IDE. |
| CO3 | Write Python functions to facilitate code reuse and manipulate strings. |
| CO4 | Utilize Python data structures effectively. |
| CO5 | Employ Python for simple image processing applications. |
| CO6 | Develop GUI applications using Python. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 24EE2081 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SENSORS AND MEASUREMENT TECHNIQUES IN BIOTECHNOLOGY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | According to Kirchhoff’s Voltage Law, the algebraic sum of all voltages in a closed loop is __________. |  | CO1 | R | 1 |
| 2. | If the frequency of an AC system is 50Hz, its Time period (T) is _____. |  | CO1 | R | 1 |
| 3. | A bridge measurement system is mainly used for the precise measurement of unknown __________. |  | CO2 | R | 1 |
| 4. | If a charge of 50µC passes a given point in a circuit in a time of 100ms, determine the current in the circuit. |  | CO2 | R | 1 |
| 5. | Controlling torque acts in opposition to the _____________ torque to show the definite value in measuring instruments. |  | CO3 | R | 1 |
| 6. | A biosensor generally consists of a biorecognition element and a __________ element. |  | CO3 | R | 1 |
| 7. | The father of Biosensor is ________. |  | CO4 | R | 1 |
| 8. | In medical diagnostics, biosensors are extensively used for monitoring blood __________ levels. |  | CO4 | R | 1 |
| 9. | Identify a key application of intelligent sensors in smart power monitoring systems. |  | CO5 | R | 1 |
| 10. | Name an essential component of a smart sensor. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain Kirchhoff’s Laws with suitable interpretation. |  | CO1 | U | 3 |
| 12. | Identify any three types of digital recorders. |  | CO2 | U | 3 |
| 13. | Identify the major components of a biosensor. |  | CO3 | U | 3 |
| 14. | Recognize any three sensors used for water level measurement. |  | CO4 | U | 3 |
| 15. | Explain the functionality of wireless sensor networks in smart power monitoring. |  | CO5 | U | 3 |
| 16. | Explain the role of big data analytics in improving sensor functionalities in biotechnology. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | A DC circuit consists of a 24 V supply connected to three resistors arranged in a combination of series and parallel. Determine the current through each branch, voltage across each resistor, and total power consumed in the circuit using Ohm’s law and Kirchhoff’s laws. | CO1 | A | 12 |
| 18. |  | With a neat functional diagram, examine the operation of a Digital Storage Oscilloscope (DSO) and interpret the function of each stage. | CO2 | An | 12 |
| 19. |  | With a neat diagram, examine the operation and types of electrochemical sensors used in biotechnology, and assess their advantages. | CO3 | An | 12 |
| 20. |  | Examine the structure of nano sensors and assess its role in smart healthcare. | CO4 | An | 12 |
| 21. |  | Explicate the structure of smart sensors and assess the role of each component in their overall functionality. | CO5 | U | 12 |
| 22. |  | With a suitable diagram, examine the working principle of a Digital Voltmeter (DVM). | CO2 | An | 12 |
| 23. |  | Examine the general working, types, and characteristics of biosensors with suitable diagrams. | CO3 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Examine the working principle, types, and applications of wearable biosensors, and assess their benefits and challenges in healthcare and fitness monitoring. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recall and describe the fundamental principles underlying sensor technology in biotechnology |
| CO2 | Demonstrate an understanding of various types of measuring instruments used in biotechnology and differentiate between their operating mechanisms |
| CO3 | Apply sensor principles to design and develop biosensing systems for specific biotechnological applications, such as medical diagnostics or environmental monitoring |
| CO4 | Synthesize knowledge of sensor technology and biotechnological principles to propose innovative solutions for real-world challenges in healthcare, agriculture, or environmental sustainability |
| CO5 | Development and deployment of sensor-based technologies in biotechnology |
| CO6 | Apply sensor calibration technique for accurate measurements |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25EE202 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DIFFERENTIAL EQUATIONS AND TRANSFORMS | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | Related CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | Identify the order and degree of this differential equation representing the RLC circuit behavior: d²y/dt² + (dy/dt)² + y = 0. | 1a | R | 1 |
| 2. | The electrical behavior of a second‑order RLC system is modeled by: (D² + 5D + 6)y =0. Determine the output signal y(x) representing the system response. | 1b | R | 1 |
| 3. | Discuss briefly the application of Laplace Transform in electrical engineering for circuit analysis and transient response. | 2a | R | 2 |
| 4. | Determine the Laplace transforms of the following signals applied to an LC circuit: (a) Constant voltage input 10. (b) sint. | 2c | R | 2 |
| 5. | Define the Fourier transform pair for discrete signals. | 3a | R | 3 |
| 6. | In AC signal processing, if F denotes the Fourier transform of signal f(x), find (a) F(f(3x)). (b) F(f(x − 2)). | 3b | R | 3 |
| 7. | In a digital control system, determine the Z-transform of the discrete signals: Z(2) and Z(Cos nθ). | 4a | R | 4 |
| 8. | Evaluate the given discrete current in a dynamic electrical system: Zsinnπ4. | 4b | U | 4 |
| 9. | While solving a nonlinear circuit equation using the Bisection method, if f(0) = −2, f(1) = −3, f(2) = −2 and f(3) = 1, determine the interval in which the root (operating point voltage) lies. | 5a | U | 5 |
| 10. | State the requirement on the number of intervals for applying Simpson’s 1/3 rule in computing energy of an electrical waveform. | 5b | U | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | In an electrical circuit, the voltage across a component is described by the differential equation (D2+1)y=0,Calculate the Wronskian function. | 1c | A | 1 |
| 12. | Find the Laplace transform of the voltage signal: (e−2t cos t + t sin t), which appears in transient analysis of electrical circuits. | 2b | A | 2 |
| 13. | Determine the finite Fourier sine and cosine transforms of the constant voltage signal f(x)=2 defined over the interval (0,1) in an electrical system. | 3d | A | 3 |
| 14. | In a digital signal processing system, the discrete-time input signal is given by Z3n+4n. Determine the corresponding Z-transform and system response. | 4b | A | 4 |
| 15. | In a power system dynamic model, current y satisfies a differential equation. Using Euler’s method, compute the values of y at x=0.1,0.2,0.3, given y'=1-y,y(0)=0. | 5d | A | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | In a control system used in electrical drives, the output signal y satisfies: (D² − 4)y = e−2x + cos x. Determine the complete system response y. | 1b | A | 1 |
| (OR) |  |  |  |  |
| 17 | Solve the differential equation arising in an electrical circuit using the method of variation of parameters: y''+y=Secx. | 1c | A | 1 |
| 18 | Determine the time-domain signal from the Laplace-domain representation using partial fractions L-13s+2s+1s-2s+3. | 2c | A | 2 |
| (OR) |  |  |  |  |
| 19 | Evaluate the circuit response using Laplace transform for the RLC differential equation y’-y=2,y0=2. | 2d | A | 2 |
| 20 | Compute the Fourier transform of the pulse signal used in communication systems: fx=2: x<10:x>1. | 3b | A | 3 |
| (OR) |  |  |  |  |
| 21 | Determine the Fourier sine and cosine transform of the voltage waveform fx=e-x that represents the decay of voltage in an RLC electrical circuit. | 3d | A | 3 |
| 22 | In a discrete signal processing circuit, find the inverse Z- transform using the partial fraction method: Z-1z2+zz-1z+2z-3. | 4c | A | 4 |
| (OR) |  |  |  |  |
| 23 | For a discrete-time electrical system, solve the given difference equation using the Z-transform technique given . | 4d | A | 4 |
|  | Compulsory Question: |  |  |  |
| 24 | A varying voltage is given by06dx1+x2. Compute the total electrical energy using numerical integration methods: (i) Trapezoidal Rule (ii) Simpson’s 1/3 Rule (iii) Simpson’s 3/8 Rule. Take step size h = 1. | 5c | A | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25EE208 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # SIGNALS AND SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | Related CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | Identify whether the signal x(t)=sin (t) is periodic or aperiodic. | 1a | R | 1 |
| 2. | State the condition for a signal to be classified as an even signal. | 1b | R | 1 |
| 3. | Label the parameters present in the continuous-time exponential signal x(t)=Ceat(t). | 2a | R | 2 |
| 4. | Sketch the waveform of a unit ramp signal. | 2b | U | 2 |
| 5. | State the Fourier Transform of a DC signal. | 3a | U | 3 |
| 6. | Identify whether the system y(t)=3x(t) is linear or nonlinear. | 3b | R | 3 |
| 7. | Determine the Nyquist rate for a signal with maximum frequency of 5 kHz. | 4a | A | 4 |
| 8. | Identify the fundamental condition necessary to prevent aliasing during signal sampling. | 4b | R | 4 |
| 9. | Identify whether a system is stable if all poles lie in the left half of the s-plane. | 5a | R | 5 |
| 10. | Describe the effect of repeated poles on system stability. | 5b | U | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Explain the different classifications of signals with suitable examples. | 1a | U | 1 |
| 12. | Find the even and odd components of the signal x(t)=cos t + sin t | 1b | A | 2 |
| 13. | Test whether the system y(t)=x(t)+5 is linear and time invariant. | 2a | A | 3 |
| 14. | Determine the poles of the transfer function Hs=5s2+6s+8 and classify the stability of the system. Also sketch the pole locations on the s-plane. | 2b | A | 4 |
| 15. | Sketch the signal x(t+4) and x (2t+3) | 2c | A | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Sketch the following signal with mathematical expression a) Unit step b) Ramp c) Impulse d) Sinusoidal e) Exponential | 1c | U | 1 |
| (OR) |  |  |  |  |
| 17 | Find the following summations - n=-͟∞n=∞e3nδ(n-3) b)n=-͟∞n=∞δn-2cos3n c) n=-͟∞n=∞n2δ(n+4) d) n=-͟∞n=∞δ(n+1) 4n | 1e | A | 1 |
| 18 | Derive the Fourier Transform of a cosine signal using exponential representation. | 2b | A | 2 |
| (OR) |  |  |  |  |
| 19 | Compute the Fourier Transform of the signal x(t)=cos(2πt) and sketch its frequency spectrum. | 2c | A | 2 |
| 20 | Find the linear convolution of x(n)={1, 2, 3, 4, 5, 6, 7} with h(n)={2, 4, 6, 8} | 3d | A | 3 |
| (OR) |  |  |  |  |
| 21 | Analyze the linear, time invariant causal and stability properties of the given continuous-Time LTI System y(t)=2x(t)+3. | 3f | An | 3 |
| 22 | Describe the process of acquisition, processing, and analysis of biological signals in medical instrumentation systems. | 4c | U | 4 |
| (OR) |  |  |  |  |
| 23 | Explain the working principle of an electronic energy meter with a neat block diagram and analyze its advantages over conventional meters. | 4e | U | 4 |
| COMPULSORY QUESTION: |  |  |  |  |
| 24 | Determine the poles of the transfer function Hs=10s2+6s+9. Evaluate the damping ratio, natural frequency, and classify the type of damping. Also comment on the stability of the system. | 5d | A | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25EE214 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | C PROGRAMMING ESSENTIALS | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | Related CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | Define data types in C and list any two examples. | 1a | R | 1 |
| 2. | Illustrate the structure of a C program with basic components. | 1b | U | 1 |
| 3. | Define decision-making statements and list their types. | 2a | R | 2 |
| 4. | Compare while loop and for loop in C programming. | 2b | U | 2 |
| 5. | Define one-dimensional array and list its applications. | 3a | R | 3 |
| 6. | Illustrate string manipulation with a suitable example. | 3b | U | 3 |
| 7. | Define pointer and identify its basic operation. | 4a | R | 4 |
| 8. | Compare structures and arrays in C programming. | 4b | U | 4 |
| 9. | Define file handling and list file operations. | 5a | R | 5 |
| 10. | Analyse the role of command-line arguments in handling input parameters for electrical applications. | 5b | An | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Analyse the role of command-line arguments in handling input parameters for electrical applications. | 1a | U | 1 |
| 12. | Illustrate the steps involved in computing electrical power using input parameters. | 1b | U | 2 |
| 13. | Construct a modular C program using functions to compute electrical parameters and validate results. | 2a | A | 3 |
| 14. | Differentiate iterative and recursive approaches for engineering computations with suitable justification. | 2b | An | 4 |
| 15. | Construct a C program to organize sensor data using arrays and analyze the output for correctness. | 2c | A | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Construct a structured C program for energy consumption calculation using control statements and test its efficiency for different input conditions. | 1c | A | 1 |
| (OR) |  |  |  |  |
| 17 | Analyse the performance of nested control structures in solving real-time electrical parameter computation problems. | 1e | An | 1 |
| 18 | Construct a modular C program using functions to compute transformer efficiency and verify the accuracy of results. | 2b | A | 2 |
| (OR) |  |  |  |  |
| 19 | Analyse recursion-based solutions for engineering computations. | 2c | An | 2 |
| 20 | Construct a C program to perform sorting of electrical measurement data | 3d | A | 3 |
| (OR) |  |  |  |  |
| 21 | Analyse the role of string manipulation techniques in processing equipment information. | 3f | An | 3 |
| 22 | Construct a C program using pointers and dynamic memory allocation for real-time data handling. | 4c | A | 4 |
| (OR) |  |  |  |  |
| 23 | Analyze the use of structures and nested structures in managing electrical records. | 4e | An | 4 |
| COMPULSORY QUESTION: |  |  |  |  |
| 24 | Construct a C-based automation program for an electrical control application using file handling and command-line arguments. | 5d | A | 5 |

END SEMESTER EXAMINATION – APRIL/MAY 2026

| # Course Code | # 25EE301 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ANALOG CIRCUIT DESIGN | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | Calculate DC output voltage of the full wave rectifier supplied with input voltage Vi = 12.56 Sin ωt | 1c | A | 1 |
| 2. | Compute the ripple voltage of a half wave rectifier that supplies the DC voltage (VDC) = 9.92V. | 1e | A | 1 |
| 3. | Infer the function of a positive clipper | 2a | An | 2 |
| 4. | Justify the reasons for high input impedance of a MOSFET amplifier. | 2c | E | 2 |
| 5. | Compute the emitter current IE of a CE transistor configuration when IC = 100 mA and IB = 500 µA. | 3c | A | 3 |
| 6. | List different configurations of BJT in different applications. | 3b | U | 3 |
| 7. | Indicate the difference between BJT and FET. | 4a | U | 4 |
| 8. | Sketch the structure of a Depletion MOSFET | 4c | R | 4 |
| 9. | State the conditions for a sustained oscillation that can be produced by the oscillator. | 5a | R | 5 |
| 10. | Recall the merits of feedback in an amplifier | 5d | R | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Summarize the effect of combining P and N typer materials on formation of PN Junction | 1f | U | 1 |
| 12. | Interpret the need of biasing in a BJT amplifier circuit in relevance to the positioning of Q point. | 2e | A | 2 |
| 13. | Interpret the different power amplifiers and compare the efficiencies. | 3f | U | 3 |
| 14. | Recall the effects of feedback on amplifiers | 4a | An | 4 |
| 15. | Write short notes on crystal oscillators | 5c | An | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Draw the VI characteristics of a PN Junction Diode and correlate its function | 1b | An | 1 |
| (OR) |  |  |  |  |
| 17 | Explain the operation of a full wave rectifier and sketch the input and output waveforms | 1c | A | 1 |
| 18 | Demonstrate the Input and output characteristics of a CE configuration Transistor | 2b | A | 2 |
| (OR) |  |  |  |  |
| 19 | Design a Transistor circuit that operates as a switch and explain on which regions of the output characteristics it performs the switching action | 2c | An | 2 |
| 20 | Determine the IBQ, ICQ, VBEQ of the fixed biased amplifier circuit. Given VCC= 12V, β = 50, RB = 240kΩ, RC = 2.2kΩ, | 3e | An | 3 |
| (OR) |  |  |  |  |
| 21 | Design a push pull amplifier and explain its functioning. State the merits of the push pull configuration. | 3d | U | 3 |
| 22 | Design a series fed class A power amplifier and derive its maximum efficiency | 4b | An | 4 |
| (OR) |  |  |  |  |
| 23 | Explain the Voltage Series amplifier and analyze its gain, input impedance, and output impedance with relevance to the feed back | 4c | An | 4 |
| COMPULSORY QUESTION: |  |  |  |  |
| 24 | Contrast the functioning of a RC Phase shift Oscillator | 5c | An | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25EE302 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRICAL CIRCUIT ANALYSIS | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | Solve the current distribution at a junction in an industrial DC motor drive circuit where four branches meet. If currents entering the node are 8 A and 5 A, and one current leaving the node is 10 A, determine the remaining branch current using Kirchhoff’s Current Law. | 1a | A | 1 |
| 2. | Choose the most appropriate circuit analysis method for the transistor circuit shown below and justify the choice based on circuit complexity. | 1d | A | 1 |
| 3. | Interpret the use of Thevenin’s Theorem in simplifying an industrial sensor interface circuit with multiple resistive branches supplying a measurement unit. Identify when Thevenin equivalent modeling becomes advantageous for analysis and diagnostics. | 2a | U | 2 |
| 4. | Interpret the applicability of the Maximum Power Transfer Theorem in an electric vehicle fast-charging circuit where the charger internal resistance is comparable to the battery input resistance. State whether this theorem is desirable in this scenario and justify the reasoning. | 2a | U | 2 |
| 5. | Apply Laplace Transform to solve the differential equation governing a signal conditioning RC circuit in a semiconductor measurement system: dv(t)dt+5v(t)=10 | 3a | A | 3 |
| 6. | Determine the time constant of an RL circuit used in an industrial motor drive, where the inductance is L=2 H and the resistance is R=10 Ω. State its significance in current rise. | 3b | U | 3 |
| 7. | For the given voltage find the rms values and frequency. | 4a | A | 4 |
| 8. | For the given voltage and current waveforms, draw the phasor diagram. | 4b | U | 4 |
| 9. | Represent the given MOSFET amplifier as a two-port network. | 5a | A | 5 |
| 10. | Interpret the network function of a control circuit given as H(s)=10s2+4s+10 to determine whether the system is stable. | 5c | A | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Solve the node voltages in an industrial DC power distribution system represented by the circuit given below using relevant node equations. | 1b | A | 1 |
| 12. | Use Thevenin’s theorem to determine the current through the load resistor in an industrial control panel circuit shown below. | 2b | A | 2 |
| 13. | A power system capacitor bank used for voltage support is modelled as an RC circuit with resistance R=10 Ωand capacitance C=0.05 F. The capacitor is initially uncharged. The circuit is connected to a DC source of 200 V through a switch that is closed at t=0. i) Determine the expression for capacitor voltage vC(t). ii) Determine the expression for charging current i(t). iii) Calculate the time constant of the circuit. iv) Find the steady-state voltage across the capacitor. v) Sketch the waveforms of: Capacitor voltage vC(t), Charging current i(t) | 3b | A | 3 |
| 14. | An aircraft auxiliary system operates from a 115 V, 400 Hz AC supply. The load consists of an inductive component modelled as a resistor R=12 Ω in series with an inductive reactance XL=16 Ω. i) Determine the impedance of the circuit. ii) Calculate the current drawn from the supply. iii) Determine the power factor of the load. iv) Calculate the real power consumed. | 4c | A | 4 |
| 15. | Derive the Z-parameters of the given resistive two-port network used in an industrial sensor interface and verify whether the network is reciprocal and symmetric. | 5b | A | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Apply mesh current analysis to determine the unknown branch currents in an industrial heating circuit. The circuit consists of two meshes: Mesh 1 has a 20 V supply and resistors of 4 Ω and 6 Ω. Mesh 2 shares the 6 Ω resistor with Mesh 1 and includes an additional 8 Ω resistor and a 10 V source opposing the first source. Determine the mesh currents and the current through the shared 6 Ω resistor. | 1c | A | 1 |
| (OR) |  |  |  |  |
| 17 | Implement nodal analysis to determine all node voltages and branch currents in an industrial DC smart grid shown below. The circuit consists of a 72 V source connected to node 1 through a 6 Ω resistor. Node 1 is connected to ground through 12 Ω and to node 2 through 4 Ω. Node 2 is connected to ground through 8 Ω and is also fed by a 5 A current source entering the node. Interpret the sensitivity of node voltages if the 4 Ω interconnection resistance increases by 50% due to thermal effects, and comment on system reliability. | 1e | An | 1 |
| 18 | Use Norton’s theorem to determine the current through a critical load in a semiconductor wafer fabrication power supply circuit. The circuit consists of a 30 V DC source in series with a 3 Ω resistor supplying a network where a 6 Ω resistor is in parallel with a branch containing a 12 Ω resistor. The load resistor RL=5 Ω(representing a precision process control unit) is connected across the output terminals. | 2c | A | 2 |
| (OR) |  |  |  |  |
| 19 | A precision DC test source of 15 V with an internal resistance of 3 Ω is connected to a signal conditioning network consisting of a series resistor of 5 Ω followed by a parallel combination of 10 Ω (measurement path) and variable load resistor RLrepresenting the IC under test. i) Evaluate the Thevenin equivalent seen by RL. ii) Evaluate the condition for maximum power transfer and determine the optimal value of RL. iii) Evaluate the maximum power delivered to the IC under test. iv) Evaluate whether operating at maximum power transfer is suitable for sensitive semiconductor devices and justify your conclusion. | 2d | E | 2 |
| 20 | A robotic joint actuator is modelled as a series RLC circuit with: Resistance R=2 Ω (winding resistance); Inductance L=0.5 H (actuator coil); Capacitance C=0.1 F (parasitic/filter capacitance). The circuit is connected to a DC source of 50 V at t=0 when the control switch is turned ON. i) Determine the nature of damping ii) Calculate the damping factor. iii) Determine expression for current i(t). iv) Sketch the wave Current Waveform i(t) | 3c | A | 3 |
| (OR) |  |  |  |  |
| 21 | Evaluate the transient response of a series RLC circuit with DC excitation used in a robotic actuator drive system. Explain the behaviour of current for the following cases: Underdamped; Critically damped; Overdamped. | 3c | E | 3 |
| 22 | A single-phase supply of 230 V, 50 Hz feeds a small workshop consisting of: A heating unit represented by a resistor R=20 Ω; A motor load represented by an inductive reactance XL=15 Ω;A power factor correction capacitor with capacitive reactance XC=10 Ω. i) Calculate the total impedance of the circuit. ii) Determine the current drawn from the supply; iii) Calculate: Real power P, Reactive power Q, Apparent power S iv) Determine the power factor and comment on its suitability for small-scale industry operation. | 4d | A | 4 |
| (OR) |  |  |  |  |
| 23 | An FM receiver front-end uses a series RLC circuit to select a desired station. The circuit parameters are: Inductance L=0.25 μH(tuning coil); Capacitance C=100 pF(variable capacitor); Resistance R=5 Ω(coil and circuit losses). i) Determine the resonant frequency of the circuit and verify whether it lies within the FM band (88–108 MHz). ii) Calculate the impedance at resonance and the corresponding current if the input signal voltage is 1 V. iii) Determine the quality factor (Q) of the circuit. iv) Calculate the bandwidth of the circuit. v) Interpret how resonance enables station selection and discuss the effect of high Q on selectivity in FM receivers. | 4e | A | 4 |
| Compulsory Question: |  |  |  |  |
| 24 | A sensor signal in an industrial automation system passes through two cascaded stages: - Stage 1 (Pre-amplifier) is represented by ABCD parameters: A1=2, B1=50 Ω, C1=0.02 S, D1=1 - Stage 2 (Power amplifier) is represented by ABCD parameters: A2=1.5, B2=30 Ω, C2=0.01 S, D2=1 The two networks are connected in cascade. i) Draw the schematic, ii) Determine the overall ABCD parameters of the cascaded system. iii) If the load connected at the output is RL=100 Ω, determine the input impedance of the overall network iv) Calculate the voltage gain of the cascaded system. v) Explore the advantage of cascading two-port networks in industrial signal processing systems. | 5d | E | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 26EE702 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DEEP LEARNING IN ELECTRICAL ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Explain the perceptron model and derive its weight update rule using gradient descent. Illustrate with a numerical example for a simple electrical signal classification problem. | CO1 | An | 10 |
|  | b. | Discuss activation functions and loss functions used in neural networks. Derive the gradient of the Mean Squared Error (MSE) loss function and explain its role in backpropagation. | CO1 | An | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Explain the steps involved in preprocessing electrical engineering datasets such as signals, waveforms and sensor data | CO1 | U | 10 |
|  | b. | Solve a numerical problem showing one iteration of weight update using the Adam optimizer. | CO2 | An | 10 |
| 3. | a. | Explain feedforward deep neural networks and discuss weight initialization techniques. | CO2 | An | 10 |
|  | b. | Discuss batch normalization and dropout in deep learning. Analyze their role in preventing overfitting and improving generalization in electrical engineering applications. | CO2 | An | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Explain the convolution operation and feature map generation in CNNs. Solve a numerical example showing convolution on a sample input matrix. | CO3 | An | 10 |
|  | b. | Solve a simple sequence prediction problem and evaluate model performance using suitable metrics. | CO4 | A | 10 |
| 5. |  | Discuss CNN architectures (LeNet, AlexNet, ResNet) and analyze their suitability for fault diagnosis in rotating machines and power quality disturbance classification. | CO3 | An | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Explain the working of RNN and LSTM networks. Derive the key equations of an LSTM cell and discuss their application in electrical load forecasting. | CO4 | A | 20 |
| 7. |  | Formulate a deep learning-based approach for power system fault detection. Include problem definition, dataset preparation, model design, and evaluation metrics. | CO5 | An | 20 |
|  |  | (OR) |  |  |  |
| 8. | a. | Explain Graph Neural Networks (GNNs) and their application in power grid modeling. Illustrate with a simple graph-based problem. | CO6 | A | 10 |
|  | b. | Explain how deep learning is used for battery State-of-Charge (SoC) estimation in electric vehicles | CO5 | U | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Explain the architecture and working of autoencoders and variational autoencoders. | CO6 | U | 10 |
|  | b. | Discuss the concept of Transformers and attention mechanisms. | CO6 | U | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Interpret the fundamentals of deep learning, neural network architectures, and training methodologies relevant to electrical engineering data. |
| CO2 | Use deep learning models for signal processing, electrical machines, and power system datasets. |
| CO3 | Predict time-series data from electrical systems using recurrent and sequence learning models. |
| CO4 | Evaluate convolutional and deep architectures for fault diagnosis, condition monitoring, and image-based inspection in electrical engineering. |
| CO5 | Assess deep learning techniques for renewable energy forecasting, smart grids, and electric vehicle applications |
| CO6 | Design advanced deep learning models and optimize their performance for real-world electrical engineering problems. |