END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 18ME3001 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # THERMODYNAMICS AND COMBUSTION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Derive the expression for second-law efficiency and explain its importance. | CO1 | A | 6 |
|  | b. | Explain the concept of exergy and availability with respect to simple systems. | CO1 | A | 6 |
|  | c. | Air flows steadily through a device with negligible changes in kinetic and potential energy. The system loses heat at a rate of 10 kJ/kg per minute for 2 minutes, while work output is 60 kJ/kg. Determine the change in enthalpy | CO1 | A | 8 |
|  |  | (OR) |  |  |  |
| 2. | a. | Explain real gas behaviour and the limitations of ideal gas assumptions. | CO2 | A | 4 |
|  | b. | Define compressibility factor and explain its variation with pressure and temperature. | CO2 | R | 8 |
|  | c. | A gas occupies 0.5 m³ under ideal conditions. At a given pressure and temperature, the compressibility factor is 0.92. Determine: (i) The actual volume (ii) The percentage deviation from ideal behaviour | CO2 | A | 8 |
| 3. | a. | Define thermodynamic potentials and explain their significance | CO3 | R | 10 |
|  | b. | Derive any two Maxwell relations from thermodynamic equations. | CO3 | A | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Explain Joule–Thomson effect and define Joule–Thomson coefficient. | CO3 | A | 6 |
|  | b. | State the Clausius–Clapeyron equation and explain its importance. | CO3 | R | 6 |
|  | c. | A gas undergoes throttling with a Joule–Thomson coefficient of 0.25 K/bar. The pressure drops from 10 bar to 2 bar. Determine: (i) The temperature change. (ii) Whether the gas undergoes cooling or heating. Assume the Joule–Thomson coefficient remains constant over the pressure range. | CO3 | A | 8 |
| 5. | a. | Define stoichiometric air–fuel ratio and excess air. | CO4 | R | 5 |
|  | b. | Methane (CH₄) is burned with 20% excess air. Determine: (i) Theoretical air required per kmol of fuel (ii) Actual air supplied (iii) Air–fuel ratio by mass (iv) Composition of products (assume complete combustion) Given: Air composition: 21% O₂ and 79% N₂ (by volume) Molar masses: CH₄ = 16 kg/kmol, O₂ = 32 kg/kmol, N₂ = 28 kg/kmol, CO₂ = 44 kg/kmol, H₂O = 18 kg/kmol | CO4 | A | 15 |
|  |  | (OR) |  |  |  |
| 6. | a. | Explain the concept of adiabatic flame temperature. | CO4 | A | 5 |
|  | b. | Ethylene (C₂H₄) is burned with theoretical air. Determine: (i) Minimum air required for complete combustion (ii) Moles of CO₂, H₂O, and N₂ in products (iii) Air–fuel ratio by mass (iv) Percentage of nitrogen in products Given: Air composition: 21% O₂ and 79% N₂ (by volume) Molar masses: C₂H₄ = 28 kg/kmol, O₂ = 32 kg/kmol, N₂ = 28 kg/kmol, CO₂ = 44 kg/kmol, H₂O = 18 kg/kmol | CO4 | A | 15 |
| 7. | a. | Explain chemical equilibrium and dissociation in combustion processes. | CO5 | A | 8 |
|  | b. | Compare homogeneous and heterogeneous combustion. | CO5 | U | 6 |
|  | c. | Explain laminar flame propagation in engines. | CO5 | A | 6 |
|  |  | (OR) |  |  |  |
| 8. | a. | Analyze turbulent flame propagation and its importance in engines. | CO5 | An | 8 |
|  | b. | Illustrate a second-law analysis of reacting mixtures. | CO5 | An | 6 |
|  | c. | Explain availability analysis of reacting systems. | CO5 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Explain the basic concepts of statistical thermodynamics. | CO6 | A | 6 |
|  | b. | Discuss the statistical interpretation of entropy. | CO6 | U | 6 |
|  | c. | Explain how the first and second laws are interpreted statistically. | CO6 | A | 8 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyse the closed and open systems by using laws of thermodynamics |
| CO2 | Calculate the properties of gases and gas mixtures |
| CO3 | Calculate change in entropy and enthalpy during the phase change of pure substances |
| CO4 | Compute air requirements and adiabatic flame temperatures |
| CO5 | Develop an understanding of the combustion process, engine emissions |
| CO6 | Develop an understanding of statistical methods |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 18ME3024 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ENERGY CONSERVATION AND MANAGEMENT | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Differentiate renewable energy sources from non-renewable energy sources with suitable examples. | CO1 | An | 10 |
|  | b. | Explain the distribution and significance of global energy reserves. | CO1 | U | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Analyze the need for a sustainable energy strategy for the future. | CO2 | An | 10 |
|  | b. | Evaluate the key provisions and impact of the Energy Conservation Act. | CO2 | E | 10 |
| 3. | a. | Explain the various forms of energy with relevant examples. | CO3 | U | 10 |
|  | b. | Classify and analyze the different grades of energy. | CO3 | An | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Explain the basic concepts of thermal energy. | CO4 | U | 10 |
|  | b | Apply thermal energy principles to simple engineering systems. | CO4 | A | 10 |
| 5. | a. | Explain the definition, objectives, and need for energy auditing. | CO5 | U | 10 |
|  | b | Analyze the benefits of conducting an energy audit in industrial applications. | CO5 | An | 10 |
|  |  | (OR) |  |  |  |
| 6. | a. | Explain the different types of energy auditing and the instruments used. | CO5 | U | 10 |
|  | b | Apply appropriate energy auditing techniques to a given system. | CO5 | A | 10 |
| 7. | a. | Analyze energy conservation techniques in thermal systems such as boilers, furnaces, and heat exchangers. | CO6 | An | 10 |
|  | b. | Evaluate energy conservation practices in buildings. | CO6 | E | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Apply energy efficiency improvement techniques in thermal systems. | CO6 | A | 10 |
|  | b. | Evaluate practical strategies for improving energy efficiency in electrical systems. | CO6 | E | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. | a. | Explain the principles of energy management. | CO6 | U | 10 |
|  | b. | Calculate and interpret the simple payback period for an energy-saving project. | CO6 | A | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Discuss the present status of the national energy scenario. |
| CO2 | Compare the different forms of energy |
| CO3 | Apply various energy auditing methods |
| CO4 | Analyze the energy conservation areas in thermal systems. |
| CO5 | Estimate the energy conservation areas in electrical systems |
| CO6 | Choose the different financial management methods. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 18ME3062 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # COMPOSITE MATERIALS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Outline the general characteristics and applications of composite materials. | CO1 | A | 20 |
|  |  | (OR) |  |  |  |
| 2. |  | Explain in detail the types of Matrix and Reinforcements used in metal matrix composites (MMC). | CO2 | U | 20 |
| 3. |  | Evaluate the reliability of composite materials in engineering applications. Discuss factors affecting the reliability of composites and methods for improving reliability. | CO3 | E | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | Describe any two manufacturing methods of composite materials. | CO4 | U | 20 |
| 5. |  | Enumerate the application of finite element analysis (FEA) in predicting the behaviour of composite materials under different loading conditions. | CO5 | U | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Define fatigue failure. Explain the low cycle fatigue and high cycle fatigue. | CO3 | U | 20 |
| 7. |  | Analyze the characteristics of metal matrix composites (MMC) in comparison to traditional alloys. | CO1 | An | 20 |
|  |  | (OR) |  |  |  |
| 8. |  | Discuss their vibration and stability behaviour, highlighting their impact on the reliability of composite structures under various loading conditions. | CO6 | A | 20 |
| 9. |  | Analyze the role of non-destructive testing techniques in the assessment of composite products. | CO6 | An | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Predict elastic properties of composites and |
| CO2 | Predict mechanical properties of fiber reinforced composite materials. |
| CO3 | Design a composite laminate for a given load condition. |
| CO4 | Describe fundamental fabrication processes for polymer matrix composites. |
| CO5 | Analyze the stresses using laminated plate theories. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 19ME1003 | # Duration | # 3 hrs |
| --- | --- | --- | --- |
| # Course Title | # ENGINEERING MECHANICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Discuss parallelogram law of forces. |  | CO1 | U | 1 |
| 2. | State the principle of transmissibility of forces. |  | CO1 | R | 1 |
| 3. | Discuss Varignon’s theorem. |  | CO2 | U | 1 |
| 4. | Identify the types of supports in a beam. |  | CO2 | U | 1 |
| 5. | The moment of inertia (Ixx) of a triangle of width (b) and height (h) is given by _____________. |  | CO3 | R | 1 |
| 6. | Explain centre of gravity and centroid. |  | CO3 | R | 1 |
| 7. | Define the term ‘Range’ of a projectile. |  | CO4 | R | 1 |
| 8. | Name the principle used to analyze the motion a lift. |  | CO4 | R | 1 |
| 9. | State the Impulse-Momentum principle. |  | CO5 | R | 1 |
| 10. | Define angle of repose. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Calculate the moment with respect of the point O in the given figure. |  | CO1 | A | 3 |
| 12. | Explain how the principle of transmissibility is applied on the van given in the following figure. |  | CO2 | A | 3 |
| 13. | Discuss on the perpendicular axis theorem. |  | CO3 | U | 3 |
| 14. | A scooter starts from rest and moves with a constant acceleration of 1.2 m/s2. Determine its velocity, after it has travelled for 60 meters. |  | CO4 | A | 3 |
| 15. | Discuss D’Alembert’s Principle. |  | CO5 | U | 3 |
| 16. | Write the definition for frictional forces and mention the types of frictional forces. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | The forces 20 N, 30 N, 40 N, 50 N and 60 N are acting at one of the angular points of a regular hexagon, towards the other five angular points, taken in order. Calculate the magnitude and direction of the resultant force. | CO1 | An | 12 |
| 18. |  | Three smooth pipes each weighing 20 kN and of diameter 60 cm are to be placed in a rectangular channel with horizontal base as in the figure. Calculate the reactions at the points of contacts between the pipes and between the channel and the pipes. Take, width of the channel as 160 cm. | CO2 | An | 12 |
| 19. |  | An I-section is made up of three rectangles as shown in figure. Estimate the moment of inertia of the section about the horizontal axis passing through the centre of gravity of the section. | CO3 | An | 12 |
| 20. | a. | Derive the mathematical expression for the normal and tangential components of acceleration for a particle moving along the curved path. | CO4 | U | 4 |
|  | b. | ﻿The position of a particle which moves along a straight line is defined by the relation x= t3-6t2-15t+40, where x is expressed in feet and t in seconds. Determine (i) the time at which the velocity will be zero, (ii) the position and distance traveled by the particle at that time. | CO4 | A | 8 |
| 21. | a. | A ball of mass 1 kg moving with a velocity of 2 m/s impinges directly on a ball of mass 2 kg at rest. The first ball, after impinging, comes to rest. Find the velocity of the second ball after the impact and the coefficient of restitution. | CO5 | A | 6 |
|  | b. | Find the acceleration of a solid body A of mass 10 kg, when it is being pulled by another body B of mass 5 kg along a smooth horizontal plane as shown in figure. Also find the tension in the string, assuming the string to be inextensible. Take g = 9.8 m/s2. | CO5 | An | 6 |
| 22. |  | A simply supported beam, AB of span 6 m is loaded as shown in figure. Calculate the reactions at the supports A and B of the beam. | CO2 | A | 12 |
| 23. |  | Two blocks of weight 150 N and 50 N are connected by a rope passing over a frictionless pulley as shown in the fugure. Estimate the acceleration of blocks A and B and the tension in the rope using Newton’s law of motion. | CO5 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | A uniform ladder weighing 100 N and 5 meters long has lower end B resting on the ground and upper end A resting against a vertical wall as shown in figure. The inclination of the ladder with horizontal is 60°.if the coefficient of the friction at all surfaces of contact is 0.25, estimate how much distance up long the ladder a man weighing 600N can ascent without causing it to slip. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Determine the resultant force and moment for a given system of forces |
| CO2 | Understand basics of equilibrium of rigid bodies |
| CO3 | Determine the centroid and second moment of area of simple solids |
| CO4 | Apply fundamental concepts of kinematics and kinetics to the analysis of simple / practical problems |
| CO5 | Understand basic kinematics concepts – displacement, velocity and acceleration |
| CO6 | Determine friction and its effects as per the laws of friction |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 19ME2025 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # THERMODYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State the difference between intensive and extensive properties. |  | CO1 | U | 1 |
| 2. | Define ‘Thermal Equilibrium’. |  | CO1 | R | 1 |
| 3. | Write the first law of thermodynamics for a closed system during a cycle. |  | CO2 | R | 1 |
| 4. | State the Clausius statement of the second law of thermodynamics. |  | CO2 | R | 1 |
| 5. | Interpret the meaning of the triple point in a phase diagram. |  | CO3 | U | 1 |
| 6. | Define ‘dryness fraction’ |  | CO3 | R | 1 |
| 7. | Interpret the relation between characteristic and universal gas constants. |  | CO4 | U | 1 |
| 8. | State Dalton’s law. |  | CO4 | R | 1 |
| 9. | Define’ Relative humidity’. |  | CO5 | U | 1 |
| 10. | Write the processes involved in the Otto cycle. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain free expansion with zero work transfer. |  | CO1 | U | 3 |
| 12. | Derive the SFEE for Nozzle and air compressor. |  | CO2 | U | 3 |
| 13. | Draw the phase equilibrium diagram for a pure substance on P-T coordinates. |  | CO3 | An | 3 |
| 14. | Explain the law of corresponding states. |  | CO4 | U | 3 |
| 15. | Define ‘Degree of saturation’ and write its limiting values. |  | CO5 | U | 3 |
| 16. | Explain the Brayton cycle with P–V and T–S diagrams. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Derive the expression for pdV work in various quasi static processes i) Constant pressure ii) Constant Volume iii) Isothermal iv) Polytropic | CO1 | A | 12 |
| 18. |  | Air flows steadily at the rate of 0.5 kg/s through an air compressor, entering at 7 m/s velocity, 100 kPa pressure and 0.95 m3/kg volume and leaving at 5 m/s, 700 kPa and 0.19 m3/kg. The internal energy of air leaving is 90 kJ/kg greater than that of air entering. Cooling water in the compressor jackets absorbs heat from the air at the rate of 58 kW. a) Compute the rate of shaft work input to the air in kW. b) Find the ratio of inlet pipe diameter to outlet pipe diameter. | CO2 | An | 12 |
| 19. |  | Illustrate the phase equilibrium diagram of a pure substance on an T − S diagram and analyze the significance of the saturation dome and important constant property lines. | CO3 | A | 12 |
| 20. | a. | Explain the difference between ‘Vanderwaals equation of state’ and ‘ideal gas equation of state’. | CO4 | An | 6 |
|  | b. | Apply thermodynamic relations to demonstrate that for an ideal gas Cp - Cv = R | CO4 | A | 6 |
| 21. | a. | Explain the following psychrometric processes. i) Cooling and dehumidification, ii) Adiabatic evaporative cooling. | CO5 | An | 6 |
|  | b. | Air at 20 C and 40% RH is mixed adiabatically with air at 40 C, 40 % RH in the ratio of 1 kg of former with 2 kg of the latter (on dry basis). Find the final condition of air | CO5 | A | 6 |
| 22. | a. | A cyclic heat engine operates between a source temperature of 800 C and a sink temperature of 30 C. Determine the least rate of heat rejection per kW net output of the engine. | CO2 | A | 6 |
|  | b. | A domestic food freezer maintains a temperature of -15 C. The ambient air temperature is 30 C. If heat leaks into the freezer at the continuous rate of 1.75 kJ/s what is the least power necessary to pump this heat out continuously. | CO2 | A | 6 |
| 23. |  | An Air conditioning system is designed under the following conditions. Outdoor conditions: 30 C dbt, 75% RH Required indoor conditions: 22 C dbt, 70% RH Amount of free air circulated: 3.33 m3/s Coil dew point temperature: 14 C The required condition is achieved first by cooling and dehumidification and then by heating. Estimate a) the capacity of cooling coil in tonnes b) the capacity of heating coil in kW, and c) the amount of water vapour removed in kg/s. | CO5 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the working of a compression ignition engine. Analyze the Diesel cycle and derive the expression for its air-standard efficiency. Discuss the effect of compression ratio on engine performance. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic concepts in thermodynamics and energy balance to systems and control volumes, in situations involving heat and work interactions. |
| CO2 | Differentiate between high grade and low-grade energies. |
| CO3 | Evaluate changes in thermodynamic properties of pure substances. |
| CO4 | Apply gas laws to solve problems related to gas mixtures. |
| CO5 | Create psychrometric chart to perform moist air process calculations |
| CO6 | Recognize the significance of I law for reacting systems and heating value of fuels. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 20ME2010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # KINEMATICS AND DYNAMICS OF MACHINERY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Determine the number of degrees of freedom for a five-bar mechanism. |  | CO1 | U | 1 |
| 2. | State the relationship between a multi-link joint and its equivalent binary joints. |  | CO1 | R | 1 |
| 3. | Write the expression for the radial component of acceleration. |  | CO2 | R | 1 |
| 4. | Identify the directional property of relative velocity in a rigid link. |  | CO2 | R | 1 |
| 5. | List out the material used for belts. |  | CO3 | U | 1 |
| 6. | State the concept of creep in belt drives. |  | CO3 | R | 1 |
| 7. | Identify few industrial applications of gear trains |  | CO4 | U | 1 |
| 8. | List the different types of gear trains. |  | CO4 | U | 1 |
| 9. | Describe gyroscopic couple. |  | CO6 | R | 1 |
| 10. | Differentiate ‘forced vibration’ from ‘free vibration’. |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate lower pair and higher pair. |  | CO1 | An | 3 |
| 12. | Sketch the velocity of a slider is obtained in a slider crank mechanism. |  | CO2 | A | 3 |
| 13. | List the merits and demerits of belt and rope drives for transmission of power. |  | CO3 | U | 3 |
| 14. | Compare the cycloidal and involute tooth forms. |  | CO4 | An | 3 |
| 15. | Differentiate ‘Proell’ from ‘Watt’ governor with a neat diagram. |  | CO6 | An | 3 |
| 16. | Evaluate the impact of balancing of rotating parts for high-speed engines. |  | CO5 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Determine the mobility (degrees of freedom) of the mechanisms shown in Fig. 1a, Fig. 1b and Fig. 1c using Kutzbach mobility criterion and classify them. Fig. 1a; Fig. 1b; Fig. 1c. | CO1 | A | 12 |
| 18. |  | The crank and connecting rod of a theoretical steam engine are 0.5 m and 2 m long respectively, as shown in the Fig. The crank makes 360 rpm. in the clockwise direction. When it has turned 45° from the inner dead centre position, calculate: - velocity of piston, and - angular velocity of connecting rod. | CO2 | A | 12 |
| 19. |  | Find the power transmitted by a belt running over a pulley of 600 mm diameter at 200 rpm. The coefficient of friction between the belt and the pulley is 0.25, angle of lap 160 degree and maximum tension in the belt is 2500 N. | CO3 | A | 12 |
| 20. |  | An epicyclic gear consists of three gears A, B and C as shown in Fig. The gear A has 72 internal teeth and gear C has 32 external teeth. The gear B meshes with both A and C and is carried on an arm EF which rotates about the centre of A at 18 rpm. If the gear A is fixed, determine the speed of gears B and C. | CO4 | A | 12 |
| 21. |  | A cam is to give the following motion to a knife-edged follower: 1) Outstroke during 60° of cam rotation; 2) Dwell for the next 30° of cam rotation; 3) Return stroke during next 60° of cam rotation, and 4) Dwell for the remaining 210° of cam rotation. The stroke of the follower is 40 mm and the minimum radius of the cam is 50 mm. The follower moves with uniform velocity during both the outstroke and return strokes. Draw the profile of the cam when the axis of the follower passes through the axis of the cam shaft. | CO3 | A | 12 |
| 22. |  | The following data are given for a vibratory system with viscous damping: Mass = 2.5 kg ; spring constant = 3 N/mm and the amplitude decreases to 0.25 of the initial value after five consecutive cycles. Determine the damping coefficient of the damper in the system. | CO5 | A | 12 |
| 23. |  | The arms of a porter governor are each 250 mm long and pivoted on the governor axis. The mass of each ball is 5 kg and the mass of the central sleeve is 30 kg. The radius of rotation of the ball is 150 mm when the sleeve begins to rise and reaches 200 mm for the maximum speed. Find the speed range of the governor. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Four masses A, B, C and D as shown below are to be completely balanced. The planes containing masses B and C are 300 mm apart. The angle between planes containing B and C is 90°. B and C make angles of 210° and 120° respectively with D in the same sense. Find: - The magnitude and the angular position of mass A; and - The position of planes A and D. | CO4 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic concepts of Mechanisms, Machines and their relative motions, then apply it to appropriate environments. |
| CO2 | Carry out kinematic analysis (Displacement, Velocity and Acceleration) of simple mechanisms (Single slider Crank Mechanism and four bar Mechanisms) by graphical and analytical method. |
| CO3 | Construct & Design different CAM profiles for given conditions using graphical & Theoretical methods. |
| CO4 | Apply the concept of balancing and use it for reducing the unbalanced forces in rotating masses and reciprocating engines under operating conditions exposure to IS standards. |
| CO5 | Acquire knowledge on types of vibrations in different systems and damping methods to minimize vibrations. |
| CO6 | Understand, apply and analyze the control mechanisms in Governors and Gyroscopes. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 20ME2011 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # FINITE ELEMENT METHODS IN ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Write the application of finite element analysis in Nuclear Engineering. |  | CO1 | U | 1 |
| 2. | Name any two finite element-based analysis software. |  | CO1 | R | 1 |
| 3. | Define Finite Element Analysis. |  | CO2 | R | 1 |
| 4. | State the methods of engineering analysis. |  | CO2 | R | 1 |
| 5. | List the functions of node. |  | CO3 | U | 1 |
| 6. | Classify the types of boundary conditions in FEA. |  | CO3 | R | 1 |
| 7. | Define Discretization. |  | CO4 | R | 1 |
| 8. | State the degrees of freedom. |  | CO4 | R | 1 |
| 9. | Define serendipity element. |  | CO5 | R | 1 |
| 10. | State the different modes of heat transfer. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Define weighted residual method and write its advantages. |  | CO1 | R | 3 |
| 12. | Write the properties of stiffness matrix. |  | CO2 | U | 3 |
| 13. | Define the functions of shape function. |  | CO3 | R | 3 |
| 14. | Write the purpose of the Grid Independence study. |  | CO4 | U | 3 |
| 15. | Differentiate local and global, from natural coordinate systems. |  | CO5 | An | 3 |
| 16. | Distinguish ‘static analysis’ from ‘dynamic analysis’. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Determine the nodal displacements and stresses induced in the axially loaded stepped bar shown in Figure 1. The bar has cross sectional areas A1 and A2 over the lengths L1 and L2 respectively. Assume the following datas: A1= 2cm2 and A2= 1cm2, L1=L2=10cm, E1=E2=2x107N/cm2, P=100N. 2 10 cm 10 cm 1 3 1 2 P Figure1 | CO1 | An | 12 |
| 18. |  | Consider the differential equation for the problem as ; 0 x 1; with the boundary conditions y (0) = 0 and y(1) = 0. Use one coefficient trial function as y=a1x(1-x3). Evaluate the value of parameter a1 using Galerkin’s method, point collocation method, sub domain method and least square method. | CO2 | An | 12 |
| 19. |  | Explain the step-by-step procedure of finite element analysis. | CO3 | U | 12 |
| 20. |  | S x Y 2(8,3) 1(5,3) 3 (8,5) 4(5,5) t Determine three points on the 50oC contour line for the rectangular element shown in Figure 2. The nodal values are T1=42oC, T2= 54oC, T3= 56oC and T4= 46oC. Nodal coordinates are node1 (5,3), node2 (8,3), node 3(8,5) and node4 (5,5). Figure. 2 | CO4 | An | 12 |
| 21. |  | Evaluate the element shape functions and calculate the value of temperature at point ‘P’ as shown in Figure 3. The nodal values of temperature are Ti = 80oC ; Tj = 100 oC ; Tk= 150 oC. Point ‘P’ is located at (2, 5). (All dimensions are in cm) P X Y i (1,1) j(3,4) k(2,7) Figure 3 | CO4 | An | 12 |
| 22. |  | Explain lagrangian element and establish the shape functions for any one lagrangian family of element. | CO3 | A | 12 |
| 23. |  | A wall of an industrial oven consists of three different materials. The first layer is composed of 5 cm of insulating cement with a clay binder that has a thermal conductivity of 0.08 W/m K. The second layer is made from 15 cm of asbestos board with a thermal conductivity of 0.074 W/m K. The exterior consists of 10 cm common brick with a thermal conductivity of 0.72 W/m K. The inside (inlet) wall temperature of the oven is 200°C, and the outside air is 30°C with a convection coefficient of 40 W/m 2 K(convective heat loss occurs at the right side of brick material). Determine the temperature distribution along the composite wall. | CO6 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Determine the temperature distribution in a one-dimensional fin of circular cross-section as shown in Figure 4. The fin is circular in shape, 8cm long and 1cm in diameter. Assume that convection heat loss occurs from the right end of the fin. (use 2 element idealization) ; L=8 cm 100C 1 cm diameter Figure 4. | CO5 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Acquire the fundamental theory of finite element analysis and develop characteristic equation. |
| CO2 | Derive element matrix equation by applying basic laws in mechanics and integration by parts |
| CO3 | Apply suitable boundary conditions to a global equation for field problems |
| CO4 | Analyse scalar and vector variable problems |
| CO5 | Understand the application and use FE method for solving heat transfer, fluid mechanics and structural problems |
| CO6 | Use professional level finite element software to solve engineering problems |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 20ME3011 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MATERIALS AND CHARACTERIZATION TECHNIQUES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Explain in detail the key components and operating principles of melting furnaces commonly used in foundries. | CO1 | U | 20 |
|  |  | (OR) |  |  |  |
| 2. |  | Illustrate the traditional casting methods used in foundries and explain how these techniques contribute to the production of various metal components. | CO1 | A | 20 |
| 3. |  | Explain the common casting defects encountered during alloy casting, along with their causes and remedies. | CO2 | U | 20 |
|  |  | (OR) |  |  |  |
| 4. |  | Illustrate the inspection and heat treatment methods used in foundries to enhance the quality and performance of castings. | CO2 | A | 20 |
| 5. |  | Explain the working principles of various rapid prototyping systems used in additive manufacturing. | CO3 | A | 20 |
|  |  | (OR) |  |  |  |
| 6. | a. | Distinguish the concept of resolution in light microscopy, including Airy rings, numerical aperture, magnification, and depth of field. | CO4 | An | 10 |
|  | b. | Illustrate the principles and applications of phase contrast microscopy, encompassing both bright-field and dark-field contrast techniques. | CO4 | U | 10 |
| 7. |  | Explain the applications of X-ray diffraction (XRD) in materials science, including the estimation of grain size, particle size, macrotexture, and residual stress in crystalline materials. | CO5 | U | 20 |
|  |  | (OR) |  |  |  |
| 8. |  | Illustrate the principles, construction and operation of scanning electron microscopy (SEM). | CO6 | A | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Explain the principles, construction, and working of Transmission Electron Microscopy (TEM) and describe its significance in finding microstructural features the nanoscale. | CO6 | U | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Familiarize the various melting practices |
| CO2 | Understand the different special casting processes. |
| CO3 | Know the implication of biomaterials and RP techniques. |
| CO4 | Understand the principles of light microscopy and quantity microscopy |
| CO5 | Appreciate the operating principle of X-Ray Diffraction and Diffractometry |
| CO6 | Comprehend the operating principle of SEM & TEM. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21ME2007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # COMPUTATIONAL FLUID DYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define the ‘control volume’? |  | CO1 | U | 1 |
| 2. | State the principle of conservation of energy in the context of fluid mechanics |  | CO1 | R | 1 |
| 3. | Describe the advantages of structured grids used in CFD. |  | CO2 | R | 1 |
| 4. | Write the importance of the ‘Hybrid grids’. |  | CO2 | R | 1 |
| 5. | Define consistency of a numerical method. |  | CO3 | U | 1 |
| 6. | Write the general discretized form of equation at node in 1D diffusion problem. |  | CO3 | R | 1 |
| 7. | State the requirement for a numerical scheme to be convergent. |  | CO4 | U | 1 |
| 8. | Write the significance of ‘Peclet number’? |  | CO4 | R | 1 |
| 9. | State one benefit of applying symmetry boundary conditions. |  | CO5 | U | 1 |
| 10. | TDMA algorithm stands for-------------------- |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Define divergence and write the significance? |  | CO1 | U | 3 |
| 12. | Differentiate between uniform and non-uniform grids |  | CO2 | U | 3 |
| 13. | Derive the forward difference approximation for f′(x) using Taylor series. |  | CO3 | An | 3 |
| 14. | Explain the difference between uniform flow and steady flow in fluid dynamics. |  | CO4 | U | 3 |
| 15. | Explain the importance of wall boundary conditions in CFD simulations. |  | CO5 | U | 3 |
| 16. | Justify the use of a staggered grid for incompressible flow simulations. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Derive the differential form of the continuity equation using the conservation of mass principle. | CO1 | A | 12 |
| 18. |  | Analyze the mesh generation process in CFD and classify mesh types based on their application, accuracy, and computational requirements. | CO2 | An | 12 |
| 19. |  | Derive the discretized form of equation for 2D steady diffusion problem. | CO3 | A | 12 |
| 20. |  | Derive the discretized equation for convection diffusion using central difference scheme. | CO4 | A | 12 |
| 21. |  | Explain how the following boundary conditions are implemented. 1. Outlet boundary condition 2. Periodic boundary condition 3. Pressure boundary conditions | CO5 | A | 12 |
| 22. |  | Explain i) Hybrid and Power law discretization techniques ii) QUICK scheme | CO4 | An | 12 |
| 23. |  | Derive the three-dimensional energy equation for a compressible fluid. | CO1 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain SIMPLE algorithm and derive the equation for pressure correction. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Formulate the required governing equations for flow and heat transfer problems. |
| CO2 | Identify suitable grids for computing |
| CO3 | Discretize the governing equations of flow and heat transfer problems |
| CO4 | Solve the diffusion equations |
| CO5 | Develop a suitable finite volume method for the convection diffusion problems |
| CO6 | Use appropriate algorithms to solve the discretized equations. |

SUPPLEMENTARY EXAMINATION – JUNE 2026

| # Course Code | # 21ME2009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # APPLICATION OF MACHINE LEARNING FOR MECHANICAL ENGINEERING SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define the term Artificial Intelligence. |  | CO1 | R | 1 |
| 2. | Express the significance of data statistics in Machine Learning. |  | CO1 | U | 1 |
| 3. | Define multiple regression. |  | CO2 | R | 1 |
| 4. | "DFS" in search techniques stands for ______. |  | CO2 | U | 1 |
| 5. | Define regularization in the context of Machine Learning. |  | CO3 | R | 1 |
| 6. | State the importance of linear classifiers. |  | CO3 | R | 1 |
| 7. | Compare hard clustering with soft clustering. |  | CO4 | U | 1 |
| 8. | Define ‘hierarchical clustering’. |  | CO4 | R | 1 |
| 9. | Classify the types of Recurrent Neural Networks (RNNs). |  | CO5 | U | 1 |
| 10. | Quote any two examples for reciprocating machines. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Describe the significance of search techniques in Artificial Intelligence. |  | CO1 | U | 3 |
| 12. | Describe Lasso regression highlighting its key features. |  | CO2 | U | 3 |
| 13. | Express the primary functions of decision trees. |  | CO3 | U | 3 |
| 14. | Indicate the main objectives of the K-means clustering algorithm |  | CO4 | U | 3 |
| 15. | Describe the functions of the three main gates in LSTM cell. |  | CO5 | U | 3 |
| 16. | Describe the benefits of condition monitoring in machining processes. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain Machine Learning highlighting its types and applications. | CO1 | A | 6 |
|  | b. | Analyze the common problems that Artificial Intelligence aims to solve and provide examples. | CO1 | An | 6 |
| 18. | a. | Explain simple linear regression and describe its key assumptions. | CO2 | A | 6 |
|  | b. | Describe the nearest neighbors algorithm and indicate its applications. | CO2 | U | 6 |
| 19. | a. | Compare the differences between Linear Regression and Logistic Regression with their key characteristics. | CO3 | An | 6 |
|  | b. | Explain overfitting in machine learning highlighting its preventive measures. | CO3 | A | 6 |
| 20. |  | Analyze the role of clustering algorithms in market segmentation with a detailed example in applying these techniques to optimize its marketing strategies. | CO4 | An | 12 |
| 21. |  | Explain the architecture of Convolutional Neural Networks (CNNs) and describe their applications in image processing tasks. | CO5 | A | 12 |
| 22. |  | Illustrate the working mechanism of a Recurrent Neural Network (RNN) and describe its applications in handling sequential data. | CO5 | A | 12 |
| 23. |  | Analyze the role of data analytics in improving prognostics and condition monitoring of machines on maintenance strategies and performance optimization. | CO6 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Compare different condition monitoring techniques used for rotary and reciprocating machines highlighting their differences, similarities, and specific applications. | CO6 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Obtain the basics terminologies of artificial intelligence and machine learning. |
| CO2 | Formulate and evaluate the prediction models using supervised learning algorithms. |
| CO3 | Design and analyses the models using unsupervised learning algorithms. |
| CO4 | Understand the basics of clustering and develop prediction model. |
| CO5 | Understand the basics of deep learning. |
| CO6 | Applying the concept of machine learning and deep learning in mechanical engineering related problems. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21ME3002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # TOTAL QUALITY MANAGEMENT | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. | a. | Compare ‘product quality’ with ‘service quality’ with respect to their attributes. | CO1 | An | 10 |
|  | b. | Explain the basic concepts and framework of Total Quality Management. | CO1 | A | 10 |
|  |  | (OR) |  |  |  |
| 2. | a. | Explain the contributions of Deming and Juran to quality management. | CO2 | A | 10 |
|  | b. | Analyze the different types of quality costs and their impact on organizations. | CO2 | An | 10 |
| 3. | a. | Explain the role of quality councils in promoting employee involvement, motivation, and empowerment. | CO3 | A | 10 |
|  | b. | Analyse the significance and applications of the seven traditional tools of quality. | CO3 | An | 10 |
|  |  | (OR) |  |  |  |
| 4. | a. | Appraise the role PDCA cycle, 5S, and Kaizen in Total Quality Management. | CO3 | An | 10 |
|  | b. | Articulate the objectives and procedure of the ‘benchmarking process’. | CO3 | A | 10 |
| 5. |  | Survey the role of ‘Fault Tree Analysis (FTA)’ in failure analysis. | CO4 | An | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Appraise the role of ‘Failure Mode, Effects, and Critical Analysis (FMECA)’ in failure analysis. | CO4 | An | 20 |
| 7. | a. | Evaluate the role of control charts and process capability index in the quality assurance. | CO4 | E | 10 |
|  | b. | Assess the ‘Quality Function Development (QFD)’ process with the house of quality table. | CO4 | E | 10 |
|  |  | (OR) |  |  |  |
| 8. | a. | Illustrate Total Productive Maintenance (TPM) concepts, | CO5 | An | 10 |
|  | b. | Articulate the need for ISO 9000 certification also list ISO 9000 Series of quality standards | CO5 | A | 10 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Summarise the ‘Axes of Quality 4.0’ towards the success of Industry 4.0 | CO6 | E | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Apply the tools and techniques of TQM in manufacturing and service sectors. |
| CO2 | Assess the barriers to TQM implementation. |
| CO3 | Formulate and implement quality circles in their workplace. |
| CO4 | Apply Six Sigma concepts in manufacturing and service sectors. 5. |
| CO5 | Apply TPM principles in manufacturing sectors. |
| CO6 | Validate the role of Quality 4.0 in Industry 4.0 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 21ME3017 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DIGITAL MANUFACTURING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. |  | Evaluate the role of digital manufacturing technologies in enhancing each stage of the product life cycle management process with a neat sketch. | CO1 | E | 16 |
| 2. |  | Analyze the architectural components of a digital manufacturing system and their collaboration for real-time monitoring, control and optimization. | CO2 | An | 16 |
| 3. |  | Analyze virtual prototyping in product development for design validation, optimization and cross-functional collaboration. | CO3 | An | 16 |
| 4. |  | Evaluate intelligent multi-information sensing and fusion in digital manufacturing systems for efficiency and quality improvement. | CO4 | E | 16 |
| 5. |  | Analyze RFID, QR code and barcode technologies for product identification, tracking and traceability in manufacturing. | CO5 | An | 16 |
| 6. |  | Design a comprehensive implementation strategy for integrating digital twin technology into an existing manufacturing facility with real time examples. | CO1 | C | 16 |
| 7. |  | Explain the 4D printing in digital manufacturing and its applications in biomedical engineering and consumer products. | CO6 | A | 16 |
| PART – B (1 X 20 = 20 MARKS) [Compulsory Question] |  |  |  |  |  |
| 8. |  | Analyze fused deposition modeling technology in direct digital manufacturing with examples of products. | CO6 | An | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Learn the Design processes and methods in product development. |
| CO2 | Get a basic knowledge on the importance of digital manufacturing. |
| CO3 | Understand the digital transformation in manufacturing. |
| CO4 | Implement decision knowledge in manufacturing. |
| CO5 | Integrate the digital technologies in product life cycle. |
| CO6 | Know the additive manufacturing technologies used in digital manufacturing. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23ME1003 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # INNOVATION AND CREATIVITY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Name the process that generates unique and useful ideas. |  | CO1 | R | 1 |
| 2. | State one common barrier to creativity in organisations. |  | CO1 | R | 1 |
| 3. | Define ‘Empathising’. |  | CO2 | R | 1 |
| 4. | List one example of technological innovation. |  | CO2 | R | 1 |
| 5. | Cite ‘creative problem-solving’. |  | CO3 | U | 1 |
| 6. | Infer ‘mind mapping’. |  | CO3 | U | 1 |
| 7. | Identify an employee who innovates within an entrepreneurial venture. |  | CO4 | U | 1 |
| 8. | Name any two successful women entrepreneurs in India. |  | CO4 | R | 1 |
| 9. | State the key factor of entrepreneurship. |  | CO5 | R | 1 |
| 10. | Distinguish an ‘entrepreneur’ from a ‘manager’. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Describe how individuals can enhance their creativity. |  | CO1 | U | 3 |
| 12. | Describe the importance of innovation in entrepreneurship. |  | CO2 | U | 3 |
| 13. | Cite the barriers to innovation and creativity. |  | CO3 | U | 3 |
| 14. | Differentiate a ‘patent’ from a ‘trade secret’. |  | CO4 | U | 3 |
| 15. | Explain the importance of ‘Intellectual Property Rights (IPRs)’. |  | CO5 | A | 3 |
| 16. | Describe the role of a social entrepreneur and explain how they contribute to solving social and environmental challenges. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the barriers to innovation and creativity with suitable examples | CO1 | A | 12 |
| 18. | a. | Compare the ‘traditional problem-solving approach’ with the ‘design thinking process’. | CO2 | A | 6 |
|  | b. | Explain the advantages of open innovation. | CO2 | A | 6 |
| 19. |  | Appraise the various tools used for fostering creativity in the creative process. | CO3 | An | 12 |
| 20. |  | Illustrate the different ways of improving empathy. | CO4 | An | 12 |
| 21. |  | Articulate the role of government initiatives in promoting entrepreneurship. | CO4 | A | 12 |
| 22. |  | Analyse the impact of technology, innovation, and social entrepreneurship on business models and growth. | CO5 | An | 12 |
| 23. |  | Discuss a few success and failure stories of women entrepreneurs in India and their key takeaways. | CO6 | An | 12 |
| 24. |  | Evaluate the prospects of the entrepreneurship journey in India. | CO6 | E | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design innovative systems with enhanced performance. |
| CO2 | Devise innovative techniques for optimizing the system's performance. |
| CO3 | Develop new products for industrial applications. |
| CO4 | Collaborate with external partners and stakeholders to develop new products. |
| CO5 | Design new processes and methodologies to improve system performance. |
| CO6 | Evolve creative technology for innovative entrepreneurial ventures |

END SEMESTER EXAMINATION – APR / MAY 2026

| # Course Code | # 23ME1004 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # INDUSTRIAL ROBOTICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define the Laws of Robotics. |  | CO1 | R | 1 |
| 2. | List any four components of a Robot. |  | CO1 | R | 1 |
| 3. | In robotics, RR represents_______________. |  | CO2 | R | 1 |
| 4. | DH notation represents__________________. |  | CO2 | R | 1 |
| 5. | Identify the different generations of robot programming. |  | CO3 | R | 1 |
| 6. | Name three key positions in space used for robot programming. |  | CO3 | R | 1 |
| 7. | Define robot perception. |  | CO4 | R | 1 |
| 8. | The basic purpose of a robot vision system is to identify________. |  | CO4 | U | 1 |
| 9. | Name four key sensors used in robots for manufacturing applications. |  | CO5 | R | 1 |
| 10. | Distinguish a ‘robot’ from a ‘cobot’. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Describe the different types of industrial robots with examples. |  | CO1 | R | 3 |
| 12. | Describe homogeneous transformations and their representation using matrices. |  | CO2 | R | 3 |
| 13. | Define Joint Space and Cartesian Space. |  | CO3 | R | 3 |
| 14. | Write the steps to recognize the 2D to 3D in robotics perception. |  | CO4 | A | 3 |
| 15. | Explain the importance of sound sensors in robot applications. |  | CO5 | A | 3 |
| 16. | Write about Human Robot Interaction (HRI). |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain the different types of automation and their applications. | CO1 | An | 12 |
| 18. | a. | Illustrate in detail the sketches of the 2 DOF and Reverse 2 DOF. | CO2 | A | 4 |
|  | b. | Explain in detail with the sketches of the 3 DOF and 4 DOF. | CO2 | A | 8 |
| 19. | a. | Write the sequence of commands for fetching and placing the object using branching commands. | CO3 | A | 6 |
|  | b. | The robot must pick up the parts from an incoming chute and deposit them onto a pallet. The plane of the pallet is assumed to be parallel to the xy plane. The rows of the pallet are parallel to the x axis and the columns of the pallet are parallel to the y axis. Figure shows the arrangement of the pallet . The objects to be picked up are about 25 mm tall (approximately 1.0 in). Use the variables as ROW,COLUMN, X,Y for solving the program. | CO3 | An | 6 |
| 20. |  | Explain the working principle of a robot vision system, and its components, and provide a clear sketch along with its applications. | CO4 | An | 12 |
| 21. |  | Describe the working principle of different types of robot control systems, their components, and applications with a clear sketch. | CO5 | An | 12 |
| 22. |  | Explain in detail the different types of robot configurations with neat sketches. | CO1 | An | 12 |
| 23. |  | Articulate the WAIT, SIGNAL, and DELAY commands in detail with examples. | CO3 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the working principles of different types of mobile robots and unmanned aerial vehicles (UAVs). | CO6 | An | 12 |

CO – COURSE OUTCOME	 BL – BLOOM’S LEVEL  M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design industrial robots specific to applications. |
| CO2 | Implement robot dynamics and control algorithms. |
| CO3 | Develop robot programming for various applications. |
| CO4 | Design robot vision system for image processing and object tracking. |
| CO5 | Use different control algorithms for optimizing the performance of robots. |
| CO6 | Create collaborative robots for industrial applications. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23ME2011 | # Duration | # 3 hrs |
| --- | --- | --- | --- |
| # Course Title | # STRENGTH OF MATERIALS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define factor of safety. |  | CO1 | R | 1 |
| 2. | Differentiate between brittle and ductile materials. |  | CO1 | U | 1 |
| 3. | Define bending moment at a section of a beam. |  | CO2 | R | 1 |
| 4. | State the shape of the BMD for a Cantilever beam subjected to a UDL. |  | CO2 | R | 1 |
| 5. | Tabulate the ymax value for a circular section of diameter D. |  | CO3 | R | 1 |
| 6. | Differentiate between the behavior of the "top fiber" and "bottom fiber" in a simply supported beam under a downward load. |  | CO3 | U | 1 |
| 7. | Define Polar modulus. |  | CO4 | R | 1 |
| 8. | Identify the primary advantage of Macaulay’s Method over the standard double integration method. |  | CO5 | U | 1 |
| 9. | Define Radius of gyration (k). |  | CO6 | R | 1 |
| 10. | Recognize the physical meaning of a "Short Column". |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | For a material, the Young’s modulus is given as 1.2 x 104 N/mm2 and the Poisson’s ratio is 0.25. Calculate the Bulk Modulus. |  | CO1 | A | 3 |
| 12. | List the types of loads on a beam. |  | CO2 | R | 3 |
| 13. | Calculate the section modulus for a rectangular beam of width 100 mm and depth of 200 mm. |  | CO3 | A | 3 |
| 14. | State the assumptions in theory of pure torsion. |  | CO4 | R | 3 |
| 15. | Compare thin cylindrical and spherical shells. |  | CO5 | U | 3 |
| 16. | Differentiate between the failure modes of a “Long column” and a “Short column”. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | A steel rod ABCD 4.5 m long and 25 mm in diameter is subjected to forces as shown in Fig 1. If the value of Young’s Modulus for steel is 200 GPA, estimate its deformation. Fig 1 | CO1 | An | 12 |
| 18. | a. | A cantilever 5 m long is loaded as shown in the Fig 2. Draw the shear force and bending moment diagrams. Fig 2 | CO2 | An | 12 |
| 19. | a. | A square beam 20 mm x 20 mm in section and 2 m long is supported at the ends. The beam fails when a point load of 400 N is applied at the centre of the beam. Evaluate the value of uniformly distributed load per metre length that will break a cantilever of the same material 40 mm wide; 60 mm deep and 3 m long. | CO3 | An | 12 |
| 20. | a. | Calculate the diameter of a solid shaft which will transmit 300 kW at 250 rpm. The maximum shear stress should not exceed 30 N/mm2; and the twist should not be more than 1˚ in a shaft length of 2 m. Take modulus of rigidity as 1 x 105 N/mm2. | CO4 | A | 12 |
| 21. | a. | A horizontal steel girder having a uniform cross-section is 14 m long and is simply supported at its ends It carries two concentrated loads as shown in Fig 3. Estimate the deflection of the beam under the load C. Take E = 200 GPa and I = 160 x 106 mm4. Fig 3 | CO5 | An | 12 |
| 22. | a. | A specimen of steel 20 mm diameter with a gauge length of 200 mm is tested to destruction. It has an extension of 0.25 mm under a load of 80 kN and the load at elastic limit is 102 kN. The maximum load is 130 kN. The total extension at fracture is 56 mm and diameter at neck is 15 mm. Determine: (i) The stress at elastic limit; (ii) Young’s modulus; (iii) Percentage elongation; and (iv) Percentage reduction in area. | CO1 | A | 12 |
| 23. | a. | Calculate: (i) the change in diameter; (ii) change in length and (iii) change in volume of a thin cylindrical shell 1000 mm diameter; 10 mm thick and 5 m long when subjected to an internal pressure of 3 N/mm2. Take the value of E = 2 x 105 N/mm2 and Poisson’s ratio µ = 0.3. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | A hollow cast iron column 200 mm outside diameter and 150 mm inside diameter, and 8 m long has both its ends fixed. It is subjected to an axial compressive load. Taking a factor of safety as 6; σc = 560 N/mm2; α = 11600 ; estimate the safe Rankine load. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze various types of stress and strains at different loading conditions. |
| CO2 | Apply shear force and bending moment diagrams for different types of beams subjected to diverse loading conditions. |
| CO3 | Evaluate bending stresses in beams with varying cross-sectional geometries. |
| CO4 | Analyze shear stresses, angles of twist in shafts experiencing torsion, and the strain energy stored in elastic materials. |
| CO5 | Predict deflection behavior of beams and determine stresses in cylindrical and spherical shells. |
| CO6 | Apply pertinent theories and principles to proficiently assess and design columns and struts. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23ME2012 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # KINEMATICS AND DYNAMICS OF MACHINERY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define kinematic chain. |  | CO1 | R | 1 |
| 2. | Illustrate a second-class lever with a neat sketch. |  | CO1 | U | 1 |
| 3. | Draw the equivalent diagram for the following mechanism. |  | CO2 | U | 1 |
| 4. | Define rubbing velocity at a pin joint. |  | CO2 | R | 1 |
| 5. | Construct the displacement diagram for a simple harmonic motion of the follower (assume appropriate data needed). |  | CO3 | U | 1 |
| 6. | Classify different types of follower motions. |  | CO3 | U | 1 |
| 7. | Outline two advantages of gear drives. |  | CO4 | R | 1 |
| 8. | List two industrial applications spur gears. |  | CO4 | R | 1 |
| 9. | Differentiate between Inertia Force and Inertia Torque. |  | CO5 | U | 1 |
| 10. | List four types of governors used in mechanical systems. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate machine and mechanism. |  | CO1 | U | 3 |
| 12. | Construct the Klein’s Velocity diagram, for a single slider crank mechanism. (Assume appropriate dimensions). |  | CO2 | R | 3 |
| 13. | Illustrate different types of followers with suitable sketches. |  | CO3 | A | 3 |
| 14. | Classify gears and illustrate the terminology of a Toothed Wheel. |  | CO4 | U | 3 |
| 15. | Justify the necessity of the dynamic analysis of a reciprocating engine. |  | CO5 | U | 3 |
| 16. | Illustrate the function of Watt-Centrifugal governor of Steam engine system. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Illustrate the following with suitable sketches: - Kinematic Pairs - Flow chart showing how kinematic pairs make up a machine. | CO1 | An | 8 |
|  | b. | Compare the inversions of a four-bar chain in terms of mobility, transmission angle, and practical usability. | CO1 | An | 4 |
| 18. |  | In a four-bar chain ABCD, AD is fixed and is 120 mm long. The crank AB is 45 mm long and rotates at 200 r.p.m. clockwise, while the link CD = 80 mm oscillates about D. BC and AD are of equal length. Draw the velocity polygon and determine: 1. Velocity of crank, 2. the angular velocity of link CD when angle BAD = 60°. | CO2 | A | 12 |
| 19. |  | A cam, with a minimum radius of 25 mm, rotating clockwise at a uniform speed is to be designed to give a roller follower, at the end of a valve rod, motion described below: 1. To raise the valve through 50 mm during 120° rotation of the cam; 2. To keep the valve fully raised through next 30°; 3. To lower the valve during next 60°; and 4. To keep the valve closed during rest of the revolution i.e. 150°; The diameter of the roller is 20 mm and the diameter of the cam shaft is 25 mm. Draw the profile of the cam when (a) the line of stroke of the valve rod passes through the axis of the cam shaft, and (b) the line of the stroke is offset 15 mm from the axis of the cam shaft. The displacement of the valve, while being raised and lowered, is to take place with simple harmonic motion. Determine the maximum acceleration of the valve rod when the cam shaft rotates at 100 r.p.m. Draw the displacement, the velocity and the acceleration diagrams for one complete revolution of the cam. | CO3 | A | 12 |
| 20. |  | Two gear wheels mesh externally and are to give a velocity ratio of 3. The teeth are of involute form of module 6. The standard addendum is 1 module. If the pressure angle is 18° and pinion rotates at 90 r.p.m., determine: 1. the number of teeth on each wheel, so that the interference is just avoided, 2. the length of the path of contact, and 3. the maximum velocity of sliding between the teeth. | CO4 | A | 12 |
| 21. |  | The speed ratio of the reverted gear train, as shown in Figure.C.1, is to be 12. The module pitch of gears A and B is 3.125 mm and of gears C and D is 2.5 mm. Calculate the suitable numbers of teeth for the gears. No gear is to have less than 24 teeth. Figure.C.1 | CO4 | A | 12 |
| 22. |  | The connecting rod Figure.C.2 of a gasoline engine is 300 mm long between its centres. It has a mass of 15 kg and mass moment of inertia of 7000 kg-mm2. Its centre of gravity is at 200 mm from its small end centre. Determine the dynamical equivalent two-mass system of the connecting rod if one of the masses is located at the small end centre. Figure.C.2 | CO5 | An | 12 |
| 23. |  | State the following with respect to centrifugal governor: - Controlling force - Height of a governor - Equilibrium speed - Mean equilibrium speed - Maximum and minimum equilibrium - speeds. - Sleeve lift. - Sensitiveness of Governors - Stability of Governors - Isochronous Governors - Hunting - Effort and Power of a Governor | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the concept of gyroscopic couple and derive the formula for its magnitude. | CO6 | A | 4 |
|  | b. | A uniform disc of diameter 300 mm and of mass 5 kg is mounted on one end of an arm of length 600 mm. The other end of the arm is free to rotate in a universal bearing. If the disc rotates about the arm with a speed of 300 r.p.m. clockwise, looking from the front, with what speed will it precess about the vertical axis? | CO6 | A | 8 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

| COURSE OUTCOMES |  |
| --- | --- |
| The student will be able to |  |
| CO1 | Apply different mechanisms to devise solutions for real-world engineering challenges. |
| CO2 | Analyze and resolve relative motion problems in kinematics and dynamics. |
| CO3 | Design CAM mechanisms for input/output motion transmission. |
| CO4 | Design various types of gears to diverse applications. |
| CO5 | Implement various balancing techniques to mitigate vibration and enhance the efficiency of dynamic systems. |
| CO6 | Design governors and gyroscopes to analyze stability control, navigation systems, and the regulation of machinery speed in vehicles. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23ME2013 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # APPLIED THERMODYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State one advantage of using the super heater in a boiler plant |  | CO1 | R | 1 |
| 2. | Define equivalent of evaporation |  | CO1 | R | 1 |
| 3. | Sketch the Temperature – Entropy diagram of a Reheat Rankine cycle |  | CO2 | R | 1 |
| 4. | Draw the T-S diagram of the Otto Cycle |  | CO2 | R | 1 |
| 5. | Write the relation to find the outlet velocity of steam flowing in a convergent nozzle |  | CO5 | A | 1 |
| 6. | Dryness fraction of steam along the saturated vapour line is ____________ |  | CO3 | R | 1 |
| 7. | Cite one difference between impulse turbine and reaction turbine |  | CO4 | U | 1 |
| 8. | Define clearance volume in a piston cylinder device |  | CO5 | R | 1 |
| 9. | Write the relation to find the mean effective pressure of air compressor |  | CO4 | A | 1 |
| 10. | List the components used in the vapour compression refrigeration system |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Describe factor of evaporation and calorific value of the fuel |  | CO1 | U | 3 |
| 12. | Determine the thermal efficiency relation for the Rankine cycle |  | CO2 | A | 3 |
| 13. | Determine the relation to find the outlet velocity of the steam flowing in a C-D nozzle |  | CO3 | A | 3 |
| 14. | List any three differences between the impulse turbine and reaction turbine |  | CO4 | R | 3 |
| 15. | Classify the different types of nozzles. |  | CO5 | U | 3 |
| 16. | Sketch the P-v and T-S diagram of a vapour compression refrigeration cycle with the condition of the refrigerant as dry and saturated before compression |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | A Lancashire boiler generates 2400 kg of dry steam per hour at a pressure of 11 bar. The grate area is 3m2 and 90 kg of coal is burnt per m2 of grate area per hour. The calorific value of the coal is 33180 kJ/kg and the temperature of the feed water is 17.5oC. Evaluate (i) Actual evaporation per kg of coal; (ii) Equivalent of evaporation; (iii) Factor of evaporation; (iv) Efficiency of boiler | CO1 | A | 12 |
| 18. |  | A simple Rankine cycle works between the pressure of 28 bar and 0.06 bar. The initial condition of steam being dry saturated. Evaluate the (i) cycle efficiency; (ii) Work ratio; and (iii) Specific steam consumption. | CO2 | A | 12 |
| 19. |  | Explain the working principle of a Regenerative Rankine Cycle and determine the expression to calculate the efficiency with a schematic layout of the cycle and the Temperature – Entropy diagram | CO2 | An | 12 |
| 20. |  | Derive an expression to determine the air standard efficiency of Otto cycle | CO3 | A | 12 |
| 21. |  | Dry saturated steam at a pressure of 11 bar enters a convergent-divergent nozzle and leaves the nozzle at a pressure of 2 bar isentropically. Evaluate the exit velocity of the steam and the ratio of cross section area of the exit and the throat. Use the Mollier Chart to solve this problem. | CO5 | A | 12 |
| 22. |  | The velocity of steam leaving the nozzle of an impulse turbine is 1200 m/s and the nozzle angle is 20o. The blade velocity is 400 m/s and the blade velocity coefficient is 0.8. Assuming a mass flow rate of 0.5 kg/s and symmetric blading and no loss due to shock at the inlet, Evaluate (i) Blade inlet angle; (ii) Tangential force on blades; (iii) Power developed; (iv) Blade efficiency; and (v) Relative velocity at inlet and outlet. | CO5 | A | 12 |
| 23. |  | Explain the working principle of a single stage reciprocating air compressor without the clearance volume with a neat sketch and determine the relation to calculate the work done during the polytropic compression process in terms of pressure ratio. | CO4 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the working principle of a vapour compression refrigeration system with a neat schematic layout along with Temperature – Entropy and Pressure – Enthalpy diagram with the condition of dry saturated refrigerant at the end of compression and determine the relation to find the Coefficient of Performance (COP) of the system. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Evaluate boiler efficiency by conducting heat balance calculations and identifying heat losses. |
| CO2 | Apply Rankine cycle concepts to assess performance and efficiency in systems with superheat, reheat, and regeneration. |
| CO3 | Evaluate the thermal efficiency of air standard cycles, including Otto, Diesel, Dual, and Brayton cycles |
| CO4 | Assess the impact of intercooling on reciprocating compressor performance and determine minimum work requirements. |
| CO5 | Design and optimize nozzle performance for various applications |
| CO6 | Analyse the vapour compression refrigeration cycle, considering superheat and sub cooling in performance calculations. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23ME2014 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ENGINEERING ECONOMICS AND OPERATIONS RESEARCH | # Max. Marks | # 100 |

The standard normal distribution table is permitted

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State the applications of engineering economics. |  | CO1 | R | 1 |
| 2. | Infer the use of break-even point in the make or buy decision. |  | CO1 | U | 1 |
| 3. | Compare ‘total cost’ with ‘marginal cost’. |  | CO2 | U | 1 |
| 4. | In a project, the total sales revenue is 72,500. The total production cost is Rs 60,000. Calculate the total profit of the project. |  | CO2 | A | 1 |
| 5. | Compare ‘slack variable’ with ‘surplus variable’. |  | CO3 | U | 1 |
| 6. | State the limitation of the graphical method in solving LPP. |  | CO3 | R | 1 |
| 7. | State the three methods to arrive at an initial solution in the transportation problem. |  | CO4 | R | 1 |
| 8. | Articulate the steps to balance an unbalanced transportation problem. |  | CO4 | A | 1 |
| 9. | Infer the critical path of the following activities of a project . Activities; 1-2; 1-3; 2-4; 3-4. Duration (Weeks); 4; 6; 5; 2. | CO6 | U | 1 |
| 10. | Sketch a ‘dummy activity’ in a network. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | State the causes of depreciation. |  | CO1 | R | 3 |
| 12. | In a manufacturing plant, the fixed cost is computed as Rupees 4000, unit selling price is Rupees 100 and unit variable cost is Rupees 70. Calculate the break-even quantity in number of units . |  | CO2 | A | 3 |
| 13. | Summarize the applications, advantages and limitations of LPP. |  | CO3 | U | 3 |
| 14. | Change the following transportation table into a balanced one. Plant 1 and Plant 2 are the sources, with availabilities of 60 and 40 units, respectively. Warehouse A and Warehouse B are the destinations with their requirements of 80 and 50 units, respectively. Inner cells represent unit transportation cost. Warehouse A; Warehouse B; Availability. Plant 1; 3; 5; 60. Plant 2; 8; 12; 40. Requirement; 80; 50. | CO4 | A | 3 |
| 15. | Compute the ‘row-wise reduction’ in the following Table A; B; C. I; 18; 15; 13. II; 5; 0; 3. III; 17; 8; 9. | CO5 | A | 3 |
| 16. | Determine the critical path of the following project using a network diagram. Activities; 1-2; 1-3; 2-4; 3-4; 4-6. Immediate Predecessor; Nil; Nil; 1-2; 1-3; 3-4 and 2-4. Duration(Weeks); 3; 4; 5; 2; 6. | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Interpret the various factors which influence demand | CO1 | A | 6 |
|  | b. | Explain about i)Perfectly elastic demand ii) Perfectly inelastic demand iii) Unit elasticity of demand | CO1 | A | 6 |
| 18. | a. | Explain the structure of the “balance sheet” with an example and also about ratio analysis. | CO2 | A | 6 |
|  | b. | Explain the straight-line method of calculating depreciation. | CO2 | A | 6 |
| 19. |  | Solve the following LPP by the graphical method Minimize Z=30XI+20X2 Subjected to the constraints 5XI+X2≥10 2X1+2X2≥12 X1+4X2≥12 X1, X2≥0 | CO3 | A | 12 |
| 20. |  | A food manufacturing company has 2 processing plants P1 and P2; three fruit cultivators are willing to supply fruits in the following quantities. Cultivator C1 is willing to supply 190 quintal at the rate of Rupees 100 per quintal, Cultivator C2 is willing to supply 290 quintal at the at the rate of Rupees 90 per quintal, Cultivator C3 is willing to supply 390 quintal at the rate of Rupees 80 per quintal. Cost of transportation from cultivator to plants is given below P1; P2. C1; 20; 26. C2; 9; 16. C3; 49; 31. Plant requirements and labour costs are as follows Details; Plant P1; Plant P2. Requirement in Quintal; 440; 360. Labour cost - Rupees/Quintal; 30; 22. Processed fruits are sold at the rate of Rupees 480 per quintal objective of this problem to maximize profit. Infer the initial solution by North-west corner method. | CO4 | An | 12 |
| 21. |  | In a small machine shop, there are 4 operators available to assign jobs for the day. There are 5 jobs and profit in rupees for each operator on each job is represented in the table JI; J2; J3; J4; J5. O 1; 6.30; 5.90; 6.80; 10.20; 7.40. O 2; 8.00; 10.10; 9.00; 8.50; 7.30. O 3; 8.70; 8.80; 9.10; 7.60; 6.50. O 4; 7.30; 8.30; 6.40; 7.70; 8.00. Evaluate the optimal assignment and find out which job is to be rejected. | CO5 | E | 12 |
| 22. |  | 10 activities of a project and their time duration is given in the following table. Compute forward and backward scheduling and also compute the total float of all the activities Activities; 1-2; 2-3; 2-4; 3-5; 3-6. Duration (In days); 2; 3; 5; 4; 1. Activities; 4-6; 4-7; 5-8; 6-8; 7-8. Duration (In days); 6; 2; 8; 7; 4. | CO6 | A | 12 |
| 23. |  | The activities of a project with their respective time estimates (in weeks) are given in the following table. Activities; 1-2; 1-6; 2-3; 2-4; 3-5; 4-5; 6-7; 5-8; 7-8. Optimistic time; 3; 2; 6; 2; 5; 3; 3; 1; 4. Most Probable Time; 6; 5; 12; 5; 11; 6; 9; 4; 9. Pessimistic time; 15; 14; 30; 8; 17; 15; 27; 7; 28. i) Draw the network of this project and compute duration, variance and standard deviation of the project (8) ii) Appraise the probability of completing the project within 42 weeks(2) iii) Appraise the probability of NOT completing the project within 40 weeks (2) | CO6 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Various activities and their duration of a project are represented in the above network. Determine the different paths and their duration and also critical path and its duration | CO6 | A | 6 |
|  | b. | Articulate about the three-time estimates adopted in the PERT model and the formula to find i) the expected time of an activity, ii) the variance of an activity iii) the standard deviation of an activity. | CO6 | A | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Evaluate the project's feasibility and financial decisions. |
| CO2 | Analyse financial statements, including balance sheets, income and cash flow statements. |
| CO3 | Apply graphical and simplex methods to solve linear programming problems. |
| CO4 | Develop transportation models for minimising transportation costs or maximizing efficiency. |
| CO5 | Evaluate and compare different approaches within assignment models |
| CO6 | Demonstrate PERT, CPM, and scheduling techniques to manage project schedules efficiently. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23ME2015 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # PIPING DESIGN AND INSTRUMENTATION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define 'plastic instability' in the context of piping materials. |  | CO1 | R | 1 |
| 2. | Recall Darcy-Weisbach equation used in pressure drop calculations. |  | CO1 | R | 1 |
| 3. | Write a short note on Piping and Instrumentation Diagram. |  | CO1 | U | 1 |
| 4. | List any two criteria used for material selection in piping design. |  | CO2 | R | 1 |
| 5. | Define 'pipe routing' and state its objective. |  | CO2 | U | 1 |
| 6. | State Distributed Control System (DCS). |  | CO3 | R | 1 |
| 7. | Distinguish between PLC and SCADA in one sentence each. |  | CO3 | U | 1 |
| 8. | State the importance of response time in instrumentation selection. |  | CO4 | R | 1 |
| 9. | Define 'leakage evaluation' in piping system performance assessment. |  | CO5 | R | 1 |
| 10. | Outline the objective function in piping system design optimization. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | State the relationship between stress, strain, and plastic instability in piping materials with a neat sketch. |  | CO1 | R | 3 |
| 12. | Define the role of fluid mechanics fundamentals in basic pipe sizing calculations. |  | CO1 | R | 3 |
| 13. | Compare the piping layout and routing considerations for oil and gas industries versus general industrial piping. |  | CO2 | U | 3 |
| 14. | Write a short note on the working principle of a Safety Instrumented System (SIS) in piping systems. |  | CO3 | R | 3 |
| 15. | List key factors for selecting instrumentation devices with respect to environmental and operating conditions. |  | CO4 | R | 3 |
| 16. | Recall the methods used to monitor flow rate and pressure in piping system performance assessment. |  | CO5 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain how thermodynamic principles and heat transfer basics influence the design of an industrial piping system with a suitable schematic diagram. | CO1 | U | 12 |
| 18. |  | Explain compliance with industry codes and standards in piping design. | CO1 | A | 12 |
| 19. |  | Apply fluid dynamics principles to analyze the flow behavior in a piping network. Perform pipe sizing calculations for a pipeline carrying water at a flow rate of 0.05 m³/s through a 50 m long horizontal pipe (assume suitable friction factor). Verify structural integrity through stress analysis. | CO2 | A | 12 |
| 20. |  | Discuss DCS, PLC, HMI, and SCADA systems and explain how they work together to achieve safe and efficient process control. | CO3 | An | 12 |
| 21. |  | Evaluate the performance of a control valve selection process for a natural gas pipeline. Justify the selection criteria. | CO4 | E | 12 |
| 22. | a. | Analyze the causes of flow and pressure irregularities in a steam distribution piping system | CO5 | An | 6 |
|  | b. | Explain the methods for identifying heat transfer and insulation issues in a piping system. | CO5 | U | 6 |
| 23. | a. | Discuss the optimization of piping system layout and routing for energy efficiency. | CO6 | U | 6 |
|  | b. | Explain how the objective function is formulated considering material costs, pressure drop, and structural integrity constraints. | CO6 | U | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Develop a design strategy for integrating control systems into an optimized piping system that complies with industry standards. Support your answer with a block diagram showing the integration of DCS, SIS, and instrumentation loops. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Design piping systems and instrumentation for oil and gas industries. |
| CO2 | Assess piping design layouts for oil and gas industries. |
| CO3 | Evaluate the performance of control systems in piping for reliability and compatibility. |
| CO4 | Optimize industrial process control systems. |
| CO5 | Analyze and troubleshoot issues to enhance both system efficiency and safety. |
| CO6 | Optimize piping system designs for performance assessment. |

END SEMESTER EXAMINATION – APRIL/MAY 2026

| # Course Code | # 23ME2019 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # FINITE ELEMENT METHODS IN ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define finite element analysis. |  | CO1 | R | 1 |
| 2. | Write the application of FEA in thermal engineering. |  | CO5 | U | 1 |
| 3. | Define node. |  | CO1 | R | 1 |
| 4. | Write the purpose of Pascal’s triangle. |  | CO2 | U | 1 |
| 5. | Define aspect ratio. |  | CO1 | A | 1 |
| 6. | Write the magnitude of natural coordinate system. |  | CO2 | A | 1 |
| 7. | Classify the types of boundary condition in FEA. |  | CO3 | U | 1 |
| 8. | Write the purpose of IGES file. |  | CO6 | A | 1 |
| 9. | Name a finite element based analysis software. |  | CO6 | U | 1 |
| 10. | State the stages in finite element analysis according to computer implementations. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | List the convergence criteria in finite element analysis. |  | CO1 | A | 3 |
| 12. | Differentiate between scalar and vector field problems. |  | CO4 | An | 3 |
| 13. | Justify the reason for using local coordinate system, while evaluating the element shape function. |  | CO2 | R | 3 |
| 14. | Write the properties of shape function. |  | CO3 | An | 3 |
| 15. | Define higher order element and write its advantages. |  | CO5 | A | 3 |
| 16. | Write the sources of error in Finite Element Analysis. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the factors to be considered in finite element analysis during the process of discretization. | CO1 | A | 8 |
|  | b. | Write the properties of stiffness matrix. | CO1 | A | 4 |
| 18. |  | (1) (2) 100N 10cm 10cm Evaluate the elongation, strain and stresses for the stepped bar shown in Figure 1. A1=2cm2; A2=1cm2 ; E1=E2=2x105N/mm2 Figure1 | CO2 | E | 12 |
| 19. |  | The following differential equation is available for a physical phenomenon d2y /dx2 + 300 x2 = 0; 0 ≤ x ≤ 1; Trial function is y = a(x – x4); Boundary conditions are y(0) = 0 and y(1)=0. Calculate the value of parameter ‘a’ using the following methods: a. Point collocation b. Least square method c. Sub domain method d. Galerkin’s method | CO3 | An | 12 |
| 20. |  | Determine three points on the 50oC contour line for the rectangular element shown in Figure3. The nodal values are T1=42oC, T2= 54oC,T3= 56oC and T4= 46oC. Nodal coordinates are node1 (5,3), node2 (8,3), node 3(8,5) and node4( 5,5). S x Y 2(8,3) 1(5,3) 3 (8,5) 4(5,5) t Figure3 | CO5 | A | 12 |
| 21. |  | Define link element and formulate the element stiffness matrix for 1Dimenisonal link element. | CO4 | C | 12 |
| 22. |  | Explain the steps involved in Finite Element Analysis using FEA software and apply them to analyze a structural problem with a suitable case study. | CO6 | A | 12 |
| 23. |  | Develop the element shape functions for 1-Dimensional quadratic element. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Evaluate the element shape functions and calculate the value of temperature at point ‘P’ as shown in Figure2. The nodal values of temperature are Ti = 80oC ; Tj = 100 oC ;Tk= 150 oC. Point ‘P’is located at (2, 5). (All dimensions are in cm) P X Y i (1,1) j(3,4) k(2,7) Figure 2 | CO5 | E | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Acquire the fundamental theory of finite element analysis and develop characteristic equation. |
| CO2 | Derive element matrix equation by applying basic laws in mechanics and integration by parts. |
| CO3 | Apply suitable boundary conditions to a global equation for field problems |
| CO4 | Implement the formulation techniques of one, two-dimensional elements |
| CO5 | Understand the application and use FE method for solving heat transfer, fluid mechanics and structural problems. |
| CO6 | Use finite element software to solve engineering problems |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23ME2020 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # GAS DYNAMICS AND JET PROPULSION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Write the expression for the velocity of sound in a compressible medium. |  | CO1 | A | 1 |
| 2. | Define critical state in compressible flow. |  | CO1 | R | 1 |
| 3. | Differentiate ‘isentropic processes’ from ‘adiabatic processes’ in terms of heat transfer and entropy |  | CO2 | U | 1 |
| 4. | Define impulse function in compressible flow. |  | CO2 | R | 1 |
| 5. | State the expression for stagnation pressure ratio in isentropic flow. |  | CO3 | U | 1 |
| 6. | List the basic assumptions made in the analysis of an oblique shock. |  | CO3 | R | 1 |
| 7. | State the relationship between Mach number and duct length in a constant area duct with friction |  | CO4 | U | 1 |
| 8. | Define maximum heat transfer in Rayleigh flow |  | CO4 | R | 1 |
| 9. | List the basic components of a pulse [jet engine |  | CO5 | U | 1 |
| 10. | State the basic principle of rocket propulsion |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Describe the physical significance of Mach number in identifying flow regions. |  | CO1 | U | 3 |
| 12. | Analyze the effect of backpressure on the flow through a convergent-divergent nozzle. |  | CO2 | An | 3 |
| 13. | Analyze the effect of increasing Mach number on static pressure and temperature ratios. |  | CO3 | An | 3 |
| 14. | Explain the physical significance of the Rayleigh line on an h–s diagram. |  | CO4 | U | 3 |
| 15. | Classify aircraft engines based on their method of air breathing and energy conversion. |  | CO5 | U | 3 |
| 16. | Explain the relationship between specific impulse and specific propellant consumption. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Air (ɣ=1.4, R=287.43J/kg k) enter a straight axisymmetric duct at 300 K, 3.45 bar and 150 m/s and leaves it at 277 K, 2.058 bar and 260 m/s. The area of crossection at entry is 500 cm2. Assuming adiabatic flow, determine: i) stagnation temperature ii) maximum velocity iii) mass flow rate iv) area of crossection at exit. | CO1 | A | 12 |
| 18. |  | A nozzle in a wind tunnel gives a test section Mach number of 2.0. Air enters the nozzle from a large reservoir at 0.69 bar and 310 K. The cross-sectional area of the throat is 1000 cm2. Determine the following quantities for the tunnel for one dimensional isentropic flow: i) Pressures, Temperatures and velocities at the throat and test sections, ii) Area of crossection of the test section, iii) Mass flow rate and iv) Power required to drive the compressor. | CO2 | A | 12 |
| 19. |  | Derive and interpret the physical significance of Prandtl–Meyer relation for isentropic expansion of a supersonic flow. | CO3 | An | 12 |
| 20. |  | The Mach number at the exit of the combustion chamber is 0.9. The ratio of stagnation temperature at the exit and entry is 3.74. If the pressure and temperature of the gas at the exit are 2.5 bar and 1000℃ respectively, determine a) Mach number, pressure and temperature of the gas at entry b) the heat supplied per kg of the gas, and c) the maximum heat that can be supplied. Take ɣ=1.3 and Cp = 1.218 kJ/kgK. | CO4 | A | 12 |
| 21. |  | Derive the expression for different component efficiencies of Turbojet engine. | CO5 | An | 12 |
| 22. |  | Derive the expressions for non-dimensional mass flow rate in terms of pressure ratio and maximum mass flow rate. | CO2 | An | 12 |
| 23. |  | An aircraft flies at 960 kmph. One of its turbojet engines takes in 40 kg/s of air and expands the gases to the ambient pressure. The air fuel ratio is 50 and the lower calorific value of the fuel is 43MJ/kg. For maximum thrust power, determine a) jet velocity, b) thrust, c) specific thrust, d) thrust power, e) propulsive, thermal and overall efficiencies and f) TSFC. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | A rocket has the following data: Propellant flow rate is 0.5 kg/s, Nozzle exit diameter is 10 cm, Nozzle exit pressure is 1.02 bar, Ambient pressure is 1.013 bar, Thrust chamber pressure is 20 bar, Thrust is 7kN. a) Determine the effective jet velocity , actual jet velocity, specific impulse and the specific propellant consumption, b) Recalculate the values of thrust and specific impulse for an altitude where the ambient pressure is 10 mbar. | CO6 | A | 6 |
|  | b. | Explain with a neat sketch, the working of liquid propellant rocket engine. | CO6 | An | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Use the Energy Equation to solve problems related to compressible flow |
| CO2 | Evaluate the Impact of changes in the area on flow properties in nozzles and diffusers. |
| CO3 | Predict flow properties before and after the shock wave in high-speed flows. |
| CO4 | Determine the effect of Friction and Heat Transfer on flow properties. |
| CO5 | Analyze the performance of aircraft engines |
| CO6 | Optimize the performance of rocket propulsion systems. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23ME2043 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # DATA SCIENCE AND ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Outline the importance of data science in modern engineering. |  | CO1 | R | 1 |
| 2. | Identify the key components involved in data science. |  | CO1 | U | 1 |
| 3. | Define data arrays. |  | CO2 | R | 1 |
| 4. | Write an overview of Matplotlib |  | CO2 | R | 1 |
| 5. | Recall the purpose of the continue statement in loops. |  | CO3 | U | 1 |
| 6. | Write down the function of return statement in a python. |  | CO3 | U | 1 |
| 7. | Define probability. |  | CO4 | R | 1 |
| 8. | Give one example for discrete random variable. |  | CO5 | U | 1 |
| 9. | Define poisson distribution |  | CO5 | R | 1 |
| 10. | State Why is predictive checking important in statistical models. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate between data science and data analytics. |  | CO1 | U | 3 |
| 12. | Outline the merits of demerits of using NumPy arrays over Python lists. |  | CO2 | R | 3 |
| 13. | Compare built-in functions and user-defined functions. |  | CO3 | U | 3 |
| 14. | Recall about the principle of insufficient reason with examples. |  | CO4 | R | 3 |
| 15. | Write in short about properties of variance. |  | CO5 | U | 3 |
| 16. | Infer the steps involved in maximum likelihood estimation. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Discuss the stages involved in the data science process to support decision making in engineering applications. | CO1 | U | 12 |
| 18. |  | Illustrate the rules for naming variables and explain different built-in data types with suitable examples. | CO1 | A | 12 |
| 19. |  | Elaborate the structure of comma separated analysis library and Infer how data can be analyzed using python with suitable examples. | CO2 | A | 12 |
| 20. |  | Explain different visualization techniques such as line plots, scatter plots, and histograms. | CO2 | U | 12 |
| 21. | a. | Summarize the characteristics of strings, basic string operations, and their role in data manipulation with suitable examples. | CO3 | U | 06 |
|  | b. | Interpret different ways of formatting output using the print function. | CO3 | U | 06 |
| 22. |  | Compare the functions of theoretical probability, simulation-based probability with empirical probability. | CO4 | An | 12 |
| 23. | a. | Explain the Bernoulli distribution along with its parameters and characteristics. | CO5 | A | 06 |
|  | b. | Outline about the probability density function and cumulative distribution function with suitable examples. | CO5 | A | 06 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Discuss the characteristics of the standard normal distribution and its significance in statistical analysis using some case examples. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Program in Python within a Jupyter notebook environment. |
| CO2 | Summarize and compare datasets using empirically estimated statistics. |
| CO3 | Demonstrate the ability to employ conditional statements and loops to control program flow and automate repetitive tasks. |
| CO4 | Apply simple probability rules to propagate uncertainty. |
| CO5 | Analyze and model data using discrete and continuous random variables and make informed decisions based on probabilistic distributions and statistical inference techniques. |
| CO6 | Understand the properties and characteristics of normal distribution, and effectively utilize covariance and correlation measures. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 23ME2044 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # INDUSTRIAL SAFETY ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Mention one reason for the evolution of modern safety concepts. |  | CO1 | R | 1 |
| 2. | Infer the term "near miss" incident. |  | CO1 | U | 1 |
| 3. | Cite the full form of the acronym ‘ZMS’. |  | CO2 | U | 1 |
| 4. | Recognize the importance of machine guards in ensuring safety in industries. |  | CO2 | U | 1 |
| 5. | Interpret any one-health risk that result from exposure to X-rays. |  | CO3 | U | 1 |
| 6. | List few examples of ionizing radiation. |  | CO3 | R | 1 |
| 7. | Define: TLV in toxicology. |  | CO4 | R | 1 |
| 8. | Cite one mode of entry of toxic substances into the human body. |  | CO4 | U | 1 |
| 9. | Write the characteristics of a good industrial layout. |  | CO5 | A | 1 |
| 10. | Indicate any two artificial sources of light. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Distinguish ‘safety legislation’ from ‘safety standards’. |  | CO1 | U | 3 |
| 12. | List the sequence of process during LOTO. |  | CO2 | R | 3 |
| 13. | Name any two atmospheric contaminants encountered by workers in industries. |  | CO3 | R | 3 |
| 14. | Cite few health problems faced by workers due to elevated temperature. |  | CO4 | U | 3 |
| 15. | Report the merits of following safety protocols in industries. |  | CO5 | U | 3 |
| 16. | Identify the effects of poor housekeeping in industries. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Describe the process and significance of conducting a safety audit, including the types of safety audits and the steps involved in conducting an effective audit. | CO1 | U | 12 |
| 18. | a. | Justify: “Machine guards are used to protect workers from fatal injuries”. | CO2 | E | 6 |
|  | b. | Describe the types of nip-point protective devices used in industries. | CO2 | U | 6 |
| 19. |  | Explain the types of infections caused by biological agents. | CO3 | U | 12 |
| 20. |  | Deduce the different long-term and short-term effects of various types of toxins. | CO4 | An | 12 |
| 21. |  | Analyze the role of safety training and audits in controlling industrial accidents. | CO5 | An | 12 |
| 22. | a. | Analyze the importance of industrial layouts in reducing industrial accidents. | CO1 | An | 6 |
|  | b. | Analyze the significance of a safety rules in controlling accidents in industries. | CO1 | An | 6 |
| 23. |  | Discuss the role of supervisory staff and management in accident prevention with examples. | CO2 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Report the importance of housekeeping in industrial plant sanitation with examples and standards. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understanding the importance of safety in process industries |
| CO2 | Understanding the ethical issues that may arise from industrial processes |
| CO3 | Communicate the difference between Hazard and Risk |
| CO4 | Be able to express Safety in terms of Risk and to recognize unacceptable/inappropriate levels of Risk. |
| CO5 | Be able to Assess & identify the potential hazards in process industries |
| CO6 | Appreciation and applying safety procedures in a process industries. |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25ME205 | # Duration | # 3 hrs |
| --- | --- | --- | --- |
| # Course Title | # ENGINEERING MECHANICS | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | Related CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | State parallelogram law of forces. | 1b | R | 1 |
| 2. | Define the term "resolution of a force" within a 2D plane. | 1c | R | 1 |
| 3. | Sketch the reaction components of a roller support. | 2b | U | 2 |
| 4. | Infer a couple in mechanics and its effect. | 2e | U | 2 |
| 5. | State the formula to locate the centroid for a semicircle. | 3a | R | 3 |
| 6. | Indicate the expression for moment of inertia (Ixx) of a hollow rectangle of width (B and b) and height (H and h). | 3c | U | 3 |
| 7. | Define the term "angle of repose" in relation to friction. | 4a | R | 4 |
| 8. | Interpret Coulomb’s law of dry friction for a body at rest. | 4b | U | 4 |
| 9. | State the equation for velocity in motion under gravity. | 5a | U | 5 |
| 10. | Summarize the principle of the Impulse-Momentum theorem. | 5f | U | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Estimate the magnitude of a second concurrent force if the resultant is 15 N, one force is 10 N, and the angle between them is 30°. | 1b | U | 1 |
| 12. | Analyze the given system of forces and transform it into an equivalent force-couple system at point B. | 2f | An | 2 |
| 13. | Identify the centroid of the composite area shown in the figure using analytical formulas. | 3a | U | 3 |
| 14. | A body of weight 100 N is placed on a rough horizontal plane and pushed by a force P of 45 N as shown in the figure, to just cause sliding over the horizontal plane. Calculate the co-efficient of friction. | 4c | An | 4 |
| 15. | A particle is projected with an initial velocity of 60 m/s and at an angle of 75º with the horizontal. Determine the time taken by the particle to reach the maximum height and time of flight. | 5b | A | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Determine the magnitude and direction of the resultant of the given system of concurrent forces, as shown in the figure. | 1c | A | 1 |
| (OR) |  |  |  |  |
| 17 | Examine the cable system and determine the tension in cables AB and AC required to support a 40 kg load., as shown in the figure. | 1d | An | 1 |
| 18 | Solve for the support reactions at A and B for a 10 m span beam under the illustrated loading conditions. | 2b | A | 2 |
| (OR) |  |  |  |  |
| 19 | Analyze the given force system and determine the resultant in terms of magnitude and exact position, as shown in the figure. | 2e | E | 2 |
| 20 | Calculate the moment of inertia of an angle section shown in the figure, about its centroidal axis using the Parallel Axis Theorem. | 3d | A | 3 |
| (OR) |  |  |  |  |
| 21 | Calculate the moment of inertia of the composite section about its horizontal centroidal axis. | 3d | An | 3 |
| 22 | Justify the safety of the ladder by calculating the coefficient of friction required between the ladder and floor at the point of sliding. A ladder 5 meters long rests on a horizontal ground and leans against a smooth vertical wall at an angle 70° with the horizontal. The weight of the ladder is 900 N and acts at its middle. The ladder is at the point of sliding, when a man weighing 750 N stands on a rung 1.5 metre from the bottom of the ladder. | 4f | E | 4 |
| (OR) |  |  |  |  |
| 23 | Block 2 rests on Block 1 and is attached by a horizontal rope AB to the wall as shown in the figure. Estimate the force P that is necessary to cause motion of Block 1 to impend. The co-efficient of friction between the Blocks is 0.25 and between the floor and Block 1 is 0.33. Mass of blocks 1 and 2 are 137 N and 88 N respectively. | 4c | An | 4 |
| COMPULSORY QUESTION: |  |  |  |  |
| 24 | The figure shows a body of weight 300 N on a smooth horizontal plane attached by a string to a 30 N weight, which hangs freely. Apply Newton’s law of motion to determine the acceleration of the system and tension in the string. | 5d | A | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25ME206 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | AI FOR MECHANICAL ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | Related CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | List the importance of learning agent in automated manufacturing. | 1a | R | 1 |
| 2. | Write the PEAS description for a robot vacuum cleaner. | 1b | R | 1 |
| 3. | Analyze the importance of completeness in search algorithms | 2a | U | 2 |
| 4. | Interpret the search technique that guarantees the shortest path in an unweighted graph. | 2b | U | 2 |
| 5. | Interpret hill-climbing algorithm with application in lathe process optimization. | 3a | R | 3 |
| 6. | Give one real-world application of Hill Climbing. | 3b | R | 3 |
| 7. | Differentiate ‘declarative knowledge’ from ‘procedural knowledge’ in mechanical systems. | 4a | U | 4 |
| 8. | Infer predicate logic used in mechanical applications. | 4b | U | 4 |
| 9. | Illustrate label in supervised learning. | 5a | A | 5 |
| 10. | Illustrate AI learning techniques with example in predictive maintenance systems. | 5b | A | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Design an intelligent agent for conveyor belt speed optimization with control logic. | 1a | A | 1 |
| 12. | A conveyor system has weighted paths (cost in seconds): S → A (2) S → B (5) A → C (4) B → C (1) C → G (3) Use Uniform Cost Search to calculate: a) Total cost of the optimal path | 2b | An | 2 |
| 13. | Write the step by step procedure for Min Max b) Final path from S to G |  | A | 3 |
| 14. | Analyze knowledge representation techniques for hydraulic system fault detection with examples. |  | An | 4 |
| 15. | Apply reinforcement learning for adaptive control in mechanical systems such as conveyor belts or robotic arms. |  | A | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Discuss Reflex Goal based and Model based agent with relevant examples. | 1c | A | 1 |
| (OR) |  |  |  |  |
| 17 | Develop a PEAS framework for an autonomous mobile robot used for material handling in a mechanical workshop including navigation, obstacle avoidance, and efficiency. | 1a | An | 1 |
| 18 | Traverse the given graph using the Breadth First Search algorithm. | 2b | A | 2 |
| (OR) |  |  |  |  |
| 19 | A smart manufacturing plant uses an Automated Guided Vehicle (AGV) to transport precision machined components from the Raw Material Store (Node 0) to the Final Assembly Station (Node 5). The factory layout is represented as a weighted graph, where each node denotes a workstation and each edge represents the travel cost (time in seconds). A heuristic table provides estimated distances from each node to the assembly station. Unexpected congestion and machine availability require the AGV to determine the optimal path in real time. Apply the A* search algorithm to determine the most efficient route for the AGV from Node 0 to Node 5. | 2c | An | 2 |
| 20 | From the given Minimax decision tree, identify the optimal move for CNC tool path planning using alpha beta pruning. | 3d | A | 3 |
| (OR) |  |  |  |  |
| 21 | Consider the above graph representing a robotic path planning problem ina mechanical workshop, where each node represents a location and h(n) indicates the heuristic (estimated distance to goal). - Explain the concept of Greedy Best-First Search (GBFS) and its working principle. - Starting from node S, apply Greedy Best-First Search and determine the path to the goal node G. - Show the order of node expansion during the search process. - Compare GBFS with A* in terms of optimality and efficiency for mechanical navigation systems. | 3f | A | 3 |
| 22 | Construct the truth table for Conjunction, disjunction and negation with suitable examples and syntax. | 4c | An | 4 |
| (OR) |  |  |  |  |
| 23 | Formulate and analyze predicate logic expressions to control a mechanical valve system used in an industrial process. A valve system operates based on the following conditions: - If pressure is high, the valve must open - If temperature is high and pressure is normal, the valve should partially open - If both temperature and pressure are low, the valve remains closed - If the system detects a fault, the valve must shut immediately - Define suitable predicates, variables, and constants to represent the system conditions. - Formulate the above statements using First Order Logic (FOL) with appropriate quantifiers and logical connectives. - For a given condition: - Pressure = High - Temperature = Normal - No fault detected - Apply inference rules and determine the valve state. | 4e | A | 4 |
| Compulsory Question: |  |  |  |  |
| 24 | Develop a reinforcement learning-based system for Mechanical applications. | 5d | A | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25ME207 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # PYTHON PROGRAMMING FOR MECHANICAL ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | Write a Python program using functions and loops to check whether a given string is a palindrome and count the number of vowels present in it. | 1e | A | 1 |
| 2. | Identify and correct the error in the following program: def check_even(n): if n % 2 = 0: print("Even") else: print("Odd") | 1f | An | 1 |
| 3. | Write a Python program using list comprehension to filter all even numbers from a list and store their squares in a new list. | 2a | A | 2 |
| 4. | Find and fix the error in the following code: numbers = [1,2,3,4] numbers.sort(reverse="True") print(numbers) | 2b | An | 2 |
| 5. | Identify the error in the following code: class Test: def __init__(self): print("Constructor") t = Test t() | 3a | An | 3 |
| 6. | Develop a Python program demonstrating operator overloading for addition of two objects. | 3h | C | 3 |
| 7. | Develop a simple GUI application using Tkinter to accept user input and display it on button click. | 4e | C | 4 |
| 8. | Write a Python program to connect to a MySQL database, insert a record, and retrieve data. | 4g | C | 4 |
| 9. | Write a Python program to perform edge detection and explain its importance in image processing. | 5a | A | 5 |
| 10. | Write a Python program using PIL/OpenCV to open, display, and resize an image. | 5b | A | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | A university wants to analyze student feedback messages. Write a Python program using functions, loops, and string manipulation to: - Count total words - Count vowels - Identify and display the longest word | 1a | A | 1 |
| 12. | An online store maintains a list of product prices. Write a Python program to: - Filter products costing above ₹1000 using list comprehension - Sort them in descending order - Display the top 3 expensive products | 2g | A | 2 |
| 13. | A transport company manages different types of vehicles. Write a Python program using inheritance and method overriding to: - Create a base class Vehicle - Derive class Car - Override a method to calculate fuel cost | 3b | C | 3 |
| 14. | A login system requires a graphical interface. Write a Python program using Tkinter to: - Accept username and password - Display a message on button click (valid/invalid login) | 4b | C | 4 |
| 15. | Write a Python program using PIL or OpenCV to: - Load an image - Crop a specified region from the image - Rotate the cropped image by a given angle - Display the final output | 5c | C | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | A digital banking system needs to validate customer transaction numbers using a recursive logic. Write a Python program that: - Accepts a number from the user - Uses a recursive function to: - Find the sum of digits repeatedly until a single digit is obtained (digital root) - Uses operators and conditional statements to: - Check whether the final result is even or odd - Display all intermediate and final results | 1c | An | 1 |
| (OR) |  |  |  |  |
| 17 | A data analysis system processes a dataset containing numbers and categories. Write a Python program that: - Stores numeric data in a list and performs: - Slicing to extract a subset of values - Sorting in ascending and descending order - Converts the list into a tuple and explains the concept of immutability - Uses a set to remove duplicate values - Uses a dictionary to: - Map each number to its square - Applies filtering techniques to: - Extract even numbers Display all results clearly and explain the behaviour of each data structure. | 1e | C | 1 |
| 18 | A transport company manages different types of vehicles such as cars and trucks. Each vehicle has common attributes like vehicle ID, fuel type, and base fare. Write a Python program that: - Creates a base class Vehicle with appropriate attributes and methods - Uses encapsulation to protect data members - Derives a class Car and Truck using inheritance - Implements method overriding to calculate fare differently for each type - Creates objects and displays details of each vehicle | 2b | C | 2 |
| (OR) |  |  |  |  |
| 19 | A manufacturing plant needs a system to manage and monitor its machines used in production. Each machine record should contain: Machine ID (Unique), Machine Name, Type, Operating Hours, and Maintenance Status. Develop a Python program using a list of dictionaries to perform the following operations: - Machine Overview: Display all machine details in a formatted manner. - Maintenance Update: Search for a machine using its Machine ID. If found, allow the user to update its operating hours and change the maintenance status (e.g., Pending/Completed). - Maintenance Alert: Identify and display all machines that have operated for more than 100 hours and require servicing. - Decommissioned Machines: Find all machines marked with status = "Decommissioned" and remove them from the list using list comprehension or appropriate list methods. | 2c | A | 2 |
| 20 | A manufacturing plant wants to develop a system to manage different types of machines such as Lathe Machines and CNC Machines. Design and develop a Python program using object-oriented programming concepts to perform the following tasks: - Create a base class Machine with appropriate data members and use a constructor to initialize the values. - Apply encapsulation by keeping data members private and accessing them using suitable methods. - Create derived classes LatheMachine and CNCMachine using inheritance. - Implement method overriding to calculate machine cost differently for each type based on operating hours and additional charges. - Demonstrate polymorphism by calling the same method for different machine objects. - Overload an operator (e.g., +) to combine the operating hours or cost of two machines. | 3d | An | 3 |
| (OR) |  |  |  |  |
| 21 | A mechanical engineering company wants to develop a system to monitor different types of hydraulic machines such as Hydraulic Press and Hydraulic Lift. Each machine has common attributes like Machine ID, Pressure Capacity, and Fluid Type. Write a Python program using object-oriented programming concepts to perform the following: - Create a base class HydraulicMachine with appropriate data members and initialize them using a constructor. - Use access specifiers (private/protected) to restrict direct access to machine details and provide methods to display them. - Create derived classes HydraulicPress and HydraulicLift using inheritance. - Add specific attributes for each derived class (e.g., load capacity for press, lifting height for lift). - Create objects of the derived classes and display all machine details using appropriate methods. | 3f | An | 3 |
| 22 | A company wants to maintain employee details in a MySQL database. Write a Python program to: - Establish a connection to a MySQL database - Create a table for storing employee details (ID, Name, Salary) - Insert new employee records into the table - Retrieve and display all employee records - Update the salary of a specific employee - Delete an employee record based on ID - Handle database connection and execution errors properly | 4c | C | 4 |
| (OR) |  |  |  |  |
| 23 | A college stores student records in a text file where each line contains Student ID, Name, and Marks. Write a Python program to: - Open and read the student data from a file - Display all student records - Calculate the average marks of all students - Identify students scoring above average - Write the details of these students into a new file - Use appropriate file handling methods - Handle possible exceptions such as: - File not found - Incorrect data format | 4e | C | 4 |
| COMPULSORY QUESTION: |  |  |  |  |
| 24 | A photo editing application allows users to perform basic image processing operations for improving and customizing images. Write a Python program using PIL or OpenCV to perform the following tasks: - Open and display an input image - Resize the image to a specified width and height - Perform cropping on a selected region of the image - Apply rotation to the image by a given angle - Perform any one image transformation (such as grayscale conversion or flipping) - Display all processed images clearly | 5c | C | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25ME208 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # TRANSFORMS, FOURIER SERIES FOR MECHANICAL ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | Related CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | Identify the sequence behavior of wind-induced oscillatory stress acting on a long-span bridge, modelled by an=(-1)n. | 1a | U | 1 |
| 2. | Compute the RMS value of the displacement of a structural member vibrating under load, given by fx=x-2 ; 0<x<l. | 1f | U | 1 |
| 3. | Determine the nature of the partial differential equation representing vibration potential in a mechanical membrane under uniform tension: ∂2∅∂x2+5∂2∅∂x∂t+2∂2∅∂t2=0. | 2a | U | 2 |
| 4. | Write the one-dimensional wave equation along with all its possible solutions and the most suitable solution. | 2b | U | 2 |
| 5. | State Parseval’s theorem for the Fourier transform, which relates the total energy of a signal in time domain to its frequency domain representation. | 3g | U | 3 |
| 6. | Analyze the beam deflection characteristics of a simply supported beam carrying a load, where the deflection is modeled as fx=e-2x ; x>0, using the Fourier cosine transform. | 3d | An | 3 |
| 7. | Compute the Laplace transform of the oscillatory displacement of a mechanical vibration system modeled by f(t)=cosh(2t) to analyze its oscillation characteristics. | 4a | U | 4 |
| 8. | Estimate the impulse response of a mechanical vibration system exhibiting damped oscillations with transfer function Hs=s+3(s+3)2+16 using the inverse Laplace transform to analyse its dynamic behaviour. | 4d | E | 4 |
| 9. | Compute the Z-transform of the oscillatory torque signal of a CNC motor given by x(n)=cos(6π​n) and describe its significance in understanding mechanical resonance and oscillatory noise. | 5a | A | 5 |
| 10. | Determine the response of a CNC machine position control system to an impulse input, given by the difference equation y(n)−0.4 y(n−1)=δ(n). | 5g | A | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Construct the thermal deformation profile of a machining process where a temperature sensor is fixed along a rod of length π, and the temperature variation is given by f(x) = x; 0<x<π, using the half-range cosine series representation. | 1d | A | 1 |
| 12. | Compute the steady-state heat conduction equation in an engine-cooling rod of length L that is perfectly insulated on its lateral surface. Assuming the system has reached steady state after a long time and write the boundary conditions T (0) =0∘C and T (L) =200∘C for the rod ends. | 2e | An | 2 |
| 13. | Verify Parseval’s theorem for a machine shutdown vibration signal modeled by f(x)=e−x, x≥0, by comparing the total energy computed in the time domain with that obtained from its Fourier transform in the frequency domain. | 3g | A | 3 |
| 14. | Determine the Laplace transforms of the motor input signals in a robotic speed control system where the input varies as sin(3t), is exponentially damped as e-2xsin3t, and increases with time as tsin3t to analyze the system behavior. | 4b | An | 4 |
| 15. | Estimate the Z-transforms of the signals obtained from a system with input xn= δn when (i) the signal is advanced and form y1n= δn-2+5δn-7 and (ii) the signal is scaled by (0.8)n to form y2n=0.8nδ(n). | 5f | U | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16 | Apply Fourier series analysis to the vibration of a rotating pump shaft whose displacement over one complete rotation, from angle –π to π, follows the function f(x)=x, by expanding it into a Fourier series, obtaining the coefficients a0​, an​, and bn​, and interpreting their significance for diagnosing the pump's condition. | 1b | A | 1 |
| (OR) |  |  |  |  |
| 17 | Compute the first three harmonics of the Fourier series for an automotive engine vibration signal measured over one crankshaft cycle. The vibration amplitudes f(x) (in mm/s) are recorded at equally spaced crankshaft angles x over (0,2π) as follows: x; 0; π3; 2π3; π; 4π3; 5π3; 2π. f(x); 1.0; 1.4; 1.9; 1.7; 1.5; 1.2; 1.0. | 1g | A | 1 |
| 18 | Evaluate the pressure oscillation in a rocket nozzle of length l rigidly supported at both ends, where the gas column is initially displaced as ux,0=asinπxl and released at t=0 with zero initial velocity. The motion follows the one-dimensional wave equation to obtain the expression for u(x,t) at any time t. | 2c | E | 2 |
| (OR) |  |  |  |  |
| 19 | Determine the temperature distribution in a machine-frame metal rod of length lm, initially heated with u(x,0)=15x, where both ends are maintained at zero excess temperature, to obtain u(x,t) at any time t. | 2d | A | 2 |
| 20 | Evaluate the noise reduction characteristics of an engine vibration signal that decays as fx= e-4x;x>0 with Fs= 14+is​ - If the vibration amplitude doubles, find the Fourier transform of 2e-4x. - If the signal is delayed by a, find the Fourier transform of e-4x-a. - If the time scale changes so that 2x=e-8x , find its Fourier transform. - If vibration velocity is considered, find the Fourier transform of ddx(e-4x). - If vibration acceleration is analyzed, find the Fourier transform of d2dx2(e-4x). | 3f | E | 3 |
| (OR) |  |  |  |  |
| 21 | Evaluate the frequency component characteristics of a gearbox vibration signal described by fx= 1 if x<a0 if x>a using Fourier transform of compound functions, and hence obtain the inverse Fourier transform to show that 0∞sinassds represents the reconstructed vibration signal behaviour. | 3c | E | 3 |
| 22 | Analyse the behaviour of a robotic speed control system subjected to actuator input current and reference speed command using Laplace transform techniques. (a) Evaluate the stability characteristics of the system for the input ft=5e-5t-3cosh2t+sin7t-3e-7tt using pole locations and region of convergence. (b) Apply Laplace transform techniques to determine the transform of the reference signal ft=5t-sin3t+cos4t-t5-7, and assess its effect on the system response (bounded or unbounded) | 4b | An | 4 |
| (OR) |  |  |  |  |
| 23 | Evaluate the motion control characteristics of a CNC axis system governed by the differential equation y′′−3y′+2y=e3t, with initial conditions y(0)=1, y′(0)=0, using the Laplace transform method to obtain the complete time-domain response y(t). | 4e | E | 4 |
| COMPULSORY QUESTION: |  |  |  |  |
| 24 | Analyze the signal reconstruction characteristics of the given discrete-time system by determining the inverse Z-transform h(n) of 1(z-2)(z-3)(z-4)​ using the partial fraction expansion method | 5e | An | 5 |

END SEMESTER EXAMINATION – APRIL / MAY 2026

| # Course Code | # 25ME209 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Title | # ELECTRICAL AND ELECTRONICS FOR MECHANICAL ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions | LUO | RBT Level | CO |
| --- | --- | --- | --- | --- |
| PART – A (10 X 2 = 20 MARKS) |  |  |  |  |
| 1. | List the benefits of conducting accurate performance evaluation of electrical components. | 1a | R | 1 |
| 2. | Identify the average lifespan of CFL lamps. | 1f | R | 1 |
| 3. | Explain how a water level indicator works using its key components. | 2c | U | 2 |
| 4. | State the formula for calculating voltage drop in a wiring system. | 2d | R | 2 |
| 5. | List the speed-torque characteristics of a DC shunt motor under different load conditions. | 3a | R | 3 |
| 6. | Define the performance parameters of a BLDC motor under load and speed variations. | 3f | R | 3 |
| 7. | List the typical design constraints in PCB development for power supply circuits. | 4c | R | 4 |
| 8. | Describe how output signals reflect the internal functionality of a circuit. | 4f | U | 4 |
| 9. | State the truth table of a 2-input Ex-OR logic gate. | 5c | R | 5 |
| 10. | Recall the basic components involved in a password-based door locking system using IoT. | 5e | R | 5 |
| PART – B (5 X 6 = 30 MARKS) |  |  |  |  |
| 11. | Calculate the maximum power output of a solar photovoltaic cell from its I-V characteristics. | 1b | A | 1 |
| 12. | Draw and label the wiring diagram for a ceiling fan connection. | 2b | A | 2 |
| 13. | Examine high-efficiency and low-efficiency conditions in a DC shunt motor based on input-output data. | 3d | A | 3 |
| 14. | Compare different types of temperature sensors (e.g., thermistor, RTD, LM35) and their roles in circuit design. | 4a | An | 4 |
| 15. | Examine the system and show how sensors, microcontrollers, and network modules work together for smart access. | 5e | An | 5 |
| PART – C (5 X 10 = 50 MARKS) |  |  |  |  |
| 16. | Evaluate the transient response of an RLC circuit and justify the observed behavior. | 1c | E | 1 |
| (OR) |  |  |  |  |
| 17. | Evaluate the energy consumption of CFL and LED lamps over a 24-hour period. | 1d | E | 1 |
| 18. | Assess the suitability of different wiring methods for a commercial building based on safety and efficiency. | 2a | E | 2 |
| (OR) |  |  |  |  |
| 19. | Investigate the causes of performance variation in an ON/OFF LED control circuit. | 2c | An | 2 |
| 20. | Assess the effectiveness of selecting a DC series motor for various industrial load applications based on its speed-torque characteristics. | 3c | E | 3 |
| (OR) |  |  |  |  |
| 21. | Inspect the effect of speed control methods using embedded systems on motor torque and speed stability. | 3e | An | 3 |
| 22. | Illustrate schematic-level and layout-level representations of a temperature sensor. | 4b | An | 4 |
| (OR) |  |  |  |  |
| 23. | Judge the impact of constraint violations on the performance and reliability of a power PCB. | 4c | E | 4 |
| Compulsory Question: |  |  |  |  |
| 24. | Evaluate the security and convenience of the door locking system in terms of IoT integration and user interaction. | 5e | E | 5 |